# ECOR 1010 – Introduction to Engineering

Derek Wu

Assignment #5

Assignment Title: Unvaried Data for Plastic Knobs

TO Marking TA:

Name: Shazad Abraham-Doman

Email: shazadabrahamdoman@cmail.carleton.ca

Lab Section: B5

Room: MC5010 – Carleton University

FROM:

Email: derekwu3@cmail.carleton.ca

Student Number: 100941853

Number of Figures and Tables (Including handwritten ones): 4

Last Date and Time of Revision: 13/11/2013 02:41

# Introduction

In this report, it will address on the probability that the production process will produce knobs with acceptable dimensions being in the range of 137 to 143 thousands of an inch. These sizes of knobs needs to be in that range to accommodate the size of the metal insert. The types of calculations explored in this report is important because it helps on the quality control of the knobs being produced and the efficiency of the process.

# Materials and Methods

- The technical data presented in Production Record Sheet were used to calculate the Mean, Sample Variance, Standard Deviation, Standard Error, Population Mean with 90% confidence interval using equations and MS excel

- All different types of calculations can referred to appendices calculations

# Results

- Results via equations: Mean-140, Sample Variance-13, Standard Deviation-3.6, Standard Error-0.53,  90% Confidence level of population mean-0.8711

- Results via MS Excel: Mean-140, Sample Variance-13, Standard Deviation-3.6, Standard Error-0.53,  90% Confidence level of population mean-0.8898

- Results via Random generated sample: Mean-140, Sample Variance-14, Standard Deviation-3.7, Standard Error-0.55,  90% Confidence level of population mean-0.92984

# Discussion

Due to the findings from the production data, it is know that the data produces a histogram that is skewed to the left. Since the acceptable range of the knobs is from 137 to 143 thou, if one of the sample had been recorded as 115 thou, this would have been considered an outlier and unacceptable for the production process because it would have been too small for the metal insert. In the two different methods used to determine probability of the data, the only difference between the numbers were the confidence level of population mean otherwise all the other results were the same. The confidence level result from equations was 0.8711 while the result from the excel was 0.8898. The only similarities between the knobs measurements and the random generated numbers were the mean being at 140, otherwise the other results were similar but not exactly the same. The estimate of proportion of knobs that were acceptable in the data came out to be 0.71 ± 0.13 of 90% confidence showing that at 90% confidence level 71 % of the knobs were in the acceptable range. The estimate based on normal Gaussian distribution was 0.6±0.19 of 95% confidence. This means that for the normal Gaussian distribution, the estimate of acceptable knobs would be 60% of the total with 95 % confidence. This production can be considered to be efficient based on the values of the two estimates because it is both over half of the total production.

# Conclusions

In conclusion, the objective has been fulfilled on checking the quality of the knobs being produced using the different types of calculations and results described in the report. It is also shown that the production of the knobs was efficient by the 2 different estimates of the knobs being in the range of the acceptable values.

# APPENDIces- Figures and Table

Figure 1.

Table 1.

Frequency table

| Bin | Frequency |
| --- | --- |
| 133 | 1 |
| 135 | 3 |
| 137 | 9 |
| 139 | 12 |
| 141 | 8 |
| 143 | 5 |
| 145 | 4 |
| 147 | 2 |
| 149 | 1 |
| More | 0 |

Table 2.

Summary Table

| Sample Statistics | Results via equations | Results via ms Exel | Random Normal Distribution using Excel with seed (853) |
| --- | --- | --- | --- |
| Mean | 139.7 | 139.7 | 139.6 |
| Sample Variance | 12.62 | 12.62 | 13.78 |
| Standard Error | 0.5296 | 0.5296 | 0.5534 |
| Sample Standard Deviation | 3.552 | 3.552 | 3.71 |
| Population Mean with 90% confidence interval | 0.8711 | 0.8898 | 0.9298 |

Table .3

Generated sample of random data

| 134.8634 | 140.4161 | 138.1328 | 142.9939 | 142.7368 | 144.6315 | 136.6411 | 136.3453 | 137.9758 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 133.1637 | 137.6094 | 130.3344 | 138.6764 | 146.1573 | 140.3804 | 137.1909 | 137.142 | 148.4818 |
| 141.3648 | 142.502 | 138.2737 | 139.1593 | 139.8828 | 145.7366 | 141.2864 | 139.9549 | 140.1943 |
| 139.5185 | 135.8781 | 141.9332 | 142.0829 | 138.7287 | 139.0714 | 134.57 | 140.417 | 140.2192 |
| 142.3832 | 144.084 | 138.0686 | 143.1051 | 141.5793 | 140.9729 | 134.0789 | 131.8633 | 139.8107 |

# APPENDIces- Caculations

Estimate of acceptable knobs 90% confidence

p=ρ ±Z90ρ(1-ρ)n

p=3245±1.963245(1-3245)45

p=3245±1.963245(1-3245)45

p=0.71 ±0.13

Estimate of acceptable knobs (normal Gaussian distribution ) 95% confidence

p=ρ ±Z95ρ(1-ρ)n

Use of Excel MS formulae

p=NORMDIST143- NORMDIST137

p= 0.822734897-0.222676556

p=0.60005831±2.580.60005831(1-0.60005831)45

p=0.60005831 ±0.19

90% confidence interval of population mean

= Standard error x Z90

= 0.529553 x 1.645

= 0.871114685

Mean

x= 1Ni=1Nxi

x= 1456278

x= 139.7111

Sample variance

S2=1N-1i=1N(xi-x)2

S2=145-1(135-139.7)2+ (140-139.7)2+…+(148-139.7)2

S2=12.61919

S2=12.62

Standard Deviation

S=1N-1i=1N(xi-x)2

S=145-1(135-139.7)2+ (140-139.7)2+…+(148-139.7)2

S=12.61919

S=3.55235

S=3.552

Standard Error

SX  = SN

SX  = 3.5523545

SX  = 0.5296