Flow Through a Venturi Meter

Author:

November 25, 2013

Lab Partners: Neal Traynor, Garrett Lihou, Alex Bouchie

Group Number: L1I

# Summary:

# Nomenclature/List of Symbols:

| Variable/symbol | Name | Units |
| --- | --- | --- |
| Q | flow rate | m3/sec |
| y | depth | inches |
| P | Static pressure | Mpa |
| ρ | density | kg/m3 |
| g | Gravitational constant | N/kg or m/s2 |
| z | elevation | metres |
| V | Velocity | m/s |
| w | width | metres |
| h | height | metres |
| m | Mass flow rate | Kg/s |

# Flow Analysis:

Static Pressure as Found by the Static Taps

Using the hydrostatic pressure equation the static pressure of a flow can be found using the measurements recorded from the manometer bank.

P=ρwatergy+Patm [1]

y is taken to be the change in elevation of the fluid within the manometer between the taps open to atmosphere and the tap in question.

True Volume Flow Rate for Venturi meter

True Volume Flow Rate at Venturi meter is calculated using  Bernoulli equations. Bernoulli's equation is applied between two points of the same elevation, one point a distance from the inlet in the room and a point at the mouth of the inlet.

Proom+12ρairVroom2+ρgyroom=P2+12ρairV22+ρgy2

Rearranging Bernoulli's equation for V2, velocity at the inlet can be found. Static pressure at point 2 can be found by using the reading of the manometer (y) and the hydrostatic pressure equation[1].

Proom+12ρairVroom2=(Proom+ρwatergy)+12ρairV22

Taking Vroom to be zero as it is assumed to be so small that it can be considered negligible and rearranging  this equation V2 can be found.

V2=2ρwatergyρair [2]

It is important to remark that Bernoulli's equation has conditions that restrict the application of this equation. The streamline that is analysed must be part of a 1-d flow, that has not lost or gained energy between the two  points over which Bernoulli's equation is applied. In this situation these criteria are met as no device is present to give energy to the flow, friction is assumed to be zero between the flow and the apparatus, and the bellmouth inlet of the apparatus creates a 1-d flow within the apparatus.

Ideal Volume Flow Rate at Venturi meter

The ideal volume flow rate of the Venturi meter is found by applying both the continuity and Bernoulli's equation between the inlet and throat of the Venturi meter. For both equations point 1 is taken to be the centre of the cross-sectional area of the mouth of the inlet while point 2 is taken to be the centre of the cross-sectional area of the throat of the Venturi outlet. Starting with Bernoulli's equation and rearranging for V2,

P1+12ρairV12+ρgy1=P2+12ρairV22+ρgy2

Applying the continuity equation and rearranging for at the throat an equation can be found to substitute velocity at the throat in Bernoulli's equation,

m1=m2=>A1V1=A2V2=> V2=A1V1A2

Plugging the above equation into the Bernoulli's equation developed for this scenario and rearranging to solve for velocity at the inlet of the Venturi

P1+12ρairV12=P2+12ρairA1V1A22      =>      12ρairV12-12ρairA1V1A22 =P2-P1

12ρairV121-A1A22=P2-P1

Finally rearranging for velocity at the inlet of the Venturi meter

V1=2P2-P1ρair1-A1A22

Static pressure at points 1 and 2 can be found by using the reading of the manometer (y) and the hydrostatic pressure equation[1].

V1=2ρwatergy1-ρwatergy2ρair1-A1A22

V1=2ρwatergy2-y1ρair1-A1A22 [3]

Velocity of a flow to Volume flow rate

Velocity of a flow can be converted to a volume flow rate by multiplying the velocity of the flow by the cross-sectional area the flow is moving through

Calculating the Venturi coefficient

The Venturi coefficient is calculated using the following equation

CV=QactualQideal

Qactual and Qideal  can be found from the actual velocities and ideal velocities calculated using equations [2] and [3] and the cross-sectional area of the Venturi meter at its inlet.

CV=VactualAinletVidealAinlet

CV=VactualVideal [4]

Cross-sectional area throughout the Venturi Meter

Area of the cross-sectional area at any point throughout the Venturi meter can be found using the basic area of a circle formula

A=πr2

where r is the radius of the cross-sectional circle in metres. Radius of the Venturi Meter is assumed to vary linearly with the distance of the cross-sectional area from the intake of the Venturi. With this and known radii at the throat and intake in mind a linear equation between radius and distance from the inlet can be derived. The slope of the equation is given by the rise over run between the inlet and throat

m=riserun=y2-y1x2-x1=1.62562cmr-2.59082cmr22.5cmd-10.7cmd=-0.04089cmrcmd

By using the calculated slope knowing that the intercept of the slope is 1.2985 cm(no translation from x-axis) the equation of the slope of the radius can be found. Therefore the equation for the radius of the Venturi meter at a distance(x) from the inlet is:

r=-0.04089*x+1.2985 [5]

Theoretical Pressure along the Venturi Meter

The theoretical pressure of the Venturi tube can be Calculated using Bernoulli's equation between points 1 and 2 where 1 is a point a distance from the inlet in the room(Patm), and point 2 is the middle of the cross-sectional area within the Venturi meter. Point 1 is assumed to have no velocity as it is a large distance from the inlet of the meter.

P1+12ρairV12+ρgy1=P2+12ρairV22+ρgy2

P2=P1-12ρairV22 [6]

Ideal Velocity along the Venturi meter

Using the continuity equation and the ideal velocity calculated at the mouth of the inlet, the ideal velocity of the Venturi meter can be calculated at any point within the Venturi meter

A1V1=A2V2

V2=A1V1A2 [7]

Stagnation Pressure of the flow at the Venturi Throat

Stagnation Pressure of the flow at the throat of the Venturi meter throat can be calculated with the following equation

Pstag=Pstatic+12ρairV2 [8]

# Experimental Setup and Procedure:

Figure 1: Setup of Venturi Meter

Refer to pages 5 of the Fluid Mechanics I (MAAE 2300) course manual for instructions on how to conduct this experiment properly.

# Results and Discussion:

Results

|  | Fast flow | Slow Flow |
| --- | --- | --- |
| Theoretical Flow Rate (m3/s) | 0.017433 | 0.014564 |
| Actual Flow Rate (m3/s) | 0.015929 | 0.013835 |
| Venturi Coefficient | 0.9137 | 0.949 |
| Stagnation Pressure at Throat(Pa) | 101325 | 101325 |

Table 1: Results of Venturi Meter Experiment

Discussion

A Venturi coefficient is used to differentiate between the theoretical flow rate and actual flow rate of a flow that travels through a Venturi Meter. That is to say that a Venturi coefficient of 1 means that the actual flow through the Venturi is perfectly identical to the theoretical flow moving through the Venturi meter. A Venturi coefficient is simply a numerical way to correct for real world losses, otherwise not considered in theoretical fluid dynamics.

Possible sources of the losses in the real world flow can be found in the assumptions made when applying different equations to the Venturi meter. One such equation  is Bernoulli`s equation as it requires a flow that has no energy loss or gain, is uncompressible, and1-d. energy loss and the 1-d flow can be considered not met as the no-slip condition will cause the flow to begin to vary  along the cross-sectional area of the as a boundary layer is generated between the wall and the flow. This boundary layer and friction will cause some of the energy of the flow to be transfer to thermal energy that isn`t considered when measuring the potential energy of the flow stored in the static and dynamic pressures of the flow. In addition, in calculating the true velocity, the velocity of air was taken to be zero within the room. This assumption can be considered valid as the air would move so little in such a large reservoir as to be considered  non-moving. Furthermore, the compressibility of air makes Bernoulli`s equation more inapplicable in this situation.

However, in order to minimize the differences caused by these assumptions, measures were taken to minimize the impacts of the assumptions. First a Bell-mouth inlet was used to provide a flow that is 1-d upon entering the Venturi meter, and the Venturi is kept short in order to minimize the size of the boundary layer generated along the length of the Venturi tube.

Figure 2: Static Pressure of the Fast Flow

Figure 3: Static Pressure of the Slow Flow

As can be seen above both flow are very similar in their differences between their experimental and theoretical pressures. Both Flows show the experimental pressure as higher than the theoretical pressure before the throat of the Venturi(lowest pressure) and lower than the theoretical pressure after the throat. The initially higher pressure can be attributed by friction and part of the flow hitting the wall of the Venturi meter as the radius of the meter shrinks. This could reduce the speed of the flow reducing dynamic pressure and increasing static pressure as energy is transferred.

The lower experimental pressure after the throat is cause to believe that the flow after throat is flowing faster than theoretical values anticipate. This could be due to turbulence caused by the flow rushing out of the throat and trying to rapidly fill the expanding cross-section. This turbulence would leave swiftly moving low pressure zones along the wall of the Venturi meter after the throat of the Venturi, explaining the lower than expected pressures after the Venturi throat.

Figure 4: Approximate Velocities along the Venturi meter

# Appendices:

## Lab Results

## Sample Calculations:

Converting Imperial Units to Metric

0.0254inch= xmetres

0.0254inch*1.02= xmetres=0.025908metres

Static Pressure as Found by the Static Taps

P=ρwatergy+Patm [1]

P=1000kg/m3*9.81*.124-.067+101325=100765.83Pa

True Volume Flow Rate for Venturi meter

V2=2ρwatergyρair [2]

V2=2*(1000kgm3*9.81ms2*.124m-.067m)1.2250kg/m3=30.21ms

Ideal Volume Flow Rate at Venturi meter

V1=2P2-P1ρair1-A1A22 [3]

V1=2*1000kgm3*9.81ms2.057m-.429m1.2250kg/m31-0.000527 m20.000207 m22=33.067ms

Velocity of a flow to Volume flow rate

Q=V*A=33.067ms*0.000527m2=0.017433m3s

Calculating the Venturi coefficient

CV=VactualVideal [4]

CV=30.21ms33.067ms=0.9137

Cross-sectional radii throughout the Venturi Meter

r=-0.04089*x+1.2985 [5]

r=-0.04089*13cm-10.7cm+1.2985=1.201cm

Theoretical Pressure along the Venturi Meter

P2=P1-12ρairV22 [6]

P2=101325-121.225*33.0682ms=100665Pa

Ideal Velocity along the Venturi meter

V2=A1V1A2 [7]

V2=0.000527m2*33.068ms0.000287 m2=60.713ms

Stagnation Pressure of the flow at the Venturi Throat

Pstag=Pstatic+12ρairV2 [8]

Pstag=97116.51Pa+121.225kgm3*82.89ms=101325Pa