Exploration Geophysics

Geophysics Methods:

- Seismic methods

- Reflection seismic

- Refraction seismic

- Gravity

- Electromagnetic (EM) methods

- Magnetic methods

- Radiometric methods

Exploration seismology:

- Mainly based on the reflection and refraction of primary (P) waves on boundaries between different media in the subsurface

- P-waves: easy to generate, travel fast. Propagate in liquids and solids

- Controlled approach: specialized equipment is used to generate seismic waves at pre-set times

- By far, the most widely used method for oil and gas exploration

Seismic impedance Z [kg/m2s]: in general the harder the rock the high the Z

Critical angle: incidence angle for which refracted waves are generated

Critical distance: minimum distance at which refractions can be recorded

Crossover distance: distance from which refracted waves are the first arrivals

Seismic sources:

- Objective: sample the subsurface at a wide range of frequencies

- Two approaches: Impulsive sources and vibratory sources

Receivers: Land receivers and marine receivers

Reflection Seismics:

1. CMP (common midpoint) sorting

2. NMO (normal moveout) correction

3. Stack

CMP sorting: Gather all traces sampling the same subsurface point from different distances

NMO correction: Gather all traces sampling the same subsurface point from different distances

Stack: After NMO correction, all traces sampling the same subsurface point are summed to produce one trace

Gravity varies because of the Earth’s:

- Ellipsoid shape

- Rotation

- Irregular surface relief

- Heterogeneous subsurface density distribution

- Gravity anomalies results from the density contrast Δρ between a rock body of density ρ1and its surroundings of density ρ2

- Gravity anomalies due to geological sources are tiny compared to the Earth’s gravitational acceleration

Gravimeter: the instrument used to make gravity measurements in the field

- Only measure vertical component

Data reduction: process of correcting gravity data to isolate only the effects due to subsurface density distribution

Types of corrections:

Tidal Correction: influence of solid Earth tide and oceanic tide

Latitude Correction: variation with latitude

- Correcting for the rotation of Earth

- Centrifugal acceleration

- Correcting for ellipsoid shape

Free Air Correction: elevation above a datum

Bouguer Correction: density / thickness of rocks situated between survey and reference level

Terrain Correction: topographic relief

Eotvos Correction: gravimeter movement on ship / aircraft

Gravity method applications:

- Petroleum exploration

- Mineral exploration

- Hydrogeology

In class poll questions:

How can a seismic ray behave when it encounters a new medium? Redirect, reflect and transmit

What type of wave is used for exploration seismology? P-wave

To account for the centrifugal force of the rotating Earth, which correction factor do we apply? Latitude correction factor