### Question 1

Consider the network diagram shown below: 

WA and WB are workstations connected to access ports in VLAN 10.
The switches all support 802.1q vlans. 
Identify the links where a frame traveling from WA to WB will be tagged.

Answer

|  |  | The frame will be dropped in transit because one of the interswitch links is not a trunk. |
| --- | --- | --- |
|  |  | WA - S1 |
|  |  | S1 - S2 |
|  |  | S2 - S3 |
|  |  | S3 - S4 |
|  |  | S4 - S5 |
|  |  | S5 - WB |

1 points

### Question 2

In this network diagram, port f0/1 on the switch links to the router. This port should be configured:

Answer

|  |  | as an access port in vlan 10. |
| --- | --- | --- |
|  |  | as a dynamic port. |
|  |  | as a trunk port with native vlan 10. |
|  |  | none of the above. |

1 points

### Question 3

Match the protocol name with the description

Answer

| 802.1q ANSWER = C Read Answer Items for Question 3. 802.3 ANSWER = F Read Answer Items for Question 3. ARP ANSWER = H Read Answer Items for Question 3. DTP ANSWER = B Read Answer Items for Question 3. ISL ANSWER = D Read Answer Items for Question 3. Proxy ARP ANSWER = E Read Answer Items for Question 3. auto-mdix ANSWER = A Read Answer Items for Question 3. | Answer A.; A method which automatically adjusts to the required ethernet cable connection type.. B.; A protocol which dynamically negotiates trunk links.. C.; A system of vlan tagging.. D.; A vlan encapsulation protocol.. E.; Device responds with its own mac-address to an arp request for a host with an IP address in a remote network.. F.; The IEEE ethernet specification standard.. G.; Used to obtain the Data Link layer address for another node when you know its Network layer address.. H.; Used to obtain the Network Layer address of another node when you know its Data Link Layer address.. |
| --- | --- |

1 points

### Question 4

One end of an interSwitch link is configured as 
     switchport mode dynamic auto

Which configurations for the other end of the link will configure a trunk link?

Answer

|  |  | switchport mode access |
| --- | --- | --- |
|  |  | switchport mode dynamic desirable |
|  |  | switchport mode dynamic auto |
|  |  | switchport mode trunk |
|  |  | switchport mode trunk switchport nonegotiate |

1 points

### Question 5

Select all of the port configurations where forwarding of DTP frames will be disabled.

Answer

|  |  | int f0/1 switchport mode access |
| --- | --- | --- |
|  |  | int f0/2 switchport mode trunk |
|  |  | int f0/3 switchport mode dynamic auto |
|  |  | int f0/4 switchport mode dynamic desirable |
|  |  | int f0/5 switchport mode trunk switchport nonegotiate |

1 points

### Question 6

Select all of the protocols that are Cisco proprietary and NOT industry standards.

Answer

|  |  | 802.1q |
| --- | --- | --- |
|  |  | 802.3 |
|  |  | ISL |
|  |  | DTP |
|  |  | Proxy ARP |
|  |  | auto-mdix |

1 points

### THESE ANSWERS WILL GET YOU MARK 5 OUT OF 6.  I THINK QUESTION 1 IS WRONG IF YOU WANT GUARNTEE 5/6 SELECT THESE ANSWERS

Bottom of Form