###   Question 1

1 out of 1 points

|  | What is the address of the gateway for W1? Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; 205.5.5.1. |  |  |  |

###   Question 2

1 out of 1 points

|  | Which access-list will allow connections from all hosts in the 123.2.3.0/24 network except for 123.2.3.1? Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; access-list 2 deny 123.2.3.1 access-list 2 permit 123.2.3.0 0.0.0.255. |  |  |  |

###   Question 3

1 out of 1 points

|  | Which command configures the switch to learn and save secure mac-addresses dynamically? Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; switchport port-security mac-address sticky. |  |  |  |

###   Question 4

1 out of 1 points

|  | Workstations are connected to ports 1, 2 and 3. Port 4 has no host connected. DHCP server is connected to port 5. Trusted ports should be: Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answers:; 5. |  |  |  |

###   Question 5

0 out of 1 points

|  | Identify all of the true statements about the differences between configuration of a static mac-address vs configuration of a secure mac-address. mac-address-table static 1111.1111.1111 vlan 1 int f0/1; interface f0/1 sw mode access sw port-security sw port-security violation restrict sw port-security maximum 1 sw port-security mac-address 1111.1111.1111. Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answers:; With the secure mac-address, only host 1111.1111.1111 can connect to port 1. With the secure mac-address, host 2222.2222.2222 cannot connect to port 1. With the static mac-address, host 1111.1111.1111 cannot move to port 2.. |  |  |  |

###   Question 6

1 out of 1 points

|  | The command to enter the configuration for a max of 2 concurrent vty sessions is: Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; line vty 0 1. |  |  |  |