### Question 1

A switch receives a vtp update (revision number 20) from a server in the same vtp domain. The switch receiving the vtp advertisement will update its vlan database if:

Answer

|  |  | It is a vtp server with revision number 10. |
| --- | --- | --- |
|  |  | It is a vtp server with revision number 30. |
|  |  | It is a vtp client with revision number 10. |
|  |  | It is a vtp client with revision number 30. |
|  |  | It is a vtp transparent switch revision number 0. |

1 points

### Question 2

A vlan is deleted on a vtp server; the ports assigned to that vlan on the vtp client:

Answer

|  |  | are unaffected. |
| --- | --- | --- |
|  |  | are reassigned to the management vlan. |
|  |  | are reassigned to the native vlan. |
|  |  | are reassigned to the default vlan. |
|  |  | remain assigned to the deleted vlan and are shutdown in software. |

1 points

### Question 3

VTP is an industry standard protocol.

Answer True 
False

1 points

### Question 4

Which VTP mode(s) allow a switch to create a new vlan?

Answer

|  |  | vtp mode client |
| --- | --- | --- |
|  |  | vtp mode relay |
|  |  | vtp mode server |
|  |  | vtp mode transparent |

1 points

### poQuestion 5

Which of the following are required for vtp convergence.

Answer

|  |  | Link between switches must be a trunk. |
| --- | --- | --- |
|  |  | Link between switches can be an access link or a trunk. |
|  |  | VTP version must be the same on both switches. |
|  |  | VTP is backwards compatible so the vtp version on the client must be the same or newer than the vtp version on the server |
|  |  | A vtp password must be configured on each switch. |