### Question 1

A router <-> switch link is implemented as a trunk link carrying traffic for multiple vlans. This is implemented on the router as:

Answer

|  |  | switchport mode trunk. |
| --- | --- | --- |
|  |  | interface f0/0 mode trunk. |
|  |  | with subinterfaces. |
|  |  | with secondary IP addresses. |

1 points

### Question 2

A router switch link should be configured to carry traffic for multiple vlans. The configuration on the switch is:
int f0/1
switchport mode trunk
switchport trunk native vlan 10
switchport trunk allowed vlan 10,20
switchport nonegotiate

What encapsulation command is required on the router subinterface associated with vlan 20?

Answer

1 points

### Question 3

A router switch link should be configured to carry traffic for multiple vlans. The configuration on the switch is:
int f0/1
switchport mode trunk
switchport trunk native vlan 10
switchport trunk allowed vlan 10,20
switchport nonegotiate

What encapsulation command is required on the router subinterface associated with vlan 10?

Answer

1 points

### Question 4

A router-switch link is configured as a link carrying traffic for several VLANs. Select all of the properties that apply to this link.

Answer

|  |  | there is one physical link per VLAN. |
| --- | --- | --- |
|  |  | there is one physical interface for many VLANs. |
|  |  | the interface on the switch must be configured as a trunk port. |
|  |  | the interface on the switch must be configured as an access port. |
|  |  | the physical interface on the router can be assigned an IP address. |
|  |  | each VLAN should be associated with a subinterface on the router. |

1 points

### Question 5

DTP can dynamically negotiate a trunk link between a cisco router & a cisco switch.

Answer True 
False