###   Question 1

1 out of 1 points

|  | By default, which switch interface is assigned an IP address? Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; int vlan 1. |  |  |  |

###   Question 2

1 out of 1 points

|  | Which commands disable logins on vty lines? Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; no password cisco ; login. |  |  |  |

###   Question 3

1 out of 1 points

|  | What command displays a switch forwarding table? Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; show mac-address-table. |  |  |  |

###   Question 4

1 out of 1 points

|  | A switch wants to find the port for mac-address 1111.2222.3333 The switch will determine the port: Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; by a lookup of the mac-address in CAM. |  |  |  |

###   Question 5

1 out of 1 points

|  | Which port-security violation mode will reject traffic from “illegal” mac-addresses without generating a log message or disabling the port? Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; switchport port-security violation protect. |  |  |  |

###   Question 6

1 out of 1 points

|  | Port-security is enabled on f0/1 by the following config commands (starting from factory-default): ena config t interface f0/1 switchport port-security max 1 switchport port-security mac-address sticky switchport port-security violation shutdown Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; False. |  |  |  |

###   Question 7

1 out of 1 points

|  | The following commands are enough to enable ssh on the vty lines starting from factory default. ena config t line vty 0 15 transport input ssh Answer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Selected Answer:; False. |  |  |  |