Final

April 21, 2014

9:20 PM

Week 1: Introduction

Key topics:

- What is geomatics?

- The power of maps

Geomatics - What is it?

- Consists of products, services and tools involved in collection, integration and management of geographic data

- Geographic information can be retrieved from various sources, including earth-orbiting satellites, ground based instruments and airborne and seaborne

- An academic and professional field that involves:

  - Collection and analysis of spatial information

  - Discovery of geographic patterns and processes

  - Communication of geographic knowledge, often as part of a broader effort to resolve environmental and social problems

  - Ongoing development of new technologies, methods and other innovations

The power of maps

- European colonization

  - Allowed for early explorers to navigate

  - Allowed for a visual representation of colonization

- War

  - Can provide information on enemy armies and the conflict area

  - Allowed pilots to learn routes and the area

- Flooding

  - Can map flooding and help to defend against it

- Humanitarian aid

  - Maps can help aid workers respond

    - Provides information of the area and the people

- Disasters

  - Before and after shots

- Emergency response and network analysis

  - Can help emergency services find the fastest way to and from a location

  - Can allow for alterations to traffic flow to alleviate congestion

- Business and marketing

  - Businesses and marketing can analyze customer behaviour, census data, city maps and other information to improve better suit customers as well as their own business

What is geospatial revolution?

- Geospatial technologies are becoming more powerful

- Geospatial technologies are affecting people's lives in more ways

- More people, organizations and agencies are using geospatial tools

- Geospatial Revolution (part 1) video

  - http://geospatialrevolution.psu.edu/episode1/complete

Week 2: Thinking spatially

Key topics:

- Spatial thinking

- Spatial relationships and spatial analysis

- Key concepts – location, distance, spatial patterns,

- movement and connectivity, scale

- Spatial data

- Basic spatial statistics

Thinking spatially

- Humans have an innate ability to think spatially and make mental maps

  - Mental maps

    - 2 categories

      - Uses landmarks of various sorts (spatial)

      - Use of cues (stimulus)

    - Affected by perception and interaction with the world around us

Spatial analysis: Cholera outbreak in London, 1853-1854

- John Snow was able to map the disease outbreak and the location of public water pumps

  - His map showed the cholera deaths were based around a water pump

Spatial relationships

- What geographic features are connected?

  - e.g. bike paths

- What geographic features are adjacent or contiguous to each other?

  - e.g. a landfill and a lake

- What is contained within an area?

  - e.g. high rises within a particular neighbourhood

- What geographic features overlap?

  - e.g. a city and a geological fault line

- What features are near others (in close proximity)?

  - e.g. an offshore oil drilling platform and a fishing zone

- What is the difference in elevation of geographic features

  - e.g. along a hiking trail

- What features are along another feature?

  - e.g. a restaurant along a highway

Location

- Communicating location

  - Nominal (e.g. "Confederation Park")

Absolute (e.g. "45⁰ 40' 15'' North 75⁰ 51' 28'' West")

Relative (e.g. "halfway between Regina and Saskatoon")

Distance

- Why is distance important?

  - First law of geography: “Everything is related to

everything else, but near things are more related than

distant things.”

- Even in an increasingly globalized economy, distance matters (e.g. for goods that are heavy or perishable)

- Measuring distance

  - Absolute, straight line distance (e.g. kilometers)

  - Distance along a path or route

  - Time

  - Timemaps (e.g. measuring the distance between Ottawa to Montreal)

  - Cost

  - Greenhouse gas emissions

- Scales of analysis

  - Local

  - Regional

  - National

  - International

  - Global

- Map scale

  - Ratio of the distance on the map to the distance on the earth

    - e.g. at a 1:50,000 scale, anything measured on the map is equal to 50,000 in real life

- Map generalization

  - A reduction in level of detail on a map covering a larger area

- Spatial data

  - Along with qualitative and quantitative data, there is also spatial data

    - Unique geographic coordinates or other spatial identifiers that allow the data to be located in geographic space

  - Types of spatial data

    - Discrete

      - Discontinuous, have definite boundaries

      - Types

        - Points

          - e.g. mailboxes, trees, lamp posts

        - Lines

          - e.g. roads, rivers, power lines

        - Areas

          - More detail than points

          - e.g. field, building, lake

- Continuous

  - Transition is gradual, not abrupt

    - e.g. elevation, temperature

Spatial distribution

Point patterns

- Random

  - Influence of boundaries

    - Random can be a cluster based on boundaries

- Clustered

- Regular

Accuracy and precision

- Not the same thing

Mean centre

- A measure of central tendency that can determine the center of a distribution

- Standard distance

  - Measures how dispersed a distribution is around a mean centre, 2/3rds of the points will be located within a standard distance

Week 3: Georeferencing

Key topics:

- Global Positioning System (GPS)

  - Components and how it works, including differential GPS

- Earth Models

  - Datums

  - Ellipsoid vs Geoid

- Coordinate Systems

  - Global or Spherical

  - Rhumb lines vs great circles

  - Calculation of distance along a parallel

  - Projected: UTM

  - DD to DMS conversions

  - Universal Transverse Mercator

Global Positioning System (GPS)

- How it works

  - Basis of GPS is triangulation (trilateration since angles are involved)

    - Trilateration - determining position based on geometry of triangles

  - Signal from satellite directly overhead is ~0.06 s

  - Even with VERY precise clocks, timing errors are a BIG problem

- Components

  - Space segment (satellites)

  - Control segment (global network of ground facilities that track and monitor GPS satellites and help key functions)

  - User segment (GPS receiver)

- Differential GPS (DGPS)

  - Two receivers can be used to increase position accuracy

  - One receiver is the base receiver, logging timing and position information constantly

  - Base receiver information is used to correct moving GPS

Earth models

- Datums

  - Set of physical reference points on earth's surface that are used to make position measurements against

  - 2 main North American Datums

    - Horizontal Control Datum (NAD83)

    - Vertical Control Datum (NAVD88)

- Ellipsoid and geoids

  - Earth is not a perfect sphere

    - Earth is an oblate ellipsoid (spheroid)

    - The geoid is the surface of the earth's gravity field, approx. mean sea level

      - Runs perpendicular to the direction of gravity pull

      - The mass of the earth is not uniform at all points, meaning that the magnitude of gravity varies, making the shape of the geoid irregular

Coordinate systems

- Global or spherical coordinate systems

  - Latitude φ

    - Angular distance between plane of equator and a line passing through the point being investigated and the center of the earth

  - Longitude λ

    - Angular distance between the Prime Meridian (0°) and meridian of point being investigated

    - Distance between 1 degree of latitude is always 111 km.

      - 1° of longitude = 111 km x cos (latitude)

      - e.g. What is the distance of 1° of longitude along the 60th parallel?

        - distance = 111 x cos(60) = 111 x 0.5 = 55.5 km

- Decimal Degrees to Degrees-Minutes-Seconds conversion

  - e.g

45.38205° N

0.38205 degrees x 60 minutes/degree = 22.92300

0.923 degrees x 60 seconds/minutes = 55.38"

45°22'55.38"N

- DMS to DD conversion

  - e.g.

45°23'2.47"N

2.47 / 60 seconds/minutes = 0.04117

23 + 0.04117 = 23.04117 / 60 minutes/degree = 0.38402

45.38402°N

- Projected coordinate systems

  - Map projection - a systematic transformation of the 3-dimensional earth into a 2-dimensional flat map

  - A projected flat map or parts of a map can have some, but never all of the following characteristics:

    - True angles/shapes

      - Prioritizes angles at the expense of angles

    - True areas

      - Prioritizes area (at expense of angles)

    - True directions

      - Prioritizes directions around a single point

    - True distances

      - Prioritizes distance around a single point

- Universal Transverse Mercator (UTM)

  - Advantages

    - High accuracy conformal projection for large scale applications

    - Coordinates easy to work with

    - Truly universal

      - Works anywhere in world except poles

  - Disadvantages

    - Hard to work with in areas that straddle multiple UTM zones

    - Not suitable for high latitudes

Week 4: An introduction to cartography

Key topics:

- What is a map?

- What are maps used for?

- A very brief history of cartography

- Types of maps

What are maps and what are they used for?

- A thematic representation of spatial or geographic info, usually on a plane surface of a region of the earth or heavens

- Maps can present enormous amounts of information very effectively, and can be used to establish theories and solve problems

- Maps are not objective, scientific representations of reality

  - Socially and historically subject to change, and have political dimensions

- What are maps used for?

  - Communication and understanding

    - Raising public awareness

  - Finding patterns

  - Deriving new information through analysis

    - Environmental conservation

  - To get status reports

  - To compile geographic information

  - Communicate ideas, concepts, plans and designs

    - Uneven development

  - Exert control over spaces, places and people

    - Lie with maps

A brief history of cartography

- 15th and 16th century cartography made major advantages through exploration

  - Provided much more accurate maps with coastlines, islands, rivers and harbours

    - Maps were highly valued for economic, military and diplomatic purposes, often treated as national or commercial secrets

Types of maps

- Topographic

  - Represent the earth's features accurately and to scale on a 2 dimensional surface

- Thematic

  - Represents a theme in a specific geographic area

Cartographic design: how to make a good map

- Smaller than the real world it represents

- Must depict curved surface of earth on a flat plane

- Must contain symbols to convey the info so the reader can interpret the map

Week 6: Geographic Information Systems

Key topics:

- Definitions of GIS and common misconceptions

- How is spatial data represented/stored in a GIS?

- How is attributed data represented/stored in a GIS?

- Map projections

  - “A map projection is a systematic transformation of the 3-dimensional Earth into a 2-dimensional flat map.”

  - Tangent

  - Secant

  - Properties

    - Can have some properties, but not all

      - True angles/shapes - prioritizes angles

      - True areas - prioritize area

      - True directions - prioritizes direction around single point

      - True distance - prioritizes distance around single point

  - UTM

    - Advantages

      - High accuracy conformal projection for large-scale applications

      - Coordinates easy to work with

      - Truly universal - works everywhere except poles

    - Disadvantages

      - Hard to work with in areas that straddle multiple UTM zones

- Roger Tomlinson and The "Canadian Geographic Information System"

  - Canadian geographer

  - Coined "Geographic Information Systems" in ~1961

  - Father of modern GIS

- Geographic Information System (GIS)

  - "information system used to manipulate, summarize, query, edit and visualize spatial and non-spatial information stored in a computer database”

  - “An automated system for the capture, storage, retrieval, analysis and display of spatial data”

  - Common misconceptions about GIS

    - GIS is only cartographic software/form of "computer mapping"

    - GIS is a type of graphic design or CAD software

    - GIS is only software

  - Trends

    - Faster paced hardware and software tech development (Moore's Law)

    - Improvements in computer networking and sharing of spatial data via internet

    - Increase in open source GIS software

    - GIS data and algorithms work together

  - Moore's Law

    - # of transistors on circuits double ~ every 2 years

  - Components of a GIS

    - Hardware

      - CPU - controls operations (more = faster)

      - Arithmetic Co-processer - increases calculation speed

      - Memory - performs system operations (>6GB, >1TB VRAM)

      - Mass Storage - >1TB

      - Display - goal is to view as much geographic space as possible at highest resolution

      - Peripheral input and output devices (HQ printers, plotters, possible requirement to digitize physical maps

      - Networking - enables sharing spatial data, GIS processing and distribution

    - Software

      - OS - widely adopted, reputable OS

      - Database Management System (DBMS) - stores and accesses attribute data

      - Functions - GIS software must be able to perform geospatial tasks

      - Network - data sharing and processing between computers

    - Cloud GIS

      - Distributed computing typically involving remote connected computers

      - Efficient for sharing data, distribution and collection; flexible; increased availability

      - e.g. Amazon's EC2 (Elastic Compute Cloud)

- GIS Data Models

  - Data models

    - Objects represented by a computer system together with their properties and relationships

    - In GIS, defines how real world spatial features are represented within a GIS

    - Manner in which spatial data are represented dictates the way their associated, non-spatial information (attribute data) are represented

  - Spatial Data Representation: raster vs vector

    - Raster data model - stores features in a user defined grid of pixels

      - Header information

        - # rows and columns

        - Geographic or projected coordinates of reference corner

        - Cellsize (same unit as coordinate system

        - No data value

        - Projection info often in other file

    - Vector data model - stores features as points, lines or polygons; more suitable for "discrete" (distinct) geographic phenomena

    - Spatial data describes locations of geographic features

      - Can be discrete or continuous

        - Discrete features - individually distinguishable features that do not exist between observations

        - Continuous features - features that exist spatially between observations

  - Attribute data storage and retrieval

    - Describes properties of spatial data, is stored and accessed differently depending on data model used

    - Raster - attribute stored as grid of values, separate grid for each attribute

    - Vector - attribute data stored as separate data table, each row relates to discrete geographic  feature (point, line or polygon), each column represents a property of interest (e.g. "geo-relational data model")

Week 7: Mapping Spatial Patterns

Key topics:

- Cartographic design

- Mapping spatial patterns

  - points, lines, areas and surfaces

  - measurement levels

  - classification and symbolization

  - issues related to boundaries

  - geovisualization

- Cartographic design

  - Review of key points

    - Maps are incomplete, selective representations of geographic areas

    - Maps use symbols to give reader information

    - Good maps depend on good spatial data

    - Maps typically show a curved surface on a flat plane

    - Well-established principles of cartographic design

      - Map composition

      - Title

      - Typography

      - Thematic content

      - Geographic reference material (lat-long)

      - Scale

      - Symbology and map legend

      - Orientation

      - "metadata"

- Mapping spatial patterns

  - Point data

    - Simple dot maps

    - Graduated symbols

  - Lines

    - Basically transportation (roads, rivers, train tracks, etc.)

  - Areas

    - Type of area (desert, water, forest, etc.)

    - Problems

      - Sharp boundaries may not exist on the ground

      - Boundaries can be arbitrary or poor match for what's being mappped

      - Differences in size of units can favour certain areas

  - Surfaces

    - Contour lines and other isolines (line on map connecting points sharing features)

    - Interpolation (discrete set of data)

- Geovisualization

  - Development and use of methods to facilitate visual exploration, analysis and presentation of spatial data

  - e.g. 3D maps, map animations

  - Interactive digital maps and spatial databases

Week 8: GIS Operations and Analysis

Key topics:

- Raster vs Vector analysis with GIS: various types and their definitions and explanations

- Various examples of problem solving with raster and vector GIS analysis

- Basic GIS Querying and Operations

  - Basic querying

    - On-screen querying

    - Spatial querying

    - Attribute querying

  - Buffering (Vector or Raster operation)

  - Creation of polygon that is a specific distance around a point, line or area feature.

  - Vector GIS operations

    - Vector operations

      - Intersect

      - Union

      - Erase

      - Symmetrical difference

- Raster GIS operations

  - Local operations - those that work on single cell locations (basically adding 2 data points together)

  - Focal operations - those that work on cell locations within a neighbourhood (focuses on centre pixel; majority)

  - Zonal operations - those that work on cell locations within zones (takes multiple data, one has grouped zones)

  - Digital Elevation Models (DEMs) (3D representation)

    - Slope

      - Calculates maximum rate of change in value from cell to neighbours

      - Max change in elevation over distance between cell and 8 neighbours identify steepest downhill decent from the cell

      - Lower the slope value, flatter the terrain, vice versa

      - Degree of slope = Ø

      - Percent  of slope = rise/run * 100

    - Aspect

      - Identifies downslope direction of the max rate of change in value from each cell to neighbours, using a DEM

      - Value of each cell in output raster indicate compass direction that the surface faces at that location

      - Measured clockwise in degrees from 0 (due north) to 360 (due north again); flat areas with no downward direction are given a value of -1

    - Aspect-Slope map

      - Simultaneously shows aspect (direction) and degree (steepness) of slope for terrain

      - Aspect categories symbolized using hues (red, orange, yellow, etc.); degree of slope classes are mapped with saturation (brilliance of colour); steeper slopes are brighter

    - Topographic Wetness Index  (TWI)

      - Common index of hydrologic similarity

      - Combines terrain slope raster with hydrologic contributing area raster to indicate relative wetness in landscape

      - Slope = neighbourhood operation, neighbourhood-type hydrologic operation, combined using local operator

      - TWI = ln(contributing area / slope)

Flowcharting GIS analysis workflows

- Complex GIS analysis workflows need to be planned beforehand

- Flowcharts commonly used - distinct symbology and colours to differentiate inputs, operations and output

- Takes care of documentation, reuse and sharing of workflows

- Problem solving and decision support analysis with GIS

  - e.g. real estate development analysis

  - Retail business analysis/marketing

  - Ecological corridor analysis

  - Flood mapping

Week 10: Remote Sensing: "making visible the invisible"

Key topics:

- How does remote sensing “make visible the invisible”?

- False vs true colour composite images

- Concept of spectral signatures

- Image classification: supervised vs unsupervised

What is remote sensing (RS)?

- Definition

  - Science of acquiring and applying information about earth's surface by measuring and analyzing emitted energy

- Remote sensing measurement

  - Instrument collets information about object within instantaneous FOV without physical contact

  - Instrument normally on satellites (orbital platform) or aircraft (suborbital platform)

  - Instrument measures electromagnetic energy emitted by earth surface

  - Requires illumination source

- Electromagnetic (EM) spectrum

- c = λv

  - Speed of light (c) = 3 x 108 m s-1

  - Wavelength (λ) = length of one wave cycle

  - Frequency (v) = number of cycles of waves passing through fixed point, per unit time

- Spectral Bands

  - Reflectance

- B, G and R bands have a low reflectance %,  Near Infrared has highest reflectance

- Dark colours = low reflectance

- Light colours = high reflectance

- Visible spectrum = ~390 - 700 nm (430 - 790 THz)

  - Nm = 10-9 m

  - THz = 1012 Hz

Visualizing remote sensing data

- Single channel records reflectance of one portion of EM spectrum

- Single channel contains many spatial data points in grid

- Multiple channels can be stacked to determine spectral signature at a X, Y location

- Spectral signatures - combination of EM data at different wave lengths to identify an object

- Any colour can be created by combining primary colours

  - Colours used for GIS are Red, Green and Blue (RGB)

  - Setting spectral bands to RGB colours can simulate natural colour (colour-composite)

    - Can be used to show color-infrared color composite

- Band Wavelengths (λ) (nm)

  - Blue (450 - 515 nm)

  - Green (525 - 605 nm)

  - Red (640 - 690 nm)

  - NIR (750 - 900 nm)

Remote sensing resolution

- 4 types

  - Spectral resolution

    - How finely you can divide the spectrum (measured by # of bands)

    - More divisions = higher spectral resolution

    - Higher spectral resolution increases ability to identify different land surface features

  - Spatial resolution

    - How finely you can resolve a point on the ground (measured in m)

    - Smaller spatial resolution results in better quality

    - Function of instantaneous FOV and altitude

  - Temporal resolution

    - Measured in time

    - How often a satellite can image a point on the ground

      - Depends on orbit

      - Can be reduced by changing the look angle of the sensor mechanically

  - Radiometric resolution (A.K.A. bit-depth)

    - How accurate you can measure/record the spectrum

    - Amount of precision that a sensor records electromagnetic radiation

    - Higher radiometric resolution means higher probability that object will be sensed more accurately

Image classification

- Why classify?

  - Characterize a landscape (classes)

    - i.e. forest, agriculture, water, buildings, etc.

  - To identify important features

- Classification scheme = structure of classes

  - Depends on user needs

  - Uses spectral (radiometric) differences to distinguish objects

- Types of classification:

  - Supervised classification

    - Requires analyst to select training areas of known feature types and digitize polygons within the area

      - Training areas characterize spectral properties of class

Computer creates mean spectral signatures using training areas

- Assign other pixels to classes by matching spectral properties of training sets (mean spectral signatures)

- Results in classified image

- Unsupervised classification

  - Spectral data imposes constraints on interpretation

  - No classes defined beforehand, uses statistical approach to divide data into clusters with best separation

  - After separation into pixels, you are left with distribution of pixels in clusters

  - Names assigned to clusters afterwards

    - Assignment of names requires some knowledge of the landscape and other data is useful (aerial photos, personal knowledge, etc.

  - Pros

    - Takes maximum advantage of spectral variability in an image

  - Cons

    - After separation, clusters may not match important classes on landscape

Some different types of remote sensing systems

- Multispectral

  - Landsat

    - Records earth

  - Landsat has a thermal channel

  - Discrete data

- Hyperspectral

  - Tens to hundreds of spectral bands

    - More narrow bands than multispectral

  - Continuous data

- Thermal

  - (3-14 um thermal-infrared energy)

- LiDAR (light detection and ranging)

  - Uses near infrared laser

  - Active remote sensing

    - Illumination source provided by instrument

    - Most multispectral instruments are passive (illumination from sun)

  - Dataset is very large (millions of points), must be split into tiles (grid) for computer processing

  - Exported in XYZ format

  - Gridded as "Digital Elevation Model"

  - Most LiDAR systems can record more than 1 return per pulse, allows for detecting portions of the pulse that were reflected from canopy and ground

- RADAR (radio detection and ranging)

  - Active remote sensing

Conclusions

- There are many different types of remote sensing instruments, working in different parts of the EM spectrum and operating off of different types of platforms; you should be able to explain and provide examples of remote sensing platforms and understand what portion of the EM spectrum they operate within

- With respect to remote sensing imagery, there are four different types of resolution to know about – can you define and explain them?

Image classification is a typical way in which multispectral imagery is turned into useful information by exploiting the fact that different landsurface features have different spectral signatures; supervised and unsupervised classification are two general approaches to image classification

- LiDAR and RADAR are two forms of active remote sensing, which differs from more common passive forms that use the sun as the illumination source

- With reference to multispectral remote sensing imagery, could you explain in general terms how remote sensing “makes visible the invisible”

Week 11: The social, cultural and political dimensions of mapping

Key Topics:

- The power of maps

- The politics of placenames

- Counter mapping/participatory mapping

- Local geographic knowledge

- Geospatial technologies and privacy

Key Idea:

- Maps are not representations of reality?

  - All maps are selective representations of geographic spaces

  - Can have significant, often subtle political dimensions

  - Content (places, boundaries, toponyms [place name])

  - Use of symbology

  - Choice of projection

Power of maps

- Colonialism

  - Europeans made maps of new lands with boundaries claimed by different nations

  - Names given by indigenous people ignored, as well as claim of ownership

  - European words and meanings written onto map

  - European maps influenced nature of space they represented

- Colonial space in British Columbia, 19th century

  - BC couldn't have been reorganized into colonial space without some sort of map

  - Maps allowed new people to identify their location and travel

  - Used European terms to help conceptualize unfamiliar spaces

- Lie with maps

  - Can influence ones opinion by providing incorrect information

Politics of placenames

- Changes in country names after independence

- First Nation placenames in BC

  - 34 names accepted in treaty in 2000

Counter mapping

- Aimed at resisting dominant official cartographies

- Originated in Nunavut, now almost world-wide practice

  - Methods, objectives and actors vary

- Current prevalence encouraged by new technologies, awareness of indigenous rights, sympathetic NGOs and university researchers

Participatory mapping (and participatory GIS)

- Includes local people in mapping process

- Represents effort to "translate" local geographic knowledge into standard formats

- Aimed at creating maps that are a better reflection of local priorities and POV

Local geographic knowledge

- Key elements:

  - How locations, distance or routes are understood and communicated by local people (e.g. traditional wayfinding, navigation)

  - Local ways of understanding spatial organization

  - Knowledge of spatial patterns of human and environmental phenomena

  - How landscape (or seascape) is conceptualized

  - Marks (using landscape as reference)

  - Can help fill gaps and errors in existing information

- Citizen cartography

  - Online mapping applications have created enormous opportunities for amateur cartographers to participate by making their own maps and sharing data

  - Concern from professional geographers about quality

Geospatial technologies and privacy, and other concerns

- Access to geospatial technologies and spatial data

  - Digital Access Index

    - Measures the ability of people in a country to access computers and the internet, along with other information and communication technologies

      - Most prevalent in first world countries, specifically those with better technology

- Geomatics and privacy

  - 2 BC elevator companies use GPS to track mechanics

    - Employees consider it an illegal intrusion into personal privacy

    - Government says it's reasonable to use such technology to manage staff and ensure workers are where they say they are

  - Car manufacturers and insurance companies tracking drivers

    - Sensors in car allow insurance companies to monitor driving (acceleration, speed, braking) in real time

    - Company analyzes data, if driver is "good" it results in a lower rate

  - Canada considering putting GPS bracelets on more migrants

    - Canadian government is investigating whether or not to expand use of satellites to monitor asylum seekers or potential immigrants who would otherwise be detained

    - Upgrading equipment required to do so

    - Same technology used to track criminals through electronic bracelets

  - Phone company watching you

    - German man obtained information his cell phone company kept on him

      - Over 6 months, the company tracked him more than 35,000 times

    - Mobile location data presents anonymity risk

      - Researchers examined 15 months of anonymous cell phone records for 1.5 million people

        - 4 locations and times were enough to identify anyone

  - RCMP use of UAVs

    - Civil liberties advocates say that the RCMP has begun to patrol the skies without consulting the public

- Geospatial Revolution (part 3) video

  - Mapping has always been driven by war to an extent

    - Mapping and separation of ethnic areas

      - Intended to prevent conflicts

    - Geospatial intelligence for conflict areas

      - Technology provides improved area awareness for combat

        - Knowledge of landscape and buildings

        - Can help detection of IEDs

        - Can help save lives

      - Human geography can help with physical geography

        - Learning what languages are spoken where

        - Learning what is valuable to locals

  - Serving and protecting

    - Geospatial technology in 911 calls

      - Communicating with dispatchers can offer them more information on the situation

      - Police can be managed by inputting location and dispatching them accordingly

      - Information on people and areas can help assist in assessing a situation

    - Hotspot mapping

      - Crimes are mapped, the map shows hotspots for crimes

    - Tracking

      - Sex offenders are monitored

        - If they go somewhere they shouldn't be, police can know about any potential situation before it happens

      - GPS trackers help law enforcement

        - Other people can also track you

          - Can be used to hurt victims

        - Cell phone providers can provide police with location data