MAAE 2300: Fluid Mechanics I

Lab Experiment #1

“Flow Through a Venturi Meter”

Katelynn Bolster

100878176

katelynnbolster@cmail.carleton.ca

October 21, 2014

Group L2B:

William Zorratto

Sagar Rash

Jonah Veenendaal

Wael Daas
Summary:

This experiment was performed to aid in our understanding of pressure and flow rates as discussed in class, using Bernoulli’s equation with simplifying assumptions to determine/calculate these parameters. This experiment made use of a Venturi meter to allow for the comparison of real flow behavior and simplified flow behavior. It also helped us understand the physical meaning of the terms static, stagnation, and dynamic pressures.

Nomenclature:

Given constants

g = gravitational acceleration = 9.81 m/s2

Patm = standard atmospheric pressure = 101325 Pa

water = density of water = 1000 kg/m3

air = density of air = 1.23 kg/m3

Din = intake diameter = 1.045 in

Dth = throat diameter = 0.707 in

Other symbols

P = gauge pressure

Ps = static pressure (Pa)

Pd = dynamic pressure (Pa)

Pstag = stagnation pressure (Pa)

Cv = Venturi coefficient

Ain = cross sectional area of intake (m2)

Ath = cross sectional area of throat (m2)

Vin = velocity at intake (m/s)

Vth = velocity at throat (m/s)

Qactual = actual volume flow rate (m3/s)

Qideal = ideal volume flow rate (m3/s)

h = height of water in the Pitot taps (m)

y = ideal radius of Venturi (cm)

x = distance from beginning of slope of the Venturi (cm)

Flow Analysis:

We start with Bernoulli’s Equation:

Patm+12ρwatervin2+ρwatergh=Pth+12ρairvth2+ρwatergh	(1)

And since all the static taps are at the same height (h = 0), we can say that

Patm-Pth=12ρairvth2-12ρwatervin2				(2)

Then to find the pressure at the point of each Pitot tube, we use

Patm-Pth=ρwatergh						(3)

Putting equations 2 and 3 together, and since vin = 0, we get:

ρwatergh=12ρairvth2							(4)

Rearranging Equation 4, we can solve for the velocity at any tap, vi:

vi = 2ρwaterghiρair							(5)

To obtain Qactual we use the formula:

Qactual=Ainvin							(6)

To obtain Qideal we start with a rearranged continuity equation:

vin= AthvthAin								(7)

Then substituting vin from Equation 7 into Equation 2, we get:

(8)

And solving Equation 8 for vth and substituting in Equation 3:

vth=2ρwaterghρair1-Ath2Ain2							(9)

The following formula can be derived to find the ideal flow rate:

Qideal = Athvth								(10)

And finally, this equation was used to find the Venturi Coefficient:

CV=QactualQideal								(11)

To find the actual pressure at each Pitot tap:

Pi=Patm-12ρairQactualAi2						(12)

The following equations calculate Static Pressure, Dynamic Pressure and Stagnation Pressure, respectively:

Ps=Patm-ρwatergh						(13)

Pd=12ρairvth2							(14)

(15)

The slope of the Venturi, starting from the beginning of the taper through tap 9 (the throat), is assumed to be linear and follow the below relationship:

y=2.6543-0.09133x						(16)

The slope of the Venturi, between tap 10 and the end of the tapering of the Venturi, is assumed to be linear and follow the below relationship:

y=1.79578+0.06707x						(17)

And subbing y into the area of a circle equation, we get the Venturi Area at any tap, where Δx is in cm, measured from the beginning of the tapering of the Venturi:

A(taps 1-9)=π2.6543-0.09133Δx2x10-4		(18)

A(taps 10-16)=π2.6543-0.09133Δx2x10-4		(19)

Experimental Setup and Procedure:

This experiment was conducted in accordance with the instructions outlined for Experiment 1 in the lab manual (“MAAE 2300: Fluid Mechanics I, Course Manual”, S.A. Sjolander, 2014), with no major deviations. The set-up for this experiment is illustrated below:

y

Δx

Results and Discussion:

The experiment was conducted with two different flows, named Flow 1 and Flow 2 accordingly. Example calculations will be shown for Flow 1 only, with the results for Flow 2 being listed as well.

1) Firstly we want to find the Venturi Coefficient for each of the flows. The sample calculations are as follows.

Steps to find Cv for the flow 1:

We use eqn (5) at tap 1 to find the inlet velocity, vin:

vin = 2ρwaterghρair=2(1000)(9.81)(0.044)1.23=26.493 m/s

We use eqn (18) at tap 1 to find the inlet area, Ain:

A(tap 1)=π2.6543-0.09133(1.32715)2x10-4=5.53337x10-4 m2

We use eqn (18) again, this time at tap 9, to find the area of the throat, Ath:

Atap 9=π2.6543-0.091330.8978992x10-4=2.5328x10-4 m2

Using eqn (5) again, this time at tap 9, we can find the velocity at the throat, vth:

vth= 2ρwaterghρair=2(1000)(9.81)(0.221)1.23=59.3736 m/s

The using eqns (6) and (10) we can calculate Qactual and Qideal, respectively:

Qactual=Ainvin=5.5333726.493=0.0146596 m3/s

Qideal=Athvth=2.532859.3736=0.015038 m3/s

And then finally using eqn (11), we can solve for the Venturi Coefficient:

CV=QactualQideal=0.01465960.015038=0.97482

The value of the Cv for Flow 2 was: 0.95897

The Cv rates for both flows were very close to 1, meaning that our calculations using Bernoulli’s equation are a very accurate estimation. We know that the simplifying assumptions that we made in using Bernoulli’s equation (i.e. 1-D, frictionless, incompressible flow) do not perfectly accurately represent the flow, but we can see from this experiment that they do provide us with a fairly accurate way to analyze flows.

Tables of values recorded for Flow 1 and Flow 2, as well as distances between each of the Pitot taps taken during the experiment can be found in Appendix A.

Tables of values calculated for Flow 1 and Flow 2 can be found in Appendix B.

2) Graphs

4) Using equations (13), (14), and (15), we can calculate the static, dynamic, and stagnation pressures for flows 1 and 2. An example calculation is done for Flow 1:

Ps=Patm-ρwatergh=101325-10009.810.313

=9.825447e+04 Pa

Pd=12ρairvth2=121.2359.37362

=3.07053e+03 Pa

Pstag=Ps+Pd=101325 Pa

For Flow 1:

Static P: 98254.47 Pa

Dynamic P: 3070.53 Pa

Stagnation P: 101325 Pa

For Flow 2:

Static P: 96449.43 Pa

Dynamic P: 3070.53 Pa

Stagnation P: 99519.96 Pa

4) The flow through the Venturi meter linearly increases in velocity as the Venturi meter decreases in area approaching the throat (Pitot tube 9), and reaches a maximum velocity at tube 9; then it linearly decreases as the Venturi meter increases in area. The pressure in the Venturi meter behaves in an opposite manner, linearly decreasing with the decrease in area of the Venturi meter, to a minimum at the throat, and then increases linearly as the area of the Venturi meter increases again after the throat.

The trends in actual and ideal static pressures shown in graphs 2 and 3, for Flow 1 and Flow 2 respectively, are in accordance with the Venturi coefficient calculated in Part 1. The actual static pressure for each of the flows very closely matches the ideal static pressure for the flow. Because our Venturi coefficients for Flow 1 and Flow 2 were very close to 1, the actual static pressures for each of the two flows very closely match the ideal static pressures for their respective flows.

Below are three approximate velocity profiles for the flow through the Venturi meter. The first figure, (i), is the velocity profile at the upstream end of the venturi; (ii) is the velocity profile at the throat of the Venturi; and (iii) is the velocity profile near the downstream end of the Venturi.

Conclusions:

The Venturi coefficients for Flow 1 and Flow 2 were 0.97482 and 0.95897, respectively. As these values are both close to 1, we can observe that Bernoulli’s equation gave us a good approximation of our calculations for both flows, however we see that for Flow 1, the slow flow, our calculations were more accurate than for Flow 2, the faster flow, because Cv(1) is closer to 1. We know that using Bernoulli’s equation will not give us perfectly accurate results because of the simplifying assumptions that need to be made to use the equation, such as assuming a 1-D, frictionless, incompressible flow in the Venturi meter. We also found that the static, dynamic, and stagnation pressures for Flow 1 were 98254.47 Pa, 3070.53 Pa, and 101325 Pa, respectively; and for Flow 2 they were 96449.43 Pa, 3070.53 Pa, and 99519.96 Pa, respectively. This lab experiment gave us a good insight into how Bernoulli’s equation can be applied using simplifying assumptions for a real flow, and the physical meanings of static, dynamic and stagnation pressures. Accuracy of this experiment could be improved by taking multiple measurements for many different flows and taking temperature measurements to determine whether there’s a change in density of the fluid.

Appendix B – Tables of Values for Flow 1 and Flow 2:

The below data was calculated/analyzed using Matlab.

The results from the sample calculations shown in the “Results and Discussion” section were calculated by hand.

As a result, the below values are more accurate than those previously shown in this report.

FLOW 1 (all pressures in Pa)

--------------------

Tube  h (m head)    x (cm)      y (cm)         A (m^2)              V (m/s^2)      Gauge Pressure  Actual Pressure  Ideal Pressure

1        0.1360        0.0000     1.3272     0.000553337     46.576451707      -431.6556        100893.3444        100870.7703

2        0.1360        0.0000     1.3272     0.000553337     46.576451707      -431.6556        100893.3444        100870.7703

3        0.1370        1.0000     1.2815     0.000515914     46.747375040      -496.5504        100828.4496        100802.4818

4        0.1470        2.5000     1.2130     0.000462235     48.423437180      -618.5746        100706.4254        100674.0761

5        0.1580        3.5000     1.1673     0.000428087     50.202516699      -721.1971        100603.8029        100566.0868

6        0.1780        5.0000     1.0988     0.000379321     53.285242546      -918.5510        100406.4490        100358.4120

7        0.2010        6.0000     1.0532     0.000348448     56.623273677     -1088.5290       100236.4710        100179.5448

8        0.2360        7.2000     0.9984     0.000313131     61.355421968     -1347.9235        99977.0765          99906.5848

9        0.3130        9.4000     0.8979     0.000253282     70.659264837     -2060.1909        99264.8091          99157.0684

10       0.3050       11.6000    0.9717     0.000296611     69.750426172     -1502.2501        99822.7499          99744.1875

11       0.2370       12.8000    1.0119     0.000321687     61.485274858     -1277.1734       100047.8266         99981.0349

12       0.2220       13.8000    1.0454     0.000343362     59.507736738     -1121.0182       100203.9818        100145.3565

13       0.1920       15.2000    1.0924     0.000374895     55.341071062      -940.3697        100384.6303        100335.4523

14       0.1800       16.4000    1.1326     0.000403023     53.583761647      -813.6872        100511.3128        100468.7598

15       0.1700       17.5000    1.1695     0.000429704     52.074056084      -715.7797        100609.2203        100571.7875

16       0.1610       20.2000    1.2605     0.000499171     50.676881726      -530.4182        100794.5818        100766.8428

Static Pressure: 9.825447000000000e+04 Pa

Dynamic Pressure: 3.070530000000001e+03 Pa

Stagnation Pressure: 101325 Pa

FLOW 2 (all pressures in Pa)

--------------------

Tube  h (m head)    x (cm)      y (cm)         A (m^2)              V (m/s^2)      Gauge Pressure  Actual Pressure  Ideal Pressure

1        0.1660        0.0000     1.3272     0.000553337     51.457773359      -774.7798        100550.2202        100482.5285

2        0.1650        0.0000     1.3272     0.000553337     51.302545936      -774.7798        100550.2202        100482.5285

3        0.1660        1.0000     1.2815     0.000515914     51.457773359      -891.2596        100433.7404        100355.8719

4        0.1860        2.5000     1.2130     0.000462235     54.469503663     -1110.2813       100214.7187        100117.7145

5        0.2040        3.5000     1.1673     0.000428087     57.044270356     -1294.4787       100030.5213         99917.4240

6        0.2420        5.0000     1.0988     0.000379321     62.130468548     -1648.7098        99676.2902          99532.2442

7        0.2870        6.0000     1.0532     0.000348448     67.660919296     -1953.8036        99371.1964          99200.4945

8        0.3530        7.2000     0.9984     0.000313131     75.038526690     -2419.3916        98905.6084          98694.2286

9        0.4970        9.4000     0.8979     0.000253282     89.037947514     -3697.8423        97627.1577          97304.0810

10       0.4700       11.6000    0.9717     0.000296611     86.585640673     -2696.3928        98628.6072          98393.0261

11       0.3520       12.8000    1.0119     0.000321687     74.932164444     -2292.4020        99032.5980          98832.3131

12       0.3170       13.8000    1.0454     0.000343362     71.109328399     -2012.1186        99312.8814          99137.0846

13       0.2650       15.2000    1.0924     0.000374895     65.015945511     -1687.8721        99637.1279          99489.6603

14       0.2410       16.4000    1.1326     0.000403023     62.001966924     -1460.4895        99864.5105          99736.9091

15       0.2250       17.5000    1.1695     0.000429704     59.908466766     -1284.7549        100040.2451        99927.9973

16       0.2090       20.2000    1.2605     0.000499171     57.739110472      -952.0492         100372.9508        100289.7713

Static Pressure: 9.644942999999999e+04

Dynamic Pressure: 3.070530000000001e+03

Stagnation Pressure: 9.951995999999999e+04