Midterm is on Lectures 1 -9 and TSE 1-4

LECTURE 2

What is engineering?

- Problem solving & critical thinking (applied analysis)

- Design (Synthesis)

- Professionalism (formal  organization)

Definition of an Engineer

A person who uses science, mathematics, experience, and judgement to create, operate, manage…Does this in a rational  and economic way with human, societal, and natural resources and environmental constraints.

Role of engineers

- Engineering is a team activity

  - Engineering teams involve individuals with widely different abilities, interests and education who cooperate to advance the project.

  - A typical technical team might consist of

    - Engineers

    - Scientists

    - Technologists

    - Technicians

    - Social workers

    - Skilled workers

Engineer

- Provides the key link between theory and practical applications

  - Has theoretical knowledge

  - Thinks creatively

  - Leads a team towards a common goal (motivation)

  - Knack for obtaining practical skills

- Education

  - Basic requirement is a bachelor's degree

  - Master's or PhD is useful and sometimes preferred

Professional Engineer

In Canada, the title "professional engineer" is restricted by law.

- Only people that can be called "professional engineer" if they have:

  - Demonstrated competence

  - Been licensed by a provincial professional engineering association (PEO in Ontario)

Canadian Engineering Accreditation Board (CEAB)

- Provincial/territorial associations rely on CEAB to evaluate and accredit engineering programs in Canada

- CEAB conducts rigorous and regular reviews of engineering programs across Canada

- They publish a list of accredited programs in Canada

Why Conform to CEAB?

If you want to be a professional engineer, one of the principal requirements is to graduate from a CEAB accredited program.

CEAB requirements?

Practical Experience (4 Years)

Professional Practice Examination

Character attested to by references

***No longer required to be a Canadian Citizen or Permanent Resident***

Technologist

- Works under the direction of an engineers

  - They apply engineering principles and methods to complex technical problems

- Required Education:

  - 3 Year technology program from community college

- Often  supervises work of others and is encouraged to have qualifications recognized by technical society.

- Engineer Vs Technologist

  - Engineering education typically provides greater theoretical depth

  - The technology diploma is more hands-on experience

Skilled Worker

- Skilled in a trade like electrician, welder, plumber, carpenter, machinist, etc

  - Typically carries out the designs and plans of others

- Has great expertise acquired thru apprenticeship or years of experience in the trade

- Most trades have a trade organization and certification procedure

Technician

- Works under the supervision of an engineer or technologist in the practical aspects of engineering

  - Ex. Of Work: Making tests and maintaining equipment.

- Education

  - Completion of two year technician program in a community college

- Association may confer to the title of Certified Engineer Technician

  - Although, Certification is not necessary to work as a technician

Open-Ended Problems

Most Engineering Problems are open-ended

- Meaning they don't have a single solution

Also just because there isn't just one answer, doesn't mean the problem is easier to solve

- What skills are needed to solve open-ended problems?

  - Broad base of engineering knowledge

  - Critical thinking

  - Innovation

  - Perseverance

Engineering Skills

- How to represent a design problem

- How to make assumptions

- How to generate ideas for designs

- How to conduct effective research to solve a problem

- How to plan and schedule activities

- How to make efficient use of resources

- How to organize components and activities of a team design project  (Teamwork)

Ingenuity

- Engineers can find clever ways to solve problems

- Innovation

- Critical thinking

- Problem solving

Canadian Engineering Accomplishments

- Transcontinental Railway - links coast to coast

- De Havilland Beaver 1947  - Water landing plane

- Very High Voltage Transmission - Transmission lines at very long distances with very high voltage

- St Lawrence Seaway - Connects Ocean and great lakes

- Nuclear Power - CANDU Reactor

- Athabasca Oil Sands - Recovering Oil from the Oilsands

- Winnipeg Floodway - Diverts flood water around city

- Alouette Satellite - 3rd Satilite in space

- CanadaARM

- Trans Canada phone Network

- Pacemaker

- Confederation Bridge

- CN Tower

- IMAX

LECTURE 3

Measurements

- Physical quantity that can be observed

  - Measurement compared to a standard quantity

- A measurement consists of two parts

  - A numerical magnitude

  - A name, symbol, or combination of symbols that define the reference standard

Base and Derived Units

- Derived in terms of base units

  - Ex. 1 Newton = 1 Kilogram x 1 Metre / Second squared

- Base Units

  - Are fundamental units from which all other units are derived from

Systeme International d'Unites (SI Units)

- Adopted by almost all countries

  - Notably the United States is an exception

- Metric system introduced as measurement system "for all people, for all time"

7 Base Units in SI

- Metre for length

- Kilogram for mass

- Second for time

- Ampere for electric current

- Kelvin for temperature

- Candela for liminous intensity

- Mol for the amount of a substance

Absolute and Gravitational Unit Systems

SI Unit system

- Absolute  System

  - Mass is a fundamental unit

  - Newton's second law (force=mass x acceleration) is invoked to derive force due to gravity

FPS unit system

- Gravitational system

  - Force is a fundamental unit

  - Mass is derived from Newton's second law.

  - Distance, force and time are all fundamental dimensions, measured in units of feet, seconds and pounds.

Forces and Gravity

- The application of gravity in Newton's Second Law (F=ma) presents several problems.

- It's treated differently in each system of units

- SI: 1N=1kg x 1m/s^2

- American:  1lbf doesn’t not equal 1lbm x 1ft/sec^2

- For American system you have to re-write Newton's 2nd law: F=ma/gravitational constant gc=32.17

Read Chapter 10 for Lecture 3

Lecture 4 - Measurements, Units and Errors

SI Magnitude Prefixes

A Unit may be proceded by a magnitude prefix:

Writing Quantities With Units

- Standardized rules for correctly representing quantities with units:

Number or Formula (italic typed)proceded by a half or full space then SI unit symbols ( Roman typed)

Rules for writing with Units

- Always put a space between numeric value and unit

- Unit symbols are treated like algebraic factors that can be multiplied or divided with numbers or other unit symbols:

  - Multiplication between units is indicated by a centred dot or a half space

  - Division between units is indicated by a slash or a negative superscript

- Unit symbols are not modified with subscripts:

  - -Vmax=200v   NOT V= 200 Vmax

- Relation between numerical quantity and unit symbols must be clear:

  - 3.7 km x 2.8 km for an area, NOT 3.7 km x 2.8

- Unit symbols not mixed with unit names in sentences:

  - Use kg/m^2 or kilogram per metre squared NOT kilogram/m^2

- Long numbers, written in groups of 3, the groups should be seperated by a non-breaking half space rather than a comma or period:

  - 930 000 000 miles NOT 930,000,000 miles

  - 21 298.046 83 miles NOT 21298.04683 miles

Fixed, Scientific and Engineering Notation

- Fixed Notation

  - Ordinary decimal notation

    - Ex. 30 140.0

- Scientific Notation

  - Ex. 3.014 x10^4

- Engineering Notation

  - Exponent of 10 is a multiple of 3 to correspond to SI prefixes

    - Ex. 30.14x10^3

Dimensions

- Dimensions differ slightly from units

  - Ex. Speed

    - Dimensions = distance/time

    - Units are known as km/hr, or mm/s, etc.

- Fundamental dimensions

  - Distance, time, mass, etc.

Measurement

Property of a physical object that can be represented using a real number

- Not like counting, measurement is not exact

Quantities are usually measured with an instrument

- Calipers, scale, measuring cup, etc.

Uncertainty

- Measuring instruments are not perfect

True Value - Measured Value = Measurement Error

Systematic Errors

- Constant deviation from the true value, is called a bias or offset.

- Error has the same measurement and sign when repeated measurements are made under the same conditions

- Systematic Errors can be detected by

  - Calibration

  - Comparison with results obtained with an independent method

Lecture 5

How do engineers Communicate?

- The 3 ways Engineers convey information:

  - Written Documents

  - Oral Presentations

  - Graphically

- Sometimes a combination of ways is used

Engineering Graphics

- The majority of engineering information is transmitted by graphical means

- But written and verbal communication is also important to engineering communication

Advantages of Graphical Communication

- Clear

- Concise

- Efficent

- Reduced Ambuiguity

Engineering Graphics

- Graphics is a universal language

- Engineering graphics are directed towards graphical problem solving

- A good engineering Graphic is worth a thousand words

Technical Illustration

- Technical Illustrations are used to describe products in cataloagues, user and maintenance manuals.

- Pictorial sketching is an important engineering communication tool

- Helpful to describe difficult to visualize components or assemblies

- How to put components together, NOT how to make them

- There common types of Technical Illustration:

  - Perspective

  - Oblique

  - Isometric

Oblique Projection

- Front face of the object is parallel to the viewer, therefore the face is true size

- The projection lines do not converge into a vanishing point

Isometric Projection

- Parallel lines remain parallel instead of converging to a vanishing point

- Axes are 120 degrees apart

Orthographic Projection

- Most important graphical drawing method

- 2D representations of a 3D figure

- Extensively used in engineering

- Useful when technical information is needed

- Enable parts to be made

- Sometimes the isometric view is included into the Orthographic Projection

- Geometric backbone of a technical drawing

Third Angle Projection

- The object is placed in the third quadrant and then view from the first quadrant

- Projection planes are between the observer and the object

- Used in Canada, US and Britian

- Top view over the front view, then to the right of that is the right view

First Angle Projection

- The object is placed in the first quadrant and then viewed from the third quadrant

- The object is between the observer and projection planes

- Used in most of the world

- Right view, to the right of that is the Front view then below is Top view

Selection of views

- The most descriptive view is usually selected to be the front view

- The remaining views are selected to be the ones with the least hidden lines

- If a tie occurs, we use the conventional view (right and top)

Read Chapters 12-14

Lecture 6 - Engineering Graphics

Standard Drawing Layout

- Three-view orthographic projection

  - Top view located above the front view

  - Right view located to the right of the front view

  - The views must be aligned

Orthographic Projection - Lines

- When two or more lines concide, there is a convention called 'precedence of lines' to establish which is visible:

  - Visible Object Lines

  - Hidden Object Lines & Cutting plane lines

  - Centre Lines

  - Extension & Leader Lines

- Line Types:

  - Object Lines: indicate all visible edges of an object; must stand out and be apparent to the eye

  - Hidden Lines: shows object likes that are hidden from the view

  - Cutting Plane Lines: indicated edge view of an imaginary cutting plane (Section views)

  - Centre Lines: indicates centres of holes and symmetrical features of an object

Dimensions

- Two fundamental rules for dimensioning:

  - Dimensions must be describe the complete geometry of the part

  - Redundant dimensions should be avoided

- More specific rules apply to the geometry, the placement and style of the dimension

  - Dimensions must be from visible lines, not hidden features

  - Repeated regular identical features may be dimensioned only once, along with a small note indicating quantity, etc.

  - The designer should be able to locate dimensions in a way to indicate how tolerances accumulate

- Dimensioning Circles

  - Radii are dimensioned using a leader line with an arrow at he end of the line

    - Capital "R" indicates radius

    - Dimension rounds and fillets with their radii

  - Similarly, a diameter has a leader line with an arrow

    - The diameter symbol indicates diameter

    - Never dimensions holes or cylinders with radius, use diameter

  - Angles

    - Fractional degree measurements can be represented with decimals or minutes and seconds

    - Dimensional lines are drawn as arcs

- Rule Zero

  - When dimensioning a drawing, set the dimensions size to a readable size for when it's printed in it’s final size

- Rule One

  - The first dimension height should be three times the letter height, from the object.

  - Successive dimensions should be two times the letter height apart

- Rule 2

  - Place dimensions between the views sharing the same dimensions

- Rule 3

  - Dimension the most descriptive view

- Rule 4

  - Dimension from visible (object) features, and not hidden features

- Rule 5

  - Give an overall dimension, omitting the last in a chain of dimensions

- Rule 6

  - Organize the dimensions to reduce the clutter in the drawings

- Rule 7

  - To avoid errors and confusion, do not duplicate dimensions

- Rule 8

  - Dimension lines never cross any other lines, unless absolutely necessary

- Rule 9

  - Extension lines may cross other extension lines, or object lines, if necessary

- Rule 10

  - Do not leave gaps where extension lines cross object lines or other object lines

- Rule 11

  - Where possible, place object lines outside the objects

- Rule 12

  - Dimension Cylinders in the RECTANGULAR view, not CIRCULAR

  - Only Diameter, NOT Radii

  - BUT dimension inside of circular holes in view where they look like holes, in the circular view

  - Dimension circular holes in the view where they look like holes ie If model is not a cylinder

Orthographic Projections

- Advantages

  - Able to contain a high degree of detail

  - Angle and length measurements can be made on all views (exceptions do exist)'

- Disadvantages

  - Effort is needed to interpret the object

  - Many conventions must be followed - not as easy as the other three

Read Chapter 13

LECTURE 6 - Engineering Graphics

Nomographs

- Also called an alignment chart, is a calculating chart with scales that contain values of three or more mathematical variables

  - Widely used in engineering, industry, and the natural and physical sciences

Why do we need CAD?

- CAD is an effective way to communicate your ideas and designs

  - Used to produce working drawings

- Manufacturing is becoming more integrated, more global

- Systems are becoming more complex

- Working drawings are needed in all disciplines

  - Electrical, Civil, Mechanical, Aerospace, Manufacturing & Architecture

CAD Advantages

- Increased accuracy

- Increased Speed

- Easy Revision

- Better Design analysis

- Better presentation

- Libraries of drawing aids

- Improved filing

Solid Modeling

- The software creates 3D virtual objects

- The virtual objects appear as axonometric projections

- The models can be virtually manipulated, just like a physical object

- Ex. Products:

  - Pro/ENGINEER

  - Solid Edge

LECTURE 8 - Introduction to Design

What is design?

- Design is a creative process

- Incorporates all areas of engineering education

- Design is basically what engineers do

- Almost always involves a team effort

- An iterative process

- Open-ended problems, often with many possible solutions

- Usually, only a handful of good solutions

Engineering Design

- Identifying problems, solving them, and implementing the solutions.

Good Design Requires:

- Organization

- Teamwork

- Communication

In addition, it requires, creativity and critism is essential

The key is knowing when to criticize and when to be creative

- Before a problem can be solved, it has to be identified

- Needs should be categorized and defined in a Statement of Requirements

- Most design problems can be categorized as:

  - Systems Problem

  - Product Problem

Design Loop

Method of Design

- Recognition of Need

  - A need can be either very obvious or hidden

  - Usually very vague

  - The vagueness of the need can cause the design process to become open-ended

  - Often the need is written in a Statement of Requirements

- Definition of the design problem

  - Critical for a good design

  - May require the gathering of information of the topic (research)

  - Must define the problem correctly in order to provide a solution

- Definition of the design criteria

  - Design criteria are performance standards to be met by the design

  - Design constraints are limitations placed on the designer

  - The design criteria and constraints must be quantitative values

- The design loop

  - Synthesis

    - Suggesting ideas or methods to solve the problem

  - Analysis

    - Calculating the expected result of each idea or method

  - Decision-making

    - Deciding which alternative is best

- Optimization

  - Optimum implies a compromise between cost and benefits - the best design at a reasonable cost

  - If the design is not optimum, the design loop may be needed to be reiterated

- Evaluation

  - Once the design is completed, it must be reviewed

  - The senior engineer and "others" have to approve of the design before they finalize it

  - If the design is flawed, it must be iterated and send back to step 2

- Communication

  - Once the design is approved, it can be built or manufactured

  - The final design must be communicated effectively

    - Reports

    - Technical Drawings

Creativity and Innovation

- Are required for the design process

  - Brainstorming

Read Chapter 16-17

LECTURE 9 - Rapid Prototyping with 3D Printing

- Ideal to complement to CAD

- Process to build concept and working models from 3D CAD Data

  - Several different techniques available

- Used for

  - Communication/collaboration

  - Time reduction

  - Cost reduction

Additive and Subtractive RP

- There are two main methods of Rapid prototyping, which are derived from similar approaches in sculpture

  - Additive Rapid Prototyping (ARP)

  - Subtractive Rapid Prototyping (SRP)

3D Printing

- 3D printing is a variation of rapid prototyping

- A 3D printer is less expensive, easier to use and usually faster then a RP System

- 3D printer is small and can fit in an office environment

Read Chapters 12-17