# Process Design, Facility Layout

Contents

Introduction and Process Types	1

Definitions	1

Process Design	1

Make or Buy	1

Classify Processes in 4 Categories	1

Job Shop	1

Batch Process	2

Repetitive/Assembly line	2

Continuous	2

Automation	4

Fixed Automation	4

Programmable Automatoin	4

Flexible Automation	5

Process Design	6

Methodology	6

Example 5	8

Service Process Fail-Safeing	9

Customer Perception in Service Process Design	9

Types of Layout	10

Product Line Layout	10

Process (Function) Layout	10

Cellular Layout	11

Warehouse Layout	12

Retail Layout	12

Office Layout	12

Layout Design Steps	12

# Introduction and Process Types

## Definitions

### Process Design

- Determining the form and function of how goods and services will be produced

### Make or Buy

- Decide whether   to make or buy a part

- Often a strategic decision

- Fctors

  - Existing/desired core capabilities

  - Available capacity

  - Quality

  - Whether demand is steady or temporary

  - Secrecy of technology

  - Costs

## Classify Processes in 4 Categories

- Depends on

  - Variety of products

  - Flexibility of products

### Job Shop

- Definition

  - Process type used when a low quantity of high-variety customized goods or services is needed

- Characteristics

  - Process is intermittent

    - Work shifts from one small job to the next; each slightly different requirements

  - Highly flexible machines

  - Skilled workers

- Example

  - Tool and die shop which makes one-of-a-kind tool and die

- Managerial Challenge

  - Scheduling jobs so due dates are met

  - Resources are used as much as possible

### Batch Process

- Definition

  - A type of process used when a moderate volume and variety of a goods or service is desired

- Characteristics

  - Equipment is not as flexible as job shop

  - Workers are not as skilled as job shops

  - Intermittent process

- Example

  - Bakeries bake/sell cookies in batches

- Managerial Challenge

  - Scheduling batches in order to meet planned production and demand while utilizing the resources at a high level

### Repetitive/Assembly line

- Definition

  - A type of process used when higher quantities of more standardized good or services are needed

    - Production line

      - A sequence of machines/workstations that perform operations on a part/product

    - Assembly line

      - A production line where parts are added to a product sequentially

- Characteristics

  - Low level of skilled workers

  - Slight flexibility in equipment

  - Lines can be either

    - Machine paced (same speed)

    - Human paced (variable speed)

- Example

  - Automobiles

- Managerial Challenges

  - Capacity balance

  - Technology management

  - Quality

  - Materials management

### Continuous

- Definition

  - Used when a high volume of highly standardized output it required

- Characteristics

  - No variety in output

  - No need for flexible equipment

  - Low skill wokers

  - Product is unable to be counted

- Example

  - Sugar

- Managerial Considerations

  - Same as assembly line

    - Capacity balance

    - Technology management

    - Quality

    - Materials management

  - But because of faster production speed

    - Greater care required for automated control of flow

    - Start/stop production is challenging

|  | Job Shop | Batch | Repetitive | Continuous |
| --- | --- | --- | --- | --- |
| Product variety | Customized | SEMI-Standard | Standardized | Highly Standardized |
| Volume | Low | Low-Moderate | High | Very High |
| Equipment Flexibility | Very High | Moderate | Low | Very Low |

- ABOVE MATRIX DEPICTS: Ideal to have the production process capabilities, i.e equipment, to match the requirements (i.e. product variety)

- Consider

  - Products often go through lifecycles, requiring a switch in production process

- Hybrid Process

  - Process with elements of other process types embedded in them, i.e.:

    - Job shop may evolve to repetitive; having part of the process run as a job shop while the other acts in a repetitive fashion

- Focused factories

  - Handle both high quantity and high variety;

- Choice of operation process

  - Nature

  - Shape

  - Size

  - Quantity

  - Variety

  - Other materials

# Automation

- Definition

  - Using machinery and equipment with sensing andcontrolling devices that enable it to operate automatically

- Disadvantages

  - Costly

  - Less flexible

  - Adverse effect on productivity (morale)

## Fixed Automation

- Most rigid

- Perfected by Ford Motor Company

- Characteristics

  - High cost, specialty

- Advantages

  - Low unit cost

  - High volume

- Limitations

  - Minimal variety

  - High cost of making maor changes

## Programmable Automatoin

- Characteristics

  - High cost, general purpose equipment controlled via a computer program which provides

    - Sequence of ops

    - Details of each ops

  - As easy to change process as it is easy to change software

  - Type of automation has capability of economically providing a fairly wide variety of low volume products (i.e.: batch)

- Numerically controlled machines

  - Machine whih perform ops by following mathematical processing instructions

  - Best used whenL

    - Parts are processed in frequent, small batch

    - Parts geometry are complex

    - Close tolerance is required

    - Mistakes are costly

    - The possibility of making changes is high

  - Disadvantages

    - Higher cost

- Robot

  - Machine consisting of a mechanical arm, a power supply, and a controller

  - Advantage

    - Can handle a variety of tasks

    - Level low robots

      - Low: follows fixed instruction

      - Programmable

      - High: can recognie objects, make simple decisions

    - Robots can be powered

      - Via aire (pneumatically)

      - Via water (hydraulically)

      - Via electricity

## Flexible Automation

- Uses equipmenttt thhhat is more customized than programmable

- Key difference

  - Flexible automation requires less change, thereby being almost continuous and batch

- Machine center

  - Machine capable for performing a variety of ops

- Flexible manufacturing system (FMS)

  - A group of machine centers controlled by a computer with automatic material handling and robots or other automated equipment

  - Advantages

    - Offers reduced labour costs

    - More consistent quality

    - Lower capital investment

    - Higher flexibility

    - Relatively quick change over

  - Limitations

    - Systems can handle relatively narrow range of part variety

    - Must be used for family/similar parts

    - Longer planning and development times of increased complexity and cost

  - Suitability depends on

    - Variety

    - Quantity

- Computer Integrated Manufacturing

  - A system for linking a broad range of manufacturing, other activities through computer systems

  - Able to

    - Orders

    - Schedules

    - Purchasing

    - Control inventory

    - Shop controls

    - Distribution in manufacturing

# Process Design

- Determine

  - Form and untion of how production of g/s is to occur

- Involves

  - Identifying activities and their sequences, resources, controls directly needed in production

## Methodology

- Define production process;

  - Determine how complete inputs should be; these are make or buy decisions

  - Set production process objectives

    - Capacity, flexibility

    - Type of process

      - Job shop

      - Assembly

      - Repetitive

      - Continuous

    - Technology and extent of automation, production start date

  - Determine the nature of the process

- Production Process Development

  - Conceptualize the Design

    - Answers:

      - How do you get from input to output

      - What is the sequence of major operations?

        - Develop a few alternative concepts

          - Incremental

            - Do one step at a time

          - Hierarchal (top down)

            - Divided into 2 operations, then divided into subsystems

        - Usually follow a process flow design to show process and materials going through

        - Evaluate each alternative

  - Make an embodiment of the design

    - Choose one process concept and comlete the design

    - Build a prototype process and test,

      - Determine resources needed

      - Estimate cost, quality

    - Refine process, re-evaluate

  - Create a detailed design

    - Finalize the process specifications

      - Determine the specific machine

      - Design plant layout

      - Design the work centers

- Buy machines, recruit workers and start trail runs

- Process flow diagram

  - Shows the operations and movement of materials through ops

  - Triangle

    - Storage

  - Rectangle

    - Operation

### Example 5

- Recipe

  - Mix flour, yeast, salt

  - Knead the dough

  - Shape the dough

  - Bake in over

  - Let rise in pan

  - Bake in over

  - Let bread cool

- Machine

  - Mixes, knreads

  - Automatically shapes,

  - Drops in pan

  - Bakes

  - Sliced and bagged

- Process flow diagram

## Service Process Fail-Safeing

- Services are vulnerable to quality and delivery problems

  - Usually during delivery, when difficult to fix problems before customer notices

- Company’s must identify points of failure

- Example

## Customer Perception in Service Process Design

- Don’t raise customers expectations too high in the beginning

- End the service positively

- If pleasurable, divide into segments

- If painful, keep as one trip

- Let customer control part of the process

- Communicate the evidence to customers

# Types of Layout

- Layout refers to the location of

  - Departments

  - Work centers

  - Equipment in facility

- Common reasons for redesign

  - Inefficient operations (excessive handling)

  - Change in design of product

  - Change in quantity mix of output

  - Change of methods, equipment

## Product Line Layout

- Arranges production resources linearly according to the progressive steps by which a product is made

- Since one, very few items, involved, it is possible to arrange entire layout to correspond to production requirements of a product

- Certain compromises may be made

  - Automatic car wash dispenses same amout regardless of level of dirt

- Efficiency

  - Product layouts achieve high degree of labour and equipment utilization (tends to offset cost)

- Preventive maintenance

  - Periodic inspections and replacement of worn parts or those with high failure ates

  - Reduce probability of breakdowns during ops

- Advantages

  - Low cost production

  - Simplified accounting

  - Purchasing

  - Inventory controls

- Disadvantages

  - Equipment cost

  - Dull, repetitive jobs

  - Repetitive stress injuries

  - Inflexible response to change in quantity of output or in a product mix

## Process (Function) Layout

- Definition

  - Arranges production resources together according to similarity of function

    - Intermittent process

      - Discontinuus work flow

- Batch require movement of batch quantity may require equipment like forklift

- Use of general purpose equipment

  - Provides the flexibility necessary  to handle a wide range of production range of production sequence

  - Often has skilled workers

- Advantages

  - System is much less vulnerable to shutdown caused by mechanical failure

  - Idle equipment usually available

  - Lower maintenance costs

  - Personnel become more skilled in specific area

  - Reduce necessary investment of spare parts

- Disadvantages

  - Routing, scheduling must be done continuously

  - Material handling is inefficient

  - Unit cost is generally higher

  - In process inventories can be sustained

  - Not uncommon to have equipment utilization under 50%

## Cellular Layout

- Definition

  - Layout in which different machines areee arrrranged in a cell thhhat can process items that have similar processing equipment

    - Group technology

      - Grouping items together with similar design of manufacturing characteristics into part families

- Observe

  - Arranged to handle ops necessary for group (family) of similar arts

  - Thus, all parts flow through the same route with minor variations

  - Cell often in the shape “U”

    - Permitting teamwork via communication

    - Increasing flexibility because worker can work at 1 station and quickly another

- Benefits

  - Faster processing time

  - Increased capacity

  - Less material handling

  - Less work in process inventory

  - Reduced set up lines

- Design Characteristics

  - Size

  - Shape

  - Material

  - Features

  - Function

- Manufacturing characteristics

  - Type of service

  - Tolerance/precision

- Once items that are similar are collected

  - Form families

  - System developed to store, retrieve specific info on familis

  - Allows for a part to be manufactured for a family, alleviated the individual specification frustrations

- Disadvantages

  - Time consuming

  - Considerable amount of data

- Primary methods

  - Visual inspections

  - Examination of design

  - Production delay

  - Production flow sequence and routing analysis

## Warehouse Layout

- Factors affecting warehouse

  - Cold storage

  - Size, frequency of order

    - High frequency placed near docks

    - Low frequency at front

  - Correlated items together

  - Consider cost, time of shipping while placing

## Retail Layout

- Objective

  - Cost minimization

  - Product   flow

  - Customer flow

  - Sales influencing

## Office Layout

- Less need for paper works allows for open offices

# Layout Design Steps

- Determine the location of receiving and shipping

- For product layout

  - Fit the process flow diagram onto a sketch of the factory floor; determine the approximate location of each part of the process

- For process layouts

  - Determine the expected work flow between each pair of department and place the two departments with the highest workflows closest toether

- Continue until all departments are located

- Keep special requirements of machine in mine (i.e.: heavy press needs strong foundation)

- Allow space for machines, in-feeds, out-feeds, workers and carts

- Keep rearranging the plan using feedback from workers until you find what works best

- On the factory floor, paint outlines of machines in-/out-feeds

- Walk through the normal sequence of activities

- Run the electricity through