# Chapter 7	Design of Work Systems

Table of Contents

Job Design	1

Efficiency School	1

Behavioural School	1

Specialization:	1

Advantages	1

Disadvantages	1

Introduction to Job Design	2

What does Job Design Involve	2

Objectives	2

Approaches	2

Job Design Decisions	2

Trends in Job Design	3

Physical Considerations in Job Design	3

Behavioural Approach	3

Teams	4

Benefits of teams	4

Methods Analysis	4

Worker-Machine Chart	5

Motion Study	5

Working Conditions: Government Regulation	5

Working Conditions	6

Noise and Vibrations	6

Work Breaks	6

Levels of Illumination	6

Safety: Protection Against Hazards	7

Safety	7

Ergonomics	7

Work Measurement	8

Work Measurement	8

Standard time	8

Stopwatch Time Study	8

Number of cycles	8

Solved Problem #2	9

Normal Time	9

Standard Time	10

Time Study Example	10

Cautions on Time Studies	10

Predetermined Element Times	11

Work Sampling	11

Compensation	11

Output-based (piece rate) pay	12

Question	12

Question	12

Question	13

# Job Design

- Job design involves specifying the content and methods of job

  - What will be done

  - Who will do the job

  - How the job will bob will be done

  - Where the job will be done

  - Ergonomics

## Efficiency School

  - A refinement of Taylor’s scientific management concepts

  - Systematic, logical approach to job design

  - Labour cost reduction

## Behavioural School

  - Emerged during the 1950s

  - Emphasizes satisfaction of wants and needs

  - Reminded managers of the complexity of human beings

## Specialization:

### Advantages

- For company

  - Simplifies training

  - High productivity

  - Low wage cost

- For worker

  - Low education and skill requirement

  - Minimum responsibilities

  - Little mental effort needed

### Disadvantages

- For Company

  - Difficult to motivate quality

  - Worker dissatisfaction

    - Absenteeism

    - High turnover

    - Disruptive tactics

- For Workers

  - Monotonous work

  - Limited opportunitiy for afvancement

  - Little control over work

  - Little opportunity for self-fulfillment

# Introduction to Job Design

## What does Job Design Involve

- Job design involves specifying the content and methods of job

  - What will be done

  - How the job will be done

## Objectives

- Objectives of job design

  - Productivity

  - Safety

  - Quality of work life

## Approaches

- Two approaches to job design

  - Efficiency school

  - Behavioral school

## Job Design Decisions

- Ultimate Job Structure

  - Who

    - Mental and physical characteristics of the workforce

  - What

    - What tasks will be performed

  - Where

    - Geographic locale of the organization;

    - Locations of work areas

  - When

    - Time of day

    - Time of occurrence in workflow

  - Why

    - Organizational rationale for the job

    - Objectives and motivation of the workforce

  - How

    - Method of performance and motivation

## Trends in Job Design

  - Quality control as a part of workers job

  - Cross-training workers to perform multi-skilled jobs

  - Employee involvement and team approaches to designing and organizing

  - Extensive use of temporary workers

  - Automation of heavy manual work

  - Creating alternative workplaces

  - Organizational commitment to providing meaningful and rewarding jobs for all employees

## Physical Considerations in Job Design

- Work physiology

  - sets work-rest cycles according to the energy expended in various parts of the job.  The harder the work, the more the need for rest periods.

- Ergonomics

  - is a term used to describe the study of the physical arrangement of the work space together with tools used to perform a task. Fit the work to the body rather than forcing the body to conform to the work.

## Behavioural Approach

- Job Enlargement

  - Giving a worker a larger portion of the total task by horizontal loading

- Job Rotation

  - Workers periodically exchange jobs

- Job Enrichment

  - Increasing responsibility for planning and coordination tasks, by vertical loading

## Teams

- Self-directed teams

  - Groups who perform the same function and are empowered to make certain decisions and changes in their work.

### Benefits of teams

  - Fewer managers are necessary

  - Higher quality

  - Higher productivity

  - Greater worker satisfaction

# Methods Analysis

- Methods analysis

  - breaks down the job into sequence of tasks and elements and improves it.

  - Basic Procedures

    - Identify the job to be studied and gather information

    - Discuss the job with the operator and supervisor

    - Study and document the present method

    - Question the present method and propose a new one

- Process Charts

    - Chart used to examine the overall sequence of an operation by focusing on  movements of the operator or flow of materials

    - Ask:

      - Why is there a delay or shortage at this point?

      - How can travel distances be shortened/avoided?

      - Can an operation be eliminated?

      - Can the sequence be changes?

      - Can similar activities be grouped?

      - Would additional or better equipment help?

  - Process Chart: Example

## Worker-Machine Chart

- Helps to see portions of work cycle during which an operator and equipment are busy or idle.

- Can help to determine how many machines one operator can manage.

- See Figure 7-2

## Motion Study

- Motion study is the systematic study of the human motions used to perform an operation.

- Motion economy principles:

  - guidelines for designing motion - efficient work procedures

- Analysis of elemental motions (therbligs):

  - basic elemental motions into which a job can be broken down

- Micro motion study:

  - use of motion pictures and slow motion to study motions that otherwise would be too rapid to analyze

- Simultaneous hand motion chart:

  - a chart that shows the elements performed by each hand, side by side, over time

## Working Conditions: Government Regulation

- Canadian Labour Code

  - safety and other work standards (e.g.. min wage)

  - Workers have rights to:

    - refuse dangerous work,

    - participate in improving health & safety,

    - know about hazards in the workplace.

- Canadian Centre for Occupational Health and Safety

- Workplace Hazardous Materials Information System (WHMIS),

    - mandates proper labeling of hazardous material

    - making available material safety data sheets.

# Working Conditions

- Working conditions

  - Termperature and humidity

  - Ventilation

  - Illumination

  - Ergonomics

  - Noise and vibrations

  - Work breaks

  - Safety

  - Cause of accidents

## Noise and Vibrations

  - Noise:

    - Selection and placement of equipment

    - Acoustical walls and ceilings or baffles

    - Provide earplugs or protective devices

  - Vibration:

    - Padding, stabilizers, shock absorbers, cushioning or rubber mountings

## Work Breaks

  - Efficiency declines as day wears on

  - Work breaks can cause an upward shift

  - Depends on amount of physical/mental efforts

## Levels of Illumination

| Task condition | Type of task or area | Illumination level | Type of illumination |
| --- | --- | --- | --- |
| Small detail, extremely accurate | Sewing, inspecting dark materials | 100 | Overhead ceiling lights and desk lamps |
| Normal detail, prolonged periods | Reading, part assembly, general office work | 20-50 | Overhead ceiling lights |
| Good contrast, fairly large objects | Recreational facilities | 5-10 | Overhead ceiling lights |
| Large objects | Restaurant, stairway, warehouse | 2-5 | Overhead ceiling lights |

## Safety: Protection Against Hazards

  - Proper lighting

  - Clearly marked danger zones

  - Protective equipment

  - Safety devices

  - Emergency equipment

  - Housekeeping

  - Thorough training

  - Enforcement of procedures

## Safety

  - Requires constant attention

  - Occupational Safety and Health Standards

  - WHMIS training required

  - Two main causes of accidents:

    - carelessness

    - hazards

## Ergonomics

  - Involves fitting the job to the worker’s capability and size.

    - Design of equipment, work methods, work space

    - Remove awkward movement

    - Try to prevent common workplace injuries (e.g. back injuries, repetitive motion injuries)

# Work Measurement

## Work Measurement

  - Determine how long it should take 
to do a job.

    - Stopwatch time study

    - Predetermined element times

    - Work Sampling

## Standard time

  - The amount of time it should take a qualified worker to complete a specific task

    - working at a sustainable rate,

    - using given methods, tools and equipment, raw materials, and workplace arrangement.

## Stopwatch Time Study

  - Development of a time standard based on observations of one worker taken over a number of cycles.

  - The basic steps in a time study:

    - Define the task to be studied

    - Determine the number of cycles to observe

    - Time the job

    - Compute the standard time

## Number of cycles

  - Number of cycles depends on:

    - variability of observed times,

    - desired accuracy, and

    - desired level of confidence for the estimated job time

  - Formulas

  - Notation

    - z =Number of Normal standard deviations needed for desired confidence

    - s =Sample standard deviation

    - a =Desired accuracy proportion (= percentage/100)

    - =Sample mean

    - e =Accuracy amount or maximum acceptable error amount

    - n = number of observations (= sample size)

## Solved Problem #2

  - Determine number of observations needed to achieve a specified maximum error, with a confidence of 95.5 percent.

    - mean = 5.2 minutes

    - standard deviation = 1.1 minutes

  - Determine:

a.	A maximum error of +/- 6% of the sample mean.

b.	A maximum error of .40 minute.

- Given

  - x-bar  = 5.2 minutes

  - z = 2.00 for 95.5%

  - s = 1.1 minutes

  - a = .06

  - e = .40

- Solution

## Normal Time

  - Normal time (NT) = Observed time  x Performance rating

    - Observed Time  is an average of recorded times

    - The Performance Rating expressed as a decimal. Performance Rating of a person working 10% faster than normal = 1.10 or 110% of normal time.  Working 10% slower = 0.90 or 90% of normal.

    - Performance rating is subjectively assigned

    - Can sum normal times for several elements

## Standard Time

  - Standard Time = Normal Time * Allowance Factor

    - Standard time = NT x (1 + Allowance % )

based on job time

    - used when different tasks have different allowances

- Standard Time = Normal time * Allowance Factor

  - Standard time =                  NT        .

1 – Allowance %

based on time in workday

    - used when different tasks have similar allowances

## Time Study Example

  - You want to determine the standard time for a job.  A worker produced 20 items in an 8 hour day.  As the worker was nervous under observation, it was estimated that the employee worked about 10% faster than normal.  Allowances for the job are 25% of the normal time.

  - What are the normal and standard times for this job?

- Normal time = Observed Time  x (Perf. rating)

= (480 minutes/20) x (1.10)

= 26.4 minutes

## Cautions on Time Studies

  - Analyst must be familiar with task

    - Workers may attempt to shorten times

    - Abnormally short times should be discarded

    - Abnormally long times should be investigated

  - Jobs need to be broken down into basic motions

    - Many jobs can’t be observed like this

  - Costly and disruptive: benefits must outweigh cost

    - Can also use historical or published elemental times to determine times for a task

    - Need high skill to describe task in basic elements

## Predetermined Element Times

  - Published data based on extensive research on element times

    - Methods Time Measurement (MTM) tables

    - Time Measurement Unit (TMU)

    - Many tables for a variety of moves and conditions

      - distance, difficulty, weight, degree of turn, etc.

    - Requires much time and skill

## Work Sampling

  - technique for estimating the proportion of time that a worker or machine spends on each activity or is idle.

  - Reason

    - Make brief observations of a worker or machine at random intervals

    - Does not require

      - Timing an activity

      - Continuous observation

  - Steps

    - Identify worker to be studied

    - Notify workers and supervisors to avoid arousing suspections

    - Compute initial estimate of sample size

    - Determine estimated proportion of time spent on specified activity

    - Begin taking observations. Re-compute required sample size periodically

    - Develop random observation schedule

## Compensation

  - Time-based pay

    - Compensation based on time an employee has worked during a pay period

  - Output-based (piece rate) pay

    - Compensation based on the amount of output an employee produces during a pay period

  - Group Incentive Plans

    - Sharing of productivity gains with employees

      - Output and cost reduction (gain-sharing)

      - Increases in profit (profit-sharing)

- Skill/Knowledge-Based Bonus Plans

    - reward workers who undergo training 
to increase their skills/knowledge

## Output-based (piece rate) pay

  - Compensation system should be:

    - Accurate

    - Easy to apply

    - Consistent

    - Easy to understand

    - Fair

## Question

- A job is said to be enlarged vertically if the employee is involved with which of the following?

  - The job’s planning

  - The job’s organizing

  - The job’s inspecting

  - All of the above

  - None of the above

## Question

- Which of the following is the Normal Time for a job whose observed performance time is 10 minutes with a performance rating of 90%?

  - 90 minutes

  - 9 minutes (10 * 0.9 = 9)

  - 1.10 minutes

  - 11.111 minutes

  - None of the above

## Question

- Which of the following is the Standard Time for a job whose Normal Time is 20 minutes and whose Allowances are 5% of Normal Time?

  - 19.05 minutes

  - 21 minutes

    - St = nt (1+allowances) = 20 (1.05) = 21

  - 400 minutes

  - 450 minutes

  - None of the above