Maintenance

- Explain what maintenance is, its importance in production systems, and the range of maintenance activities

    - Maintenance

      - All activates that keep facilities and equipment in good working order so that a system can perform as intended

    - Breakdown (emergency) maintenance

      - Dealing with breakdowns (repair/replacement) only after they occur

    - Preventive maintenance

      - Reducing or avoiding breakdowns through a program of lubrication, bolt tightening, cleaning, inspection, and replacement of worn parts

    - Reliability-centered maintenance (RCM)

      - An approach that reduces the effect of each major cause of failure for equipment functions

  - Maintenance activities

    - Building and grounds

      - Appearance and functioning of buildings, roads, parking lots, lawns, fences, and the like

    - Equipment maintenance

      - Responsible for maintaining machinery and equipment in good working condition and making all the necessary repairs

      - **focus of the chapter

  - Goal of maintenance management

    - Keep the production system in good working order at minimum cost

- Describe preventive maintenance and determine the economic amount of preventive maintenance

  - Goal

    - Reduce breakdowns

    - Avoid associated costs

      - Loss of output

      - Idle workers

      - Unexpected repairs

  - Most preventive maintenance is periodic

    - Can be scheduled according to

      - The result of planned inspections that reveal a need for maintenance

      - According to the calendar (passage of time)

      - After a predetermined number of operating hours (or miles for trucks)

  - Important issue

    - Frequency

      - As the time between two periodic maintenance increase, the cost of preventive maintenance per unit time decrease while the risk (and cost) of breakdowns increase

    - Goal

      - Balance two cost

  - Given preventive maintenance (PM) in interval L,

    - If a breakdown occurs before L,

      - then a breakdown maintenance cost will be incurred, and machine will be repaired and the next time between breakdowns (TBB) will start

    - If there is no breakdown before L,

      - then a PM cost will be incurred at time L

  - We determine L so that the expected cost of each consequence is equal;

      - Breakdown cost * Prob (TBB < L) = Preventive cost * Prob (TBB > L)

      - Breakdown cost * Prob (TBB < L) = Preventive cost * (1 – Prob (TBB < L)

      - (Preventive cost + Breakdown) * Prob (TBB < L) = Preventive cost

      - Rob (TBB<L) = Preventive Cost / (Preventive Cost + Breakdown Cost)

    - Choose L so that the probability that time between breakdowns is smaller than L is equal to Preventive cost/(Breakdown cost + Preventive cost)

    - Marginal cost method is similar to that used in the single period model

  - Example 4

    - Suppose that the time before a breakdown is Normally distributed and has a mean of 3 weeks and a standard deviation of 0.60 week

    - If a breakdown costs an average of $1000

    - If preventive maintenance costs $250

    - What is the optimal preventive maintenance interval?

      - Preventive Cost / (Preventive Cost + Breakdown Cost)

      - 250 / 250 + 1000 = 0.20

    - Find the number of standard deviations from the mean, represented by the area under the normal curve of 0.2 (from left tail)

      - 0.20 = z of -0.84

      - Mean + z(standard deviation) = 3 – 0.84(0.60) = 2.496 (round to 2.5 weeks)

- Discuss breakdown (emergency) maintenance

  - Occasional breakdown is likely to occur even if there is an effective preventive maintenance program established

  - Organizations that rely less on preventive need to effectively deal with breakdowns

  - Unlike preventive maintenance, breakdown maintenance cannot be scheduled

  - Provisions to deal with breakdowns

    - Standby or backup equipment

      - Can be quickly brought into service

    - Inventories of spare parts

      - That can be installed as needed, thereby avoiding lead times involved in ordering parts

      - Buffer inventories of W.I.P. so that other equipment will be less likely to be affected by short-term downtime of a particular piece of equipment

    - Operations

      - Able to deal with emergencies and perform at least some minor repairs on the equipment

    - Maintenance staff

      - Well trained and readily available to diagnose and correct problems with equipment

  - The degree depends on importance of the equipment

    - At one extreme, the equipment is the focal point of the system

    - The other, the equipment is seldom used

- Decide when to replace a machine or a group of components

  - When breakdowns become frequent and costly, managers must trade of costs

    - What is the cost of replacement compared with the cost of repair?

  - The question is sometimes difficult to resolve

    - Especially if future breakdowns cannot be predicted

    - Historical records may help project

  - Another situation

    - Replacement before failure may be economical is in group replacement of components

      - Allowing to buy in bulk

  - Example 6

    - Facts

      - The maintenance manager wants to know whether he should have all the 3,000 fluorescent light bulbs to illuminate the plant, replaced together in order to save labour costs

      - There are three light bulbs in a fixture (1,000 fixtures)

      - They are lit about 4,000 hours a years

      - The light bulbs have a rated life of 25,000 hour swath a standard deviation of 5,000 hours

      - If all the light bulbs are replaced together, it will take 12 minutes per fixture

      - Individually, it will take 20 minutes (because the latter has to be brought from storage and returned there after the work)

      - Labour costs are $15 an hour

      - Each bulb costs $1.35 (group replacements with result in a break in price..  buying in bulk [ignore this fact])

    - Solution

      - The average life of a bulb in years of usage is 25,000/4,000 = 6.25 years with a standard deviation of 4,000/5,000 = 0.8 years

      - It will likely last 3 years

Outline

- Introduction

- Preventive maintenance

- Breakdown (emergency) maintenance

- Replacement

Introduction

- Vocab

  - Maintenance

    - All activates that keep facilities and equipment in good working order so that a system can perform as intended

  - Breakdown (emergency) maintenance

    - Dealing with breakdowns (repair/replacement) only after they occur

  - Preventive maintenance

    - Reducing or avoiding breakdowns through a program of lubrication, bolt tightening, cleaning, inspection, and replacement of worn parts

  - Reliability-centered maintenance (RCM)

    - An approach that reduces the effect of each major cause of failure for equipment functions

- Maintenance activities

  - Building and grounds

    - Appearance and functioning of buildings, roads, parking lots, lawns, fences, and the like

  - Equipment maintenance

    - Responsible for maintaining machinery and equipment in good working condition and making all the necessary repairs

    - **focus of the chapter

- Goal of maintenance management

  - Keep the production system in good working order at minimum cost

- Reasons for wanting to keep equipment and facilities in good operating condition

  - Avoid production disruptions (down time), including slowdowns and reduced yield

  - Maintain high quality and safety

  - Avoid missed delivery dates

- Basic options with respect to maintenance

  - Breakdown (emergency) maintenance

  - Preventive maintenance

- Example

  - Historical frequency of breakdowns of a machine per month are displayed on the table

| Number of Breakdowns | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Frequency of Occurrence | 0.2 | 0.3 | 0.4 | 0.1 |

  - Cost of breakdown repair: $1000

  - Cost of preventive maintenance : $1250

  - The expected number of breakdowns without preventive maintenance is 1.40 per month

| Number of Breakdowns | Frequency of Occurrence | Expected Number of Breakdowns |  |
| --- | --- | --- | --- |
| 0 | 0.2 | 0 | = 0*0.2 |
| 1 | 0.3 | 0.3 | = 1 * 0.3 |
| 2 | 0.4 | 0.8 | = 2 * 0.4 |
| 3 | 0.1 | 0.3 | = 3 * 0.1 |
|  | 1 | 1.4 |  |

    - Expected cost of breakdown repair

      - 1.40 breakdowns/month * $1,000 breakdown = $1,400 per month

    - Preventive Maintenance

      - $1250/month

  - Preventive Maintenance would yield savings of $150/month

    - $1,400 – 1,250

- Total maintenance cost as a function of preventive maintenance efforts

- With no preventive maintenance

  - Breakdown and repair/replacement costs would be tremendous

    - Cost of lost production and wages while equipment is not in service must be factored in

    - Cost of injuries or damages to other equipment

  - However, beyond the optimum preventive maintenance exceeds its benefit

- Example

  - A person has never changed the oil in their car, and never had the brakes or tires inspected, but had done repairs when it was absolutely necessary

    - Preventive costs would be negligible but the repair costs would be high (so many parts could fail; on the highway, during rush hour)

  - Having it changed every morning

    - Obvious excess; automobiles are designed to perform for longer periods

- Concept of maintaining production systems

  - Strike balance between prevention costs and breakdown costs

- Factors affect the amount of preventive maintenance is required

  - In a new car, there is little preventive maintenance

  - Gets worn thru use, desirability for preventive maintenance increases because risk of breakdown increases

    - When tires and breaks show signs of wear, they should be replaced

    - Dents should be checked before they rust

    - Car oil should be changed after high exposure to dirt

    - Inspection of critical parts should be done

- Reliability centered maintenance

  - An approach that reduces the effect of each major cause of failure for equipment functions

  - Different approach to redesigning the item to prevent failures

  - Steps

    - Major causes of failure identified

    - Major possible failure problems are determined

    - Major cause of each failure (AKA: failure mode) are identified

    - Various ways (i.e.: preventive maintenance) used to reduce the effect of each major cause

- Example 2

  - Study performed by a maintenance engineer

  - Each electrical distribution substation, there is a battery that will operate a relay switch when electricity is interrupted or there is a surge

  - The system includes a battery charge and cables

  - Dominant failure problems for the battery system

    - The cable does not conduct electricity

      - Major cause are loose connections or corrosion

        - Loose connection – annual electrical resistance test is prescribed

        - Corrosion – the preventive maintenance of cleaning and grease-coating every quarter is prescribed

    - The battery loses its capacity

    - The battery charger does not charge the battery

  - For each critical problem, the dominant failure cause (modes) are identified, and a maintenance program suggests how to reduce the effect of the failure mode

  - RCM should be the ultimate goal of every maintenance systems

    - Yet orgs. Lack the basic maintenance management

      - Identify importance maintenance jobs

      - Schedule preventive maintenance jobs effectively

      - Perform necessary maintenance effectively and efficiently

- Example 3

  - STK is a paper mill that needs to cut costs

  - Maintenance had become reactionary

  - Relationship with operations department was no good

    - 20% of planned jobs, 30% of emergency jobs in a week could be performed

    - Results in excessive use of contractors

  - The plant manager calls for a re-assessment of the maintenance function and a consulting firm was brought in

    - Tradesmen (pipefitter, electrician, machinist, instrument technician) were employed as maintenance work planners

    - Weekly metrics (efficiency, equipment availability) were developed

    - Operations, maintenance meet every day

    - Guidelines for most repair jobs were developed

    - Standardizing maintenance procedures and established quality norms

    - Store staff prepares kits of necessary parts for maintenance so technicians don’t have to waste time looking for parts

  - The number of maintenance jobs performed has now doubled

  - Next step is equipment failure analysis

Preventive maintenance

- Goal

  - Reduce breakdowns

  - Avoid associated costs

    - Loss of output

    - Idle workers

    - Unexpected repairs

- Most preventive maintenance is periodic

  - Can be scheduled according to

    - The result of planned inspections that reveal a need for maintenance

    - According to the calendar (passage of time)

    - After a predetermined number of operating hours (or miles for trucks)

- Important issue

  - Frequency

    - As the time between two periodic maintenance increase, the cost of preventive maintenance per unit time decrease while the risk (and cost) of breakdowns increase

  - Goal

    - Balance two cost

- Given preventive maintenance (PM) in interval L,

  - If a breakdown occurs before L,

    - then a breakdown maintenance cost will be incurred, and machine will be repaired and the next time between breakdowns (TBB) will start

  - If there is no breakdown before L,

    - then a PM cost will be incurred at time L

- We determine L so that the expected cost of each consequence is equal;

    - Breakdown cost * Prob (TBB < L) = Preventive cost * Prob (TBB > L)

    - Breakdown cost * Prob (TBB < L) = Preventive cost * (1 – Prob (TBB < L)

    - (Preventive cost + Breakdown) * Prob (TBB < L) = Preventive cost

    - Rob (TBB<L) = Preventive Cost / (Preventive Cost + Breakdown Cost)

  - Choose L so that the probability that time between breakdowns is smaller than L is equal to Preventive cost/(Breakdown cost + Preventive cost)

  - Marginal cost method is similar to that used in the single period model

- Example 4

  - Suppose that the time before a breakdown is Normally distributed and has a mean of 3 weeks and a standard deviation of 0.60 week

  - If a breakdown costs an average of $1000

  - If preventive maintenance costs $250

  - What is the optimal preventive maintenance interval?

    - Preventive Cost / (Preventive Cost + Breakdown Cost)

    - 250 / 250 + 1000 = 0.20

  - Find the number of standard deviations from the mean, represented by the area under the normal curve of 0.2 (from left tail)

    - 0.20 = z of -0.84

    - Mean + z(standard deviation) = 3 – 0.84(0.60) = 2.496 (round to 2.5 weeks)

- Preventive maintenance should be performed prior to breakdown

- Predictive or condition-based maintenance

  - Maintenance based on historical data and ongoing monitoring of equipment condition

    - Pressure

    - Rates of flow

    - Vibration

    - Noise

    - Humidity

    - Changes in electricity

- Good predictive maintenance

  - Relies on complete records of each piece of equipment … to monitor the health of the equipment

    - Date of installation

    - Operating hours

    - Dates

    - Types of repairs

- Example 5

  - Management wants to cut unscheduled downtime to less than 1%

  - Standard maintenance check was a walk0through by operators, then by maintenance staff

  - Problems were noted in a notepad and not recorded on a computer so it was difficult to identify trends

  - They purchase Ivara

    - Operators walk through

    - Measure equipment

    - Enter into handheld device

    - Data is checked by software and an alarm will trigger for measurements out of range

  - Typical problems (around 6 per day) include

    - Too much vibration

    - Low oil

    - Leaking seal

- Total productive maintenance

  - Approach to maintenance where operators perform basic preventative maintenance on their machines

  - Consistent with Jit/lean

    - Employees are given greater responsibility for quality, productivity and general functioning of the system

- Training employees in the proper procedures are a major part of TPM

  - How to keep equipment in good operating order

  - Providing the incentive to do so

Breakdown (Emergency) Maintenance

- Occasional breakdown is likely to occur even if there is an effective preventive maintenance program established

- Organizations that rely less on preventive need to effectively deal with breakdowns

- Unlike preventive maintenance, breakdown maintenance cannot be scheduled

- Provisions to deal with breakdowns

  - Standby or backup equipment

    - Can be quickly brought into service

  - Inventories of spare parts

    - That can be installed as needed, thereby avoiding lead times involved in ordering parts

    - Buffer inventories of W.I.P. so that other equipment will be less likely to be affected by short-term downtime of a particular piece of equipment

  - Operations

    - Able to deal with emergencies and perform at least some minor repairs on the equipment

  - Maintenance staff

    - Well trained and readily available to diagnose and correct problems with equipment

- The degree depends on importance of the equipment

  - At one extreme, the equipment is the focal point of the system

  - The other, the equipment is seldom used

Replacement

- When breakdowns become frequent and costly, managers must trade of costs

  - What is the cost of replacement compared with the cost of repair?

- The question is sometimes difficult to resolve

  - Especially if future breakdowns cannot be predicted

  - Historical records may help project

- Another situation

  - Replacement before failure may be economical is in group replacement of components

    - Allowing to buy in bulk

- Example 6

  - Facts

    - The maintenance manager wants to know whether he should have all the 3,000 fluorescent light bulbs to illuminate the plant, replaced together in order to save labour costs

    - There are three light bulbs in a fixture (1,000 fixtures)

    - They are lit about 4,000 hours a years

    - The light bulbs have a rated life of 25,000 hour swath a standard deviation of 5,000 hours

    - If all the light bulbs are replaced together, it will take 12 minutes per fixture

    - Individually, it will take 20 minutes (because the latter has to be brought from storage and returned there after the work)

    - Labour costs are $15 an hour

    - Each bulb costs $1.35 (group replacements with result in a break in price..  buying in bulk [ignore this fact])

  - Solution

    - The average life of a bulb in years of usage is 25,000/4,000 = 6.25 years with a standard deviation of 4,000/5,000 = 0.8 years

    - It will likely last 3 years