# ECOR 1010 – Introduction to Engineering

Name

Assignment #6

Assignment Title:  Predicting the strength of two new steels

TO Marking TA:

Name: [Email]

Email: [Email]

Lab Section: B1

Room: ME2556 – Carleton University

FROM:

Email: [Email]

Student Number: [Email]

Number of Figures and Tables (Including handwritten ones): 4

Last Date and Time of Revision: 16 November 2013 7:29pm

# Introduction

The purpose of this lab is to use graphing and statistics to develop a regression equation which will predict the strengths of two new types of steel based on carbon content.

# Materials and Methods

The lab provides sample tests done on steel based on their carbon content.  Using the measurements provided, a line of best fit is calculated. A table of values (Table 2) is then completed using MS Excel for the arithmetic. The correlation coefficient and Coefficient of determination for the data is then calculated using the equations provided in class. All of the calculated values are then checked against the values produced with Excel’s “regression” function. Plots are then constructed from the linear regression equation as well as third and six order polynomial lines of best fit. Using these three equations, the strength of the two new types of steel can be calculated.

# Results

The slope for the linear regression was found to be 326.981 with a y-intercept at 279.98. The correlation coefficient was found to be 0.9248. The coefficient of determination was found to be 0.8553. According to the linear model, steel with a carbon content of .33% would have a strength value of 387.89 MPa and a content of 1% would have a strength value of 606.96 MPa. According to the third order polynomial model of the data, the .33% carbon steel would have a strength value of 381.04 MPa and the 1% carbon would have a strength value of 499.79 MPa.  According to the sixth order polynomial model of the data, the .33% carbon steel would have a strength value of 358.01 MPa and the 1% carbon would have a strength value of -158.74 MPa.

# Discussion

The data shows us that a higher order regression equation does not always mean a more accurate prediction. While it looks like the sixth order regression is holding truer to the data points, we can see that it is predicting a negative strength value if the steel has a 1% carbon content.  This result seems unreasonable so this equation cannot be trusted.  The third order regression equation seems to be better except that the only value that predicts the decrease in strength due to carbon content is the last value. It cannot be proven that this data point isn’t an obvious outlier. Without further data past the last point, the safest estimate for the steel strength is the linear regression.

# Conclusions

After investigating the data provided regarding the strength of steel verses the carbon content, it appears that the steel with a carbon content of .33% will have a strength value of 387.89 MPa and a content of 1% would have strength of 606.96 MPa.

1)

m=i=1n(xi-x)(yi-y)i=1n(xi-x)2

m=205.13330.623756

m= 326.981

b=y-mx

b=429.66-326.981×0.4577

b=279.98

2)

rxy=i=1n(xi-x)(yi-y)n-1sxsy

=0.9248

3)

SSE=1n(y-y)2

=113447.3

SSR=1n(y-y)2

67074.7

TSS=SSE+SSR

=78422

r2=SSRTSS

=0.8553

4)

Using the CORREL and RSQ function in excel, the following results were obtained:

Excel CORREL: 0.924827

Excel RSQ: 0.8553046

This Is very close to my calculated values.

5)

| SUMMARY OUTPUT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Regression Statistics |  |  |  |  |  |
| Multiple R | 0.9248268 |  |  |  |  |
| R Square | 0.85530461 |  |  |  |  |
| Adjusted R Square | 0.83463384 |  |  |  |  |
| Standard Error | 40.26218 |  |  |  |  |
| Observations | 9 |  |  |  |  |
| ANOVA |  |  |  |  |  |
|  | df | SS | MS | F | Significance F |
| Regression | 1 | 67074.69803 | 67074.7 | 41.37749 | 0.000356109 |
| Residual | 7 | 11347.30197 | 1621.043 |  |  |
| Total | 8 | 78422 |  |  |  |
|  | Coefficients | Standard Error | t Stat | P-value | Lower 95% |
| Intercept | 279.982041 | 26.8627186 | 10.4227 | 1.63E-05 | 216.4618051 |
| X Variable 1 | 326.980978 | 50.83237625 | 6.432534 | 0.000356 | 206.7815087 |

| Upper 95% | Lower 95.0% | Upper 95.0% |
| --- | --- | --- |
| 343.5022769 | 216.4618051 | 343.5022769 |
| 447.180448 | 206.7815087 | 447.180448 |

My m value is the same as well as the y intercept. The correlation coefficient is the same as well as the coefficient of determination.

6)

The lower 95% confidence level for the slope is 206.78

The upper 95% confidence level for the slope is 447.18

The lower 95% confidence level for the y-intercept  is 216.46

The upper 95% confidence level for the y-intercept is 343.50

7-11)

| Carbon content (%) | Strength (Mpa) | Predicted Strength (linear) | Predicted Strength (Polynomial Degree = 3) | Predicted Strength (Polynomial Degree=6) |
| --- | --- | --- | --- | --- |
| 0.15 | 315 | 329.02919 | 321.0156625 | 312.1094401 |
| 0.2 | 330 | 345.37824 | 331.2812 | 346.5560512 |
| 0.22 | 358 | 351.91786 | 336.9927992 | 343.6984069 |
| 0.3 | 345 | 378.07633 | 367.2583 | 341.8428758 |
| 0.4 | 415 | 410.77443 | 416.5196 | 423.0168704 |
| 0.5 | 507 | 443.47253 | 470.2925 | 499.583125 |
| 0.6 | 483 | 476.17063 | 519.8044 | 485.6307776 |
| 0.8 | 585 | 541.56682 | 570.9548 | 584.3387008 |
| 0.95 | 529 | 590.61397 | 532.8862625 | 528.7186134 |
| R^2 value for the regression |  | 0.8553 | 0.9504 | 0.99 |
| Interpolation:0.33 |  | 387.88576 | 381.0410023 | 358.0131264 |
| Extrapolation: 1.00 |  | 606.96302 | 499.79 | -158.74 |