Intro to Silicon wafers and transistors

Moore’s Law – Silicon density doubles every 18 months

1971 – 2k transistors – 0.1 MHz

2014 – 4.3B transistors – 3600MHz

Size of transistor: 25 nm

Op-Amp

Uses: +,-, integration, differentiation

Applications: amplifiers, buffers, oscillators, filters

A: Open-loop Gain, ideal A=∞, real = 104, 106

V0 = A(V2 – V1)       Difference Amplifier

If V2 = V1, then V0 = 0

V2 and V1 are said to be virtually shorted

Ri is HIGH (∞), R0 is LOW (0)

Inverting: V1- (amplifies and inverts signal)

Inv: A = -R2/R1  Non: A = 1+R2/R1

Summing: amplifies all inputs then adds them

Differentiator Circuit: Gain = RCω, 270° PS between Vin and Vout

Integrator: Swap C and R (above)

Gain = 1/RCω, 90° PS between Vin and Vout

Low Pass

High pass: Cap goes in series with R1

Semiconductor Physics & Diodes

R = pl/A  (resis, physical parms, length, area)

Diode: Most fundamental analog device

Materials: Si, Ge, gallium arsenide, silicon carbide

Conductors: LOW resistance for current flow

Insulators: HIGH resistance for current flow

Semiconductors: Medium resistance - advants!

Free Electron: not confined to atom, -ve carier

Free Hole: acts as a positive charge carrier

Diffusion current: flow of carriers due to concentration gradient. HIGH  LOW

Drift current: movement of carriers due to and electric field

Silicon: abundant in nature, 4 valence e-, forms crystalline lattice structure

Doping: makes intrinsic silicon impure, creating extrinsic silicon

N-type: introduce e- donor (ex: P)

P-type: introduce acceptor (ex: B)

Depletion region (DR): at the boundry p-n, e-s and holes combine and make it void of charge carriers

Depletion region creates a barrier separating p-n and obstruct movement of charge across it

Forward Bias: diode ON, reduces DR, only when voltage applied is higher than barrier potential (BP)

ID=IS(eVD/nVT-1)

ID Current thru diode

VD  voltage across diode

n  ideality factor

Is  reverse saturation current

VT  thermal voltage (@ room temp 25mV)

Vt = kT/q

k Boltzman’s constant 1.38e-23

q  1.6e-19

Reverse Biasing: DR grows, applied voltage further reinforces the barrier potential, OFF,

Electric field across junction increases

Rectifier: Converts AC to DC

PIV: Peak Inverse Voltage

Two types: half (HWR) and full wave (FWR) bridge and center-tapped(below)

Zener Diodes: Breakdown Diodes, operate under RB, if FB, acts like a normal diode

Breakdown due to:

Zener Effect: increased Efield breaks the covalent bonds

Avalanche Effect: Kinetic energy of the minority carriers

Voltage Regulator Circuit:

Peak Rectifers: +ve ½ cycle  Cap charges fast,

-ve ½ cycle  cap dischanges slowly, to be good, RC >>T

Clamper: Shifts DC level of given signal

Clipper: limits signal level to a certain magnitude, changes the wave shape