EXPERIMENT 2 – Jet Pump

Shaun Lewinson

100941697

Shaun.lewinson@carleton.ca

Monday, March 15th, 2015

G8: -Daniel Mason

-Adam Beaudoin

Summary

The purpose of this laboratory is to compare the performance of the jet pump with a theoretical analysis of the flow within the pump. There were a number of assumptions made in this lab, such as: no friction within the walls of the tube, no energy losses, steady flow, and constant air density. The experimental results obtained in the lab will be compared to the theoretical results calculated to get an understanding of how jet pumps function.

Nomenclature

- g – Gravitational constant (9.81 m/s2)

- Patm – Atmospheric pressure (101, 325 Pa)

- PC1 – Fluid supply pressure (Pa)

- PC2 – Nozzle pressure (Pa)

- Pp – Primary steam pressure (Pa)

- Ps – Secondary steam pressure (Pa)

- ρair – Density of air (1.2 kg/m3)

- ρwater – Density of water (1000 kg/m3)

- Cp – Static pressure coefficient (-0.045)

- Cq – Dynamic pressure coefficient (0.93)

- h – Height at pressure tap (m)

- Vp – Primary steam velocity (m/s)

- Vs – Secondary steam velocity (m/s)

- Vout – Velocity at outlet (m/s)

- Ap – Primary steam area (m2)

- As – Secondary steam area (m2)

- Aout – Area at steam outlet (m2)

Flow Analysis

Pressure at pitot tubes

Pstatictap=Patm+ρfluidg(hatm-hstatictap)                      [1]

CP=PP-PC2PC1-PC2                                                                 [1.2]

Rearrange for Pp

PP=PC2+CP(PC1-PC2)                                        [1.3]

Stream velocities

Primary velocity

Cq=qPC1-PC2                               [2]

q=0.5ρVP2                                 [3]

Sub in [2] and [3] and isolate for velocity

VP=2Cq(PC1-PC2)ρair                      [4]

Secondary velocity

Patm+ρgz1+12ρv12=P2+ρgz2+12ρv22    [5]

Since h1=h2 and v1=0, isolate for Vs

Vs=2(Patm-Ps)ρair                   [6]

Pressure and velocity at pitot tube rake

Velocity

APVP+AsVs=AoutVout           [7]

Where A is A=πd2/4

Vout= APVP+AsVsAout                            [8]

Pressure

From Linear momentum

PPAP+PsAs-PoutAout=ρairAoutVout2-ρairAPVP2-ρairAsVs2  [9]

Isolate Pout

Pout= PPAP+PsAs-ρair(AoutVout2-APVP2-AsVs2)Aout              [10]

Mass flow rate

dmdt= 0rρVr2πrdr                 [11]

Procedure

Refer to Experiment 2: Jet Pump in the lab manual for procedure

Experimental Setup

Figure 1: Experiment Apparatus

Results & Discussion

Part 1

Under the assumptions of no friction and steady flow, the outlet pressure and velocity were calculated using the data obtained from the inlet. Using continuity, the velocity at the outlet was found to be 19.12 m/s for flow 1 and 20.54 m/s for flow 2. The pressure at the outlet was calculated by applying linear momentum, where Pout for flow 1 and flow 2 was calculated to be 60205.48 Pa and 60233.14 Pa respectively.

Part 2

Figure 2: Static pressure along mixing tube

From figure 2, it can be seen that both flows have a rapid rise in pressure before leveling off. This shows that the mixing process was complete and that the mixing tube was long enough, because the pressure leveling off indicates uniform velocity. The predicted values for the pressure at the outlet show quite a large error because the predicted Pout for both flows were around 60000 Pa. This error could have been caused by a number of reasons, but most likely occurred from either an error in the data, or an error in the calculations. When determining the validity of the assumptions made in the lab, flow rate can affect the results. For example, an assumption of no friction was assumed within the tube; however, with increased flow rate, the friction around the walls will also increase due to the higher velocity. If the flow rate becomes so large that friction must be taken into account, then the tube can no longer be assumed to be frictionless.

Part 3

Figure 3: Velocity through the Mixing Tube

As seen in figure 3, the assumption of uniform flow was not achieved completely due to the fact that friction acts strongly on the surface of the walls. As the radius increases to get closer to the walls, there is more fluctuation within the data. This is why the assumption of no friction was never going to be completely satisfied because there will always be some amount of friction acting on the walls that can skew the data. This non-uniform velocity is caused by a higher flow rate than expected. The higher the flow rate is inside the tube, the more non-uniform the velocity data will become due to the losses of kinetic energy within the tube.

Part 4

From integrating the curves of best fit seen in figure 3, a mass flow rate was calculated to be 0.1378 kg/s for flow 1 and 0.1529 kg/s for flow 2. Using continuity, a mass flow rate of 0.0872 kg/s for flow 1 was calculated, while flow 2 was calculated to be 0.0936 kg/s. When comparing these two flow rates, a large discrepancy can be seen. This can be caused from a number of reasons such as: human reading error on the data, inaccurate data being produced from the jet pump, or simply from miscalculation. Also, using improper assumptions such as steady air density or no friction within the walls of the tubes could have also skewed the results.

Conclusion

The theoretical and experimental results obtained in the lab were compared and it was concluded that the assumptions made in the lab were slightly off. Many of the values compared had large discrepancies between them, which were likely caused by false assumptions being made. Flow 1 and flow 2 of the data showed to be fairly uniform with each other, especially regarding the velocity and pressure within the tube.

References

“MAAE 2300 Fluid Mechanics I LABORATORY EXERCISES (Revised January 2010)”, internet: https://culearn.carleton.ca/moodle/pluginfile.php/1126498/mod_resource/content/1/MAAE2300_LabManual.pdf, [March 15, 2015].

Appendices

Table 1: Jet Pump Dimensions

| Distance between pitot tubes | 2 in |
| --- | --- |
| Distance to first pitot tube | 9 in |
| Diameter of nozzle | 0.875 in |
| Diameter of pump | 3 in |
| Distance between rake port centers | 3.25 mm |

Table 2: Flow 1 data

| Utube | Height (m) | Tube | Static (m) | Pitot (m) | Static pressure (Pa) | Stagnation pressure (Pa) | Pitot velocity (m/s) | Radius from center (m) | Sum of flow rates (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.32 | 1 | 0.124 | 0.082 | 101266.14 | 101776.26 | 21.77498565 | 0.0381 | 0.1378 |
| 2 | 0.374 | 2 | 0.144 | 0.076 | 101069.94 | 101835.12 | 23.92174743 | 0.0316 |  |
| Diff | 0.694 | 3 | 0.145 | 0.074 | 101060.13 | 101854.74 | 24.59573134 | 0.02835 |  |
|  |  | 4 | 0.142 | 0.07 | 101089.56 | 101893.98 | 25.89111817 | 0.0251 |  |
|  |  | 5 | 0.139 | 0.068 | 101118.99 | 101913.6 | 26.51509004 | 0.02185 |  |
|  |  | 6 | 0.134 | 0.068 | 101168.04 | 101913.6 | 26.51509004 | 0.0186 |  |
|  |  | 7 | 0.128 | 0.064 | 101226.9 | 101952.84 | 27.72093072 | 0.01535 |  |
|  |  | 8 | 0.12 | 0.062 | 101305.38 | 101972.46 | 28.30459327 | 0.0121 |  |
|  |  | 9 | 0.113 | 0.062 | 101374.05 | 101972.46 | 28.30459327 | 0.00885 |  |
|  |  | 10 | 0.108 | 0.06 | 101423.1 | 101992.08 | 28.876461 | 0.0056 |  |
|  |  | 11 | 0.106 | 0.064 | 101442.72 | 101952.84 | 27.72093072 | 0.00235 |  |
|  |  | 12 | 0.104 | 0.064 | 101462.34 | 101952.84 | 27.72093072 | -0.0009 |  |
|  |  | 13 | 0.103 | 0.064 | 101472.15 | 101952.84 | 27.72093072 | -0.00415 |  |
|  |  | 14 | 0.104 | 0.064 | 101462.34 | 101952.84 | 27.72093072 | -0.0074 |  |
|  |  | 15 | 0.102 | 0.064 | 101481.96 | 101952.84 | 27.72093072 | -0.01065 |  |
|  |  | 16 | 0.101 | 0.066 | 101491.77 | 101933.22 | 27.12471198 | -0.0139 |  |
|  |  | 17 | 0.101 | 0.066 | 101491.77 | 101933.22 | 27.12471198 | -0.01715 |  |
|  |  | 18 | N/A | 0.068 | 101325 | 101913.6 | 26.51509004 | -0.0204 |  |
|  |  | 19 | N/A | 0.068 | 101325 | 101913.6 | 26.51509004 | -0.02365 |  |
|  |  | 20 | 0.118 | 0.128 | 101325 | 101325 |  | -0.0269 |  |

Table 3: Flow 2 data

| Utube | Height (m) | Tube | Static (m) | Pitot (m) | Static pressure (Pa) | Stagnation pressure (Pa) | Pitot velocity (m/s) | Radius from center (m) | Sum of flow rates (m3/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.34 | 1 | 0.128 | 0.084 | 101207.28 | 101756.64 | 23.92174743 | 0.0381 | 0.1529 |
| 2 | 0.394 | 2 | 0.148 | 0.074 | 101011.08 | 101854.74 | 27.12471198 | 0.0316 |  |
| Diff | 0.734 | 3 | 0.15 | 0.07 | 100991.46 | 101893.98 | 28.30459327 | 0.02835 |  |
|  |  | 4 | 0.146 | 0.068 | 101030.7 | 101913.6 | 28.876461 | 0.0251 |  |
|  |  | 5 | 0.144 | 0.068 | 101050.32 | 101913.6 | 28.876461 | 0.02185 |  |
|  |  | 6 | 0.138 | 0.066 | 101109.18 | 101933.22 | 29.43722134 | 0.0186 |  |
|  |  | 7 | 0.134 | 0.064 | 101148.42 | 101952.84 | 29.98749739 | 0.01535 |  |
|  |  | 8 | 0.126 | 0.062 | 101226.9 | 101972.46 | 30.52785613 | 0.0121 |  |
|  |  | 9 | 0.12 | 0.062 | 101285.76 | 101972.46 | 30.52785613 | 0.00885 |  |
|  |  | 10 | 0.116 | 0.06 | 101325 | 101992.08 | 31.05881517 | 0.0056 |  |
|  |  | 11 | 0.112 | 0.06 | 101364.24 | 101992.08 | 31.05881517 | 0.00235 |  |
|  |  | 12 | 0.11 | 0.06 | 101383.86 | 101992.08 | 31.05881517 | -0.0009 |  |
|  |  | 13 | 0.109 | 0.06 | 101393.67 | 101992.08 | 31.05881517 | -0.00415 |  |
|  |  | 14 | 0.109 | 0.062 | 101393.67 | 101972.46 | 30.52785613 | -0.0074 |  |
|  |  | 15 | 0.109 | 0.062 | 101393.67 | 101972.46 | 30.52785613 | -0.01065 |  |
|  |  | 16 | 0.107 | 0.064 | 101413.29 | 101952.84 | 29.98749739 | -0.0139 |  |
|  |  | 17 | 0.107 | 0.066 | 101413.29 | 101933.22 | 29.43722134 | -0.01715 |  |
|  |  | 18 | N/A | 0.066 | 101325 | 101933.22 | 29.43722134 | -0.0204 |  |
|  |  | 19 | N/A | 0.068 | 101325 | 101913.6 | 28.876461 | -0.02365 |  |
|  |  | 20 | 0.116 | 0.128 | 101325 | 101325 |  | -0.0269 |  |