Corrosion and Degradation of Materials

Metals:

- Loss by dissolution

- Film formation (Oxide)

Polymers:

- Loss by Dissolution (In solvent)

- Absorption and Swelling (In solvent)

- Structural Change Due to Electromagnetic Radiation (Example: Ultraviolet)

In Metals:

Corrosion is destructive and an unintentional attack of a metal.

(Example: Rusting, Autobody Panels, Radiator, Exhaust Components)

Useful Corrosion Processes:

- Metal Etching (Example: Microscopy)

- Dry Cell Batteries

Electrochemical Considerations

Half Reactions

Oxidation: Metal giving up electrons

Reduction: Metal gaining electrons

Oxidation:

M  Mn+ + ne-  M

Fe  Fe2+ + 2e-

Al  Al3+ + 3e-

Happens at anode (Anodic Reaction).

Reduction:

Mn+ + ne-  M

2H+ + 2e-  H2(gas)

Happens at cathode (Cathodic Reaction).

O2 + 4H+ + 4e-  2H2O

Dissolved oxygen in acid solutions.

O2 + 2H2O + 4e-  4(OH-)

Dissolved oxygen in neutral or basic solutions.

Rusting

Oxidation Stage 1

Fe oxidized to Fe2+ (and then to Fe(OH)2)

Fe + ½O2 + H2O  Fe2+ + 2OH-  Fe(OH)2

Oxidation Stage 2

2Fe(OH)2 + ½ O2 + H2O  2Fe(OH)3

Metal ions go into corroding solution as ions. *(Example: Zn  Zn2+ + 2e-).

They may form an insoluble compound with non-metallic elements (Fe(OH)3, rust).

Electrode Potentials

Half Cells

Fe  Fe2+ + 2e- 	Oxidation

Cu2+ + 2e-  Cu	Reduction

Galvanic Couple:

Two metals electically connected in a liquid electrolyte.

One metal anode corrodes (oxidizes), the other metal-cathode enables reduction.

Cu-Fe Galvanic Cell:

25°C (77°F): Fe + Cu2+  Fe2+ + Cu 		0.780 V

Fe2+ + Zn  Fe + Zn2+	IM solution

Fe2+ + 2e-  Fe

Zn  Zn2+ + 2e-

0.323 V

Different voltages for different pairs of electrodes.

Molarity (M): The number of moles of solute per million cubic mm (106mm3 or 1000cm3) of solution.

Magnitude of Voltage: Driving force for electrochemical oxidation-reduction reaction.

emf Series: Rating of metallic materials as to their tendency to experience oxidation when coupled to other metals in solution of respective ions.

Mole: Quantity corresponding to 6.023 x 1023 atoms or molecules.

Standard Half-cell: Pure metal immersed in IM solution of its ions and at 25°C (77°F).

The Standard emf Series

Reference cell to which other half-cells are compared