Comp 445

Instructor : Dr. Aiman Hanna

Theory Assignment #3

Chao Tang

1)

Both pure Aloha and slotted Aloha are random access protocols. In pure Aloha, the idea is whenever the sender has data, it will send it to the receiver regardless any other condition. This works only well for one single node; however, when it comes to multiple nodes, this protocol has a tremendous problem with the collision since there is only one channel to share. Slotted Aloha works a little bit better than Pure aloha because the former divides the channel at a certain amount of time, and the sender can only send the frame at the beginning of one particular time slot. Nonetheless, the collision can also occur but at a smaller rate than pure aloha. When there is only one single node, pure aloha is better than slotted aloha since there is unnecessary waiting time for slotted aloha protocol.

2)

3)

The security level will not be increased with the double encryption since the addition of j and i can be done in one step,say x=i+j, so it is the same as adding I to the ASCII code of addition J to the plaintext character as providing in the first scenario.

4)

001011010101000011111101001101 (plain text)

101101011010110101101011010110 (encryption key)

XOR ------------------------------------------------------

1001100011111101100101100110111 (cipher text)

101101011010110101101011010110 (decryption key)

XOR ----------------------------------------------------

001011010101000011111101001101 (original plaintext)

5)

So there are 264 different keys, so we need (264/30days/24hours/60mins) keys/s to crack it.

6)

So the Cable Internet Access connects huge amount of cable modems to a cable modem termination system (CMTS) at the cable network headend. The DOCSIS will then divide the downstream using FDM to multiple frequency channels. The collision will not occur since every channel is independent to each other. While for the upstream, each channel is divided into intervals of time, each has a sequence of mini-slots. And the CMTS will send a control message(MAP) on a downstream channel to specify which cable modem can transmit during which mini-slot of the interval of time. So no collision cannot occur.

7)

a) Advantages: when one node is down, it will not affect the other nodes, meaning that the communication between host – host will not be affected. Data now has multiple paths to follow so the collision probability becomes significantly small.

b) Disadvantages: Since every node is connected to all the other nodes, this requires a huge amount of cables and produces a lot of redundant connections. Thus the cost of such network is significantly costly. The maintenance of such network is very difficult, so whenever there is one part of this network damaged or corrupted, the whole network must be replaced by a new one.

c)	Suppose there are 4 server racks belong to each Tier-2 switch, each rack has 5 hosts, and links between hosts under the same Tier-2 switch is 1Gbps. Suppose links between switches are 5G. Assume now we want to send the whole 4 racks of hosts( 4*5= 25 hosts) from Tier-2 switch, called A, to other 4 racks of hosts(25hosts) under Tier-2 switch, called B in parallel. So in total of 25*1Gbps = 25Gbps will be sent through A, but since there is only one cable that connects A and tier-1 switch, so the speed is reduced by 1/4 which is 5G/25 =200Mbps which is much less than 1Gbps within the same rack. However, if we use fully connected topology network, the data from every server rack has one distinct path to travel through , and the capacity of Tier-1 switch becomes 25Gbps which is sufficient.

8)

a)

If the data sting has 11bits, then 4 parity bits will be enough since 24 = 16 > 11+4=15

b1 b2 b3 b4 b5 b6 b7 b8 b9 b10 b11

p1 checks position 1,3,5,7,9,11

p2 checks position 2,3,6,7,10,11

p3 checks position 4,5,6,7,12

p4 checks position 8,9,10,11,12

put parity bits into the data at appropriate positions:

p1 b1 p2 b2 b3 p3 b4 b5 b6 b7 p4 b8 b9 b10 b11

total bits = 15

b)

if the data string has 19 bits, then 5 parity bits will be enough since 25=32 > 19+5=24

p1 checks position 1,3,5,7,9,11

p2 checks position 2,3,6,7,10,11

p3 checks position 4,5,6,7,12

p4 checks position 8,9,10,11,12

p5 checks 10000(16), 10001(17), 10010(18), 10011(19)

9) The token ring algorithm provides a LAN with 0 collision. Each node is taking turn to perform its task by holding a token, and the other nodes must be idle and wait till their turns. This algorithm has a major drawback with the time complexity since the main algorithm becomes a matter of choosing Master that controls the token, how frames are transmitted, how tokens can be distinguished, device damaged and etc. So the main idea of this type of network deviates from high speed data transmission and low collision to the maintenance of the network, which makes the whole network super slow. However, Ethernet is implemented with switched which also reduces the collision to zero, but also provides a high speed transmission.