- No, circuit switching always keep a stable and constant transmission process where reliability and transmission rate are guaranteed. However, the only disadvantage is that it can reserves few transmission links than packet switching in situation like: 
 We know that not everyone will be using the network all the time. Thus packet switching will be more suitable, since it shares the network. At a certain amount of transmission users, packet switch can serve more users than circuit switching. Thus, circuit switching is suitable for constant transmission process.

- Because Cable network access are shared, when there are less users, the downstream speed is really high. In comparison, the DSL network is dedicated and the speed is more constant. To speed up DSL, the company could use twisted pair to reduce the electrical interference from other wires.

- The non-persistent HTTP connection has simple architecture, since each connection ends with one request. In this case, more clients could be served. Despite of these simple architecture, more CPU usage are involved for making new connection every time. It also takes more time to retrieve multiple objects (2 RTTs and file transmission time per object). The advantages of persistent HTTP connection are less CPU usage, reduced network congestion, and faster content delivery. The reasons are, there are less connections made and less RTTs per file transmission. Nevertheless, the connections are not ended for a period of time even after file transmission, this keep the network busy by few users. 

Thus, for a single file request like AJAX request, XML files, basic HTML files, it would be better to apply non-persistent HTTP connection. For multiple files request, persistent really improves the efficiency of file transmission.

- There are 3 types of attack to the DNS: bombard either the root server or the TLD servers, redirect attacks (Man-In-The-Middle or DNS poisoning), and exploit DNS DDos attack. To reduce the traffic produced by bombard, the server can have cache to the TLD servers or local DNS server. The server can also set up a blacklist of IPs and a list of undesirable protocols. To prevent the redirect attacks, the protocol can apply encryption over the request from client and response from the server. To prevent DNS name server to be exploited, the server can request more confirmation to the client to perform a source IP address validation.

- Trying to interact with BAIDU, the search engine in China, whose IP address is 115.239.211.110. The trace route is done through the site traceroute.online-domain-tools.com. The ISP of my home network is

There are 3 RTT which probably indicates RTT in every level of DNS. The request is first send to Europe server hosting90.net with a RTT time of less than 1ms in every DNS. At every hop, we are requesting to connect to a server. Then traveling around server and closer and closer to Asia with a RTT time of less than 1ms in average to 19ms. Basically the network that my request went over is the telia.net. At the 8th hop, a time-out happened. This means that the packet was lost or the delay was very bad. As picture shows, the RTT dramatically rises after 8th hop. Before the 16th hop, we are travelling around the DNS hierarchy of China. At 16th hop, we succeed to connect to the website www.baidu.com. From the picture, we see that a packet can easily be lost when interacting with China’s server and the RTT is quite high.

- A) propagation delay = (10000*1000)/(2.5 x 10^8) = 0.04 sec
B) transmission delay = (2*8)/5 = 3.2 sec

- A) propagation delay = (10000*1000)/(2.5 x 10^8) = 0.04 sec
B) transmission delay = (2*2)/4 = 1 sec
C) total delay = 0.04 + (20/2)/4 = 0.04 + 2.5 = 2.54 sec

- Traffic might happen in the tit-for-tat scheme. When distributing files, a peer A tend to find a neighboring peer with most effective transfer speed. This leaves to the other peers who wish to upload their files to peer A being blocked if their upload rate is not one of the top. However, to solve this, tit-for-tat scheme allows to choose a random peer over the network to distribute files with A. The time is somehow long for these peers.

- Client-Server

|  | 10 | 100 | 1000 |
| --- | --- | --- | --- |
| 300kbps | 7680 sec | 51200 | 512000 |
| 700kbps | 7680 sec | 51200 | 512000 |
| 2mbps | 7680 sec | 51200 | 512000 |

N = 10

NF/Us = (10*15(2^30))/30(2^20) = 5120, F/Dmin = 15(2^20)/2(2^10) = 7680

Dcs = max {NF/U, F/Dmin} = max {5120, 7680} = 7680 sec

N = 100

NF/Us = (100*15(2^30))/30(2^20) = 51200, F/Dmin = 15(2^20)/2(2^10) = 7680

Dcs = max {NF/U, F/Dmin} = max {51200, 7680} = 51200 sec

N = 1000

NF/Us = (1000*15(2^30))/30(2^20) = 512000, F/Dmin = 15(2^20)/2(2^10) = 7680

Dcs = max {NF/U, F/Dmin} = max {512000, 7680} = 512000 sec

Peer to Peer

|  | 10 | 100 | 1000 |
| --- | --- | --- | --- |
| 300kbps | 7680 | 46644.8 | 466448 |
| 700kbps | 7680 | 41698.4 | 416984 |
| 2mbps | 7680 | 30720 | 307200 |

N = 10

Utotal = 30(2^20) + 10*300(2^10) = 34529280 bps

NF/Utotal = (10*15(2^30))/34529280 = 4664.48 sec

Utotal = 30(2^20) + 10*700(2^10) = 38625280 bps

NF/Utotal = (10*15(2^30))/38625280 = 4169.84 sec

Utotal = 30(2^20) + 10*2(2^20) = 52428800 bps

NF/Utotal = (10*15(2^30))/52428800 = 3072 sec

F/Us = 15(2^30) / 30(2^20) = 512 sec

F/Dmin = 15(2^20)/2(2^10) = 7680 sec

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 4664.48, 512, 7680 } = 7680

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 4169.84, 512, 7680 } = 7680

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 3072, 512, 7680 } = 7680

N = 100

NF/Utotal = (100*15(2^30))/34529280 = 46644.8 sec

NF/Utotal = (100*15(2^30))/38625280 = 41698.4 sec

NF/Utotal = (100*15(2^30))/52428800 = 30720 sec

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 46644.8, 512, 7680 } = 46644.8

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 41698.4, 512, 7680 } = 41698.4

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 30720, 512, 7680 } = 30720

N = 1000

NF/Utotal = (1000*15(2^30))/34529280 = 466448 sec

NF/Utotal = (1000*15(2^30))/38625280 = 416984 sec

NF/Utotal = (1000*15(2^30))/52428800 = 307200 sec

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 466448, 512, 7680 } = 466448

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 416984, 512, 7680 } = 416984

Dp2p = max { NF/U, F/Dmin, NF/Utotal } = max { 307200, 512, 7680 } = 307200