Decision Variables:

P1 number of units produced during month 1

P2 number of units produced during month 2

P3 number of units produced during month 3

P4 number of units produced during month 4

i1 number of units in inventory at the end of month 1

i2 number of units in inventory at the end of month 2

i3 number of units in inventory at the end of month 3

i4 number of units in inventory at the end of month 4

Objective function: Minimize total costs =

20(P1) + 25(P2) + 30(P3) + 35(P4) + 2(i1) + 2(i2) + 2(i3) + 2(i4)  (cost of production and inventory)

Constraints:

50 ≤ P1, P2, P3, P4 ≤ 150   (wrote it that way because it says in the table that monthly production capacity is 150 units, so im assuming the firm cant produce more than 150 units a month, so P1, P2, P3, and P4 MUST be less than or equal to 150. Also, they could be greater than or equal to 50 because the paragraph after the table says that the “firm wants to produce at least 50 units per month)

Constraints on demand (as the book calls them):

P1 - i1 = 100

I1 + P2 – i2 = 125

I2 + P3 – i3 = 200

I3 + P4 – i4 = 175

(Got those using the formula to find the current inventory-on-hand, which is basically: inventory at the end of the month = inventory at the end of last month + current month’s production – current month sales. To find the inventory at the end of month 1 (which is the variable i1), it would be:

i1 = 0 (since the question asks us to assume that the beginning inventory is 0) + P1 (current production) – 100 (demand), then I just rearranged the equation to have the variables on one side and the contant on the other side.

Non-negativity:

All variables ≥ 0