Purpose

The main goal of this experiment is to determine the density using two ways: the direct method and Archimedes’ Principle. In the first method we measured the mass of the aluminum cylinder using a scale. By using the Vernier Caliper, we were able to determine the length of the cylinder and we used the Micrometer to find the density. In the second part of the experiment, we used Archimedes’ Principle which involves immersing the cylinder in a beaker filled with water, and hanging it from the scale; we made sure that it does not touch the bottom of the beaker.

Theory

The Direct Method

ρ=MV

In order to find the density of the object, its mass, length and diameter must be obtained, which gives us the equation:

Eqn. 1

V=Al

Eqn. 2

where, M is the mass, V is volume and ρ is density. Since the object that we are using is cylinder, the equation below must be used to find its volume:

ρ=πd24

where, A is the cross section surface area of the cylinder, and l is the length. To find the area, the equation is given by:

Eqn. 3

With all the equations above combined, we will end up with:

ρ=4Mπld2

Eqn. 4

σρ=ρ×σMM2+σll2+4σdd2

In experiments like this, there are always errors. To determine the error, we use the equation:

Eqn. 5

Archimedes’ Principle

𝐹 = 𝑀𝑎 = 𝜌𝑓V𝑓𝑔

Eqn. 6

The second method indicates that the vertical upward force that is exerted on the object immersed in a fluid, is equal to the magnitude to the weight of the substance.

Wwater=Wair-pwVg

Eqn. 7

Where F represent the force, M is the mass, a is the acceleration, 𝜌𝑓 is the substance’s density, V𝑓 is the volume of the fluid displaced, and g is the acceleration due to gravity. When the object is totally submerged, the volume, V, of both of the object and water is given by:

Where Wwater is the weight of the water, Wairis the weight of the cylinder in air, Pwis the density of the water, and Vg is volume. The weight of the cylinder in air is given by the equation:

Wair=paVg

Eqn. 8

To solve for pa, we combine these equation to get:

pa=pwMairMair-Mwater

Eqn. 9

σPa=∂pa∂Mair2∂2Mair+∂pa∂Mwater2∂2Mwater

In order to determine the errors, we use the equation:

Eqn. 10

Which then turns to:

σPa= MwaterσMairMair-Mwater22+MairσMwaterMair-Mwater2

Eqn. 11

Apparatus

The set-up is fairly simple. In the first part of the experiment, the mass of the aluminium cylinder is measured using an electronic balance scale. The length is then measured using Vernier Caliper, and using a Micrometer, the diameter is measured. In the second part of the experiment, a 50mL beaker was filled with water and was placed under the scale in which the cylinder is attached to. This causes the cylinder to be immersed in the water, but not touching the bottom of the beaker.

Figure 1: Apparatus for determining the density using the direct method

The electronic balance scale can measure mass within the 0.0g to 9999g and was accurate to 0.05.  The measurements in the Vernier Caliper can range from 0.15cm and has an accuracy measuring to 0.02mm. The measurements in the Micrometer can range from 0-0.26mm and 0-49mm.

Figure 2: Apparatus for determining the density using the Archimedes’ Principle

The electronic balance scale can measure mass within the 0.0g to 9999g and was accurate to 0.05.

Observation

Table 1: Values that were obtained during the experiment using the direct method

|  | Mass of the cylinder in air 𝑀 (𝑔) | Diameter of the cylinder (Vernier Caliper) 𝑑 (𝑚𝑚) | Length of the cylinder (Micrometer) 𝑙 (𝑚𝑚) | Micrometer zero 𝑙0 (𝑚𝑚) |
| --- | --- | --- | --- | --- |
| Readings | - 14.16 - 14.16 - 14.15 | Top: - 19.1 - 19.05 Middle: - 19.05 - 19.1 Bottom: - 19.1 - 19.05 | - 18.6 - 18.61 - 18.61 | - 0.02 - 0.02 - 0.02 |
| Average | 14.16 | 19.1 | 18.6 | 0.02 |
| Reading Error | ±0.005 | ±0.03 | ±0.005 | ±0.005 |
| σIS | 0.00057 | 0.020 | 0.00057 | 0 |
| σmean(stat. error) | 0.0033 | 0.0081 | 0.0033 | 0 |
| Error Chosen (Reading or Stat.) | 0.01(reading) | 0.0033(statistical) | 0.005(reading) | 0.005(reading) |
| Final Values | ± | ± | lcorr=18.58 |  |
|  |  |  | Error on lcorr= 0.005 |  |
|  |  |  | ± |  |

Table 2: Values that were obtained during the experiment using the Archimedes’ Principle

|  | Mass of the cylinder in water 𝑀𝑤 (𝑔) |
| --- | --- |
| Reading | - 8.94g - 8.96 - 8.96 |
| Average | 8.95 |
| Reading Error | ±0.005 |
| 𝝈𝑰𝑺 | 0.0115 |
| 𝝈𝒎𝒆𝒂𝒏 (stat. error) | 0.0115 |
| Error used | 0.006 |
| Final Value |  |

Calculations

Results

Discussion