http://www.ProfessorMesser.com The 220-901 CompTIA A+ Exam

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 1

BIOS and UEFI

Basic Input/Output System (System BIOS, ROM BIOS)

• Software (firmware) used to start your computer

• POST - Power-On Self-Test

• BIOS settings are stored in nonvolatile BIOS memory

• Formerly the CMOS (Complimentary Metal-Oxide Semiconductor)

• Usually flash memory these days

BIOS Configurations

• Use BIOS configuration key during startup

• Configure and enable/disable components

• May include built-in hardware diagnostics

• Always have a backup before making changes

BIOS Passwords

• User Password - Needed to start the computer

• Supervisor Password - Neeed to make configuration changes

Trusted Platform Module (TPM)

• Adds advanced cryptographic functions

• Used by full disk encryption software

• May be a separate module or integrated into the motherboard

LoJack for Laptops

• Built into the BIOS

• Software installed into the OS

• Reinstalls itself if removed or new storage drive installed

• “Phone home” function provides location information

• Theft mode remotely locks the laptop and/or delete files and can force a startup password

Secure Boot

• Part of the UEFI specification

• Digitally sign known-good software

• Software won’t run without the proper signature

• Support in many different operating systems

• Windows 8, 8.1, Windows Server 2012 and 2012 R2

• Linux Fedora, openSUSE, Ubuntu Built into the BIOS

Upgrading your BIOS firmware

• Identify current firmware version

• Read the upgrade documentation

• Have a reliable power source

Motherboards ATX (Advanced Technology Extended)

• Standardized by Intel 1995

• Still very popular

microATX

• Small form-factor

• Similar mounting points and power as ATX

Mini-ITX

• An even-smaller form factor

• Similar mounting points as ATX and microATX

• Designed for single-purpose computing, i.e., streaming media

Professor Messer’s CompTIA A+ 220-901 Course Notes

220-901 Exam Objectives

• 1.0 - Hardware (34%)

• 2.0 - Networking (21%)

• 3.0 - Mobile Devices (17%)

• 4.0 - Hardware and Network Troubleshooting (28%)

Legacy BIOS

• The original / traditional BIOS

• No drivers for modern network, video, and storage devices

UEFI BIOS

• Unified Extensible Firmware Interface

• Based on Intel’s EFI (Extensible Firmware Interface)

• Designed to replace the legacy BIOS

• Boot from large (> 2.2 TB) GUID partition table (GPT) disks

Version 0x0C

• Maximum of 90 questions

• Multiple choice and performance-based questions

• 90 minutes

• Passing score is 675 on a scale of 100-900

• Six to twelve months of hands-on experience recommended

Expansion Slots and Bus Speeds

PCI (Peripheral Component Interconnect)

• Legacy expansion slots - Speeds from 133 MB/s to 533 MB/s

PCI-X (PCI Extended)

• Designed for servers (this is not PCI Express)

• 1,064 MB/s throughput

PCIe (PCI Express)

• Serial communication - xl, x2, x4, x8, xl6, x32 lanes

• High performance for devices like high-end graphics adapters

• PCI Express throughput per-lane in each direction

• Speeds from 250 MB/s to 2 GB/s

Mini PCI and PCI Express Mini Card

• PCI and PCI Express for laptops

• Adapter cards are internal to the device

The “CMOS” Battery

• Not needed for today’s flash-based storage

• Maintains older BIOS configurations

• May only be used to maintain date/time

• A bad battery will require a BIOS configuration on every boot

• Can reset the BIOS configuration by removing the battery on older systems

Memory Types

DIMM (Dual lnline Memory Module)

• Electrical contacts are different on each side

• SDRAM - 168 pins

• DDR SDRAM - 184 pins

• DDR2 and DDR3 SDRAM - 240 pins

SO-DIMM (Small Outline Dual lnline Memory Module)

• Used in laptops and mobile devices

• DDR and DDR2 SDRAM - 200 pins

• DDR3 SDRAM - 204 pins

Micro-DIMM (Micro Dual lnline Memory Module)

• Electrical contacts are different on each side

• DDR SDRAM - 172 pins

• DDR2 and DDR3 SDRAM - 214 pins

CPU Sockets

© 2015 Messer Studios, LLC Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 2

http://www.ProfessorMesser.com

Pin Grid Array (PGA)

Zero Insertion Force (ZIF)

Land Grid Array (LGA)

Legacy Chipsets

Graphics+Adapter+Slot (PCI+Express+or+AGP)

Front&Side+ Bus+(FSB)

PCI+Bus

Memory Bus

PCI+

Northbridge Onboard Graphics

Adapter+ Slots

• Manages the connection between the Controller High&speed+ Graphics+

PCI+

CPU and memory

• May also connect high-end graphics Bus

Bus

such as PCI Express

Northbridge+

Southbridge CPU

(Memory+

Internal+Bus

(I/O+Controller+

• High speed processor

Southbridge Controller+

Hub) Hub)

USB,+Ethernet, IDE,+SATA,+etc.

• Relatively slower connections

• USB, Ethernet, SATA, etc.

Low+Pin+

• BIOS

Count+(LPC)+ Bus

Flash BIOS ROM

Super+I/O Memory+ Banks

Serial+Port+Control Parallel+Port+Control Floppy+Drive+Control Keyboard Mouse

The Chipset Evolution

Today’s integrated CPUs

• More functions are moving into the CPU

• Multiple processor cores

• Memory controller

• Graphics processing unit (GPU)

System Memory ROM (Read-Only Memory)

SRAM (Static RAM)

• Can’t be changed or erased

• Maintains data without constant refreshing

PROM (Programmable Read-Only Memory)

• Programmable ROM

• Very fast, very expensive

• Often used in processor caches

• Write once

DRAM (Dynamic RAM)

EPROM (Eraseable PROM)

• Write / Erase / Write again

• Requires constant refreshing to maintain dataWrite once

EEPROM (Electrically Erasable PROM)

• Flash memory

DDR (Double Data Rate SDRAM)

• Twice the data rate of SDRAM

DDR2 (Double Data Rate 2 SDRAM)

• 2x the bus clock multiplier of DDR

DDR3 (Double Data Rate 3 SDRAM)

• 4x the bus clock multipler of DDR

SDRAM (Synchronous Dynamic RAM)

• Synchronous with the common system clock

EEPROM (Electrically Erasable PROM)

• Flash memory SDR and DDR DDR Speeds

One$Clock$Cycle

Clock

Data

One$Clock$Cycle

Clock

Data Data$1 Data$2 Data$3 Data$4 Data$5 Data$6

Data$1 Data$2 Data$3 Data$4 Data$5 Data$6 Data$7 Data$8 Data$9 Data$10 Data$11 Data$12

Memory Clock Speed

I/O Bus Clock Speed

DDR Speed

Transfers per Second

Transfer Rate

Module Name

100 MHz 100 MHz DDR-200 200 Million 1,600 MB/s PC1600 SDR

133 MHz 133 MHz DDR-266 266 Million 2,133 MB/s PC2100

166 MHz 166 MHz DDR-333 333 Million 2,667 MB/s PC2700

200 MHz 200 MHz DDR-400 400 Million 3,200 MB/s PC3200

Memory Clock Speed DDR

DDR2 Speeds

DDR3 Speeds

Memory Clock Speed

I/O Bus Clock Speed

DDR Speed

Transfers per Second

Transfer Rate

Module Name

100 MHz 200 MHz DDR2-400 400 Million 3,200 MB/s PC2-3200

133 MHz 266 MHz DDR2-533 533 Million 4,266 MB/s PC2-4200

166 Mhz 333 MHz DDR2-667 667 Million 5,333 MB/s PC2-5300

Memory Types

200 MHz 400 MHz DDR2-800 800 Million 6,400 MB/s PC2-6400

266 MHz 533 MHz DDR2-1066 1.066 Billion 8,533 MB/s PC2-8500 Parity Memory

• Additional parity bit

• Can’t correct an error

Error Correcting Code (ECC) Memory

• Dectects errors

• Corrects on the fly

• Ideal for servers

Multi-channel Memory

• Installed in pairs or trios for maximum throughput

Registered Memory

• Includes an additional register between the RAM module and the memory controller

• Also called “buffered” memory

Single Sided vs. Double Sided Memory

• Groups of memory on a module that can be independently accessed

• Memory controller moves between the ranks/sides

I/O Bus Clock Speed

DDR Speed

Transfers per Second

Transfer Rate

Module Name

100 MHz 400 MHz DDR3-800 800 Million 6,400 MB/s PC3-6400

133 MHz 533 MHz DDR3-1066 1.066 Billion 8,533 MB/s PC3-8500

167 MHz 667 MHz DDR3-1333 1.333 Billion 10,667 MB/s PC3-10600

200 MHz 800 MHz DDR3-1600 1.600 Billion 12,800 MB/s PC3-12800

233 MHz 933 MHz DDR3-1866 1.866 Billion 14,933 MB/s PC3-14900

267 MHz 1067 MHz DDR3-2133 2.133 Billion 17,066 MB/s PC3-17000

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 3

Core Core Core Core

Processor Graphics

Shared'L3'Cache

Memory'Controller'I/O

System' Agent' and' Memory' Controller

Installing Expansion Cards

Storage Formats

• Check your motherboard documentation to determine number and type of slots

• CD - 700 MB capacity

• Check adapter card documentation for hardware and software requirements

• DVD - 4.7 GB (single layer), 8.5 GB (dual-layer)

• Use ESD strap, anti-static bag, and other electrostatic discharge precautions

• Blu-ray - 25 GB (single layer), 50 GB (dual-layer)

• Confirm driver installation through Windows Device Manager

• ROM - Read-only memory - Cannot write to media

• RW - Read-write - Used for backups

• BD-R - Blu-ray Disc Recordable

• BD-RE - Blu-ray Disc Recordable Erasable

• Magnetic Tape - 100 GB to multiple TB / cartridge

Hot swappable formats

• USB, FireWire, SATA, eSATA

© 2015 Messer Studios, LLC Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 4

http://www.ProfessorMesser.com

Flash Memory

RAID (Redundant Array of Independent Disks)

Block&7A

Inside a Hard Disk Drive

RAID 0 - Striping RAID 1 - Mirroring RAID 5 - Striping with Parity

Block&1A

Block&2A

Disk%0 Disk%1

Disk%0 Disk%1

• File blocks are split between physical drives

• High performance

• No redundancy

• Minimum of 2 drives

Block&1

Block&1

Block'1A

Block'2A

Block&3A

Block&4A

Block&2

Block&2

Block'1B

Block'2B

Block&5A

Block&6A

Block&3

Block&3

Block'1C

Parity'C

Block&8A

Block&4

Block&4

Parity'D

Block'1D

Disk%0 Disk%1

• File blocks are duplicated

• File blocks are striped along with a parity block between physical drives

• Efficient use of disk space

• High disk space utilization

• High redundancy

• High redundancy

• Minimum of 3 drives

• Minimum of 2 drives

RAID 1+0 - A Stripe of Mirrors

• Stripe file blocks across mirrored drives

• High disk space utilization

• High redundancy

• Minimum of 4 drives

Block&10

RAID%0

RAID%0 RAID%1 RAID%1

Block&12

RAID%1

Block&1

Block&1

Block&2

Block&2

Block&3

Block&3

Block&4

Block&4

Block&5

Block&5

Block&6

Block&6

Block&7

Block&7

Block&8

Block&8

Block&9

Block&9

Block&10

Block&11

Block&11

Block&12

Block'3A

Parity'A

Parity'B

Block'3B

Block'2C

Block'3C

Block'2D

Block'3D

Disk%2 Disk%3

Rota%onal   Speed   (rpm)

Average   Latency

10,000 3  ms

7,200 4.16  ms

5,400 5.55  ms

Intel Sockets

AMD Sockets

Socket Type Pins

Release Date

Supported  CPUs Supported  RAM

Socket Type Pins

Release Date

Supported CPUs Supported RAM

Socket  T LGA 775 2004 Pen3um  4,  Core  2 DDR2  and  DDR3

AM3 PGA 941 2009 Athlon II / Phenom II DDR2/DDR3 dual-channel Socket  B LGA 1366 2008 Core  i7  (Nehalem) DDR3  triple-­‐channel

AM3+ PGA 942 2011 Athlon II / Phenom II DDR3 dual-channel Socket  H  /  H1 LGA 1156 2009 Core  i3/5/7  (Nehalem) DDR3  dual-­‐channel

FM1 PGA 905 2011 AMD 10h DDR3 dual-channel Socket  H2 LGA 1155 2011 Sandy/Ivy  Bridge DDR3  dual-­‐channel

FM2 PGA 904 2012 Piledriver DDR3 dual-channel Socket  H3 LGA 1150 2013 Haswell,  Broadwell DDR3  dual-­‐channel

Socket  R LGA 2011 2011

Ivy  Bridge-­‐E/EP,  Haswell-­‐E/EP Sandy  Bridge-­‐E/EP,

FM2+ PGA 906 2014 DDR3/DDR4  quad-­‐channel

Steamroller DDR3 dual-channel

Interface Speeds and Distances USB - Maximum of 127 ports

FireWire 400 (Alpha mode) - IEEE 1394a USB 1.1

• Low speed: 1.5 megabits per second, 3 meters

• 100, 200, or 400 Mbit/s half-duplex

• 4.5 meters (15 feet), 72 meters max

• Full speed: 12 megabits per second, 5 meters

FireWire 800 (Beta mode) - IEEE 1394b USB 2.0

• 480 megabits per second, 5 meters

SATA

• Revision 1.0 - 1.5 Gbit/s, 1 meter

• Revision 2.0 - 3.0 Gbit/s, 1 meter

• Revision 3.0 - 6.0 Gbit/s, 1 meter

• 800 Mbit/s full-duplex

eSATA

• Optical connections - 100 meters max

• Matches the associated SATA revision USB 3.0

IrDA (Infrared Data Association)

• 2 meters

• SuperSpeed: 4.8 gigabits per second, 3 meters

• 4 Mbit/s speed

• Line of sight, 1 meter VGA (Video Graphics Array)

• 10 meters DVI (Digital Visual Interface)

• Max distance isn’t part of the standard

• 15 feet at 1920 x 1200 resolution

© 2015 Messer Studios, LLC Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 5

http://www.ProfessorMesser.com Bluetooth Speeds and Distances Thunderbolt

• Thunderbolt v1: 10 Gbit/s per channel, 20 Gbit/s total throughput

• Thunderbolt v2: 20 Gbit/s aggregated channels

• Thunderbolt v3: 40 Gbit/s

• Max 3 meters (copper), 60 meters (optical)

Frequency

Connection Characteristics

Radio Class

Maximum Power  (mW)

Typical  Range   in  meters

1 100 100

802.11 Network Speeds and Distances

2 2.5 10

(per  stream)   throughput theore1cal Maximum

Maximum allowable   streams

throughput   theore1cal   Maximum

(total)

Approximate

3 1 1

outdoor range

Bluetooth Version 802.11a 5  GHz 54  Mbit/s 1 54  Mbit/s 120  meters

802.11b 2.4  GHz 11  Mbit/s 1 11  Mbit/s 140  meters

802.11g 2.4  GHz 54  Mbit/s 1 54  Mbit/s 140  meters

802.11n 5  GHz  and/or  2.4  GHz 150  Mbit/s 4 600  Mbit/s 250  meters

802.11ac 5  GHz 866.7  Mbit/s 8 6,934  Mbit/s 250  meters

Data Rate 1.2 1  Mbit/s 2.0  +  EDR 3  Mbit/s

3.0  +  HS 24  Mbit/s 4.0 24  Mbit/s

Analog modulation

• Send an analog signal over an analog channel

• AM and FM audio over radio frequencies

Digital modulation

• Send a digital bitstream over an analog channel

• Send digital audio signal over satellite radio frequencies

Connectors

USB 1.1/2.0 USB 3.0

Standard-A Plug Standard-B Plug Mini-B Plug Micro-B Plug

FireWire

USB

6-pin Alpha Type A Plug

(powered)

VGA 15-pin DB15 Analog Video

TRS - Analog Audio

RCA Connectors

© 2015 Messer Studios, LLC Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 6

http://www.ProfessorMesser.com

HDMI

miniHDMI

HDMI

DVI Digital Video and Audio

Analog and Digital Video

4-pin Alpha (unpowered)

BNC Connectors

TOSLINK (Toshiba Link) Optical Fiber Audio

RJ11 RJ45

Mini DisplayPort or Thunderbolt

9-pin Beta (powered)

SATA

SATA Power

Mini-DIN-4 (S-Video)

USB 3.0 Standard-B Plug

SATA Data

USB 3.0 Standard-A Plug

Mini-DIN-6 (Keyboard/Mouse)

eSATA

SATA Data

USB 3.0 Micro-B Plug

DisplayPort

24-pin motherboard power

Worksta(on Processor Video RAM Storage Audio Notes Graphics/CAD/CAM ✓ ✓ ✓ Maximum  RAM Audio/Video  Edi8ng ✓ ✓ Fast  storage,  dual  monitor Virtualiza8on  Worksta8on ✓ ✓ Maximum  CPU  and  RAM Gaming ✓ ✓ ✓ High-­‐end  cooling Home  Theater ✓ ✓ HDMI,  surround  sound,  TV  tuner Thick  Client Recommended  Windows  requirements

Thin  Client Basic  applica8ons

Laser Printer

• Uses heat and pressure to melt toner pellets onto the page + Very high quality + Fast printing speeds - Many moving parts - Requires memory in the printer - Requires high voltages

Inkjet Printer

• Prints with very fine drops of ink +Inexpensive technology + High quality +Relatively quiet - Ink is expensive and proprietary - Eventually fades - Print head clogs easily

Thermal Printer

• Applies heat to special paper +Very inexpensive +Almost silent - Requires a specially coated paper - Sensitive to light and heat - Image degrades rapidly

Impact Printer

• Printer head strikes a ribbon and the paper +Low cost per page +Good for multiple copies - Very noisy - Poor graphics

Print to File

• Print to the driver, but save it as a file

• File will be in a output format specific to that printer

• Must copy the file to the printer: copy filename LPT1:

Print to PDF

• One-way print from app to PDF

• Requires specialized software to create and view a PDF

• PDF viewers built into many Internet browsers

• Many third party tools available

Print to XPS

• Microsoft XML Paper Specification

• Similar use case to Adobe PDF

• Print to XPS, view in any operating system

Print to image

• Print to a graphics image

• Not integrated into the OS

• Some third-party image print drivers are available

Power connectors

Molex connector

SATA power - 15 pins Custom Workstation Configurations

Understanding Printers

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 7

PCIe 6-pin and 8-pin power

4-pin ATX +12 V

8-pin EPS +12 V power

Step 1: Processing

• Build the entire page in memory Step 2: Charging

• Prepare the drum with a negative electrostatic charge Step 3: Exposing

• Write the image with the laser Step 4: Developing

• Add toner to the charged areas of the imaging drum Step 5: Transferring

• Move the toner from the drum to the paper Step 6: Fusing

• Heat and pressure Step 7: Cleaning

• Remove excess toner

Laser Printer Maintenance Toner Cartridge Replacement

• Replace cartridge when output is faded

• Keep in the bag to protect OPC from light

• Power down printer before replacement

• Remove packing strips from the new cartridge Laser Printer Maintenance Kit

• Periodic maintenance based on page count

• Replacement feed rollers, transfer rollers, pickup rollers and fuser unit

• Power down during maintenance

• Reset the page counter when done

Thermal Printer Maintenance

Impact Printer Maintenance Inkjet Printer Maintenance

Cleaning print heads

• Output may have streaks or sections of missing color

• Cleaning process can be started manually

• Some print heads/cartridges can be removed and cleaned manually Replacing the Inkjet Cartridge

• Usually separate colors

• Takes seconds to replace Inkjet printer calibration

• Align nozzles to the paper

• Printer includes a calibration option

Study Tips Exam Preparation

• Download the exam objectives, and use them as a master checklist

• Use as many training materials as possible. Books, videos, and Q&A guides can all provide a different perspective of the same informatiovn.

• It's useful to have some hands-on, especially with network troubleshooting commands.

Thermal Paper Replacement

• Relatively inexpensive

• Must use the correct size

• Replace spool and feed through the printer

Cleaning the Heating Element

• Use an isopropyl alcohol (IPA) cleaning pen

• Follow printer manufacturer recommendations

• Usually a small area

• Cleaning cards can be used for the paper path

Printer Ribbon Replacement

• Self-contained cartridge

• Replace when output is faded

• Modular design, replace in a few minutes

Print Head Replacement

• Gets very hot during use

• Another modular part, look for release bar

• Consider replacing both head and ribbon simultaneously for best effect

Taking the Exam

• Use your time wisely. You've got 90 minutes to get through everything.

• Choose your exam location carefully. Some sites are better than others.

• Get there early. Don't stress the journey.

• Manage your time wisely. You've got 90 minutes to get through everything.

• Wrong answers aren't counted against you. Don't leave any blanks!

• Mark difficult questions and come back later. You can answer the questions in any order.

The Laser Printing Process

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 8

Network Connectors

TIA/EIA Standards

Cable Categories Cable Category

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 9

ST Connectors - Straight Tip

Maximum   Distance

Category  3 10BASE-­‐T 100  meters

Category  5

100BASE-­‐TX,   1000BASE-­‐T

100  meters

Category  5e (enhanced)

100BASE-­‐TX   1000BASE-­‐T

100  meters

Category  6 10GBASE-­‐T 37  to  55  meters

Category  6A   (augmented)

10GBASE-­‐T 100  meters

Category  7* (shielded)

10GBASE-­‐T 100  meters

SC Connectors LC Connectors

RJ11 Connector

Ethernet Standard

BNC Connectors F Connector RJ45 Connector

Fiber Optics

Multimode Fiber - Short-range communication, Up to 2 km

Single-mode Fiber - Long-range communication, Up to 100 km

Fiber connector

Copper Cable Categories RFC 1918 Private Addresses

IP  address  range

Number  of   addresses

descrip6on Classful

Host  ID  size

10.0.0.0  – 10.255.255.255

192 11000000

Largest  CIDR  block (subnet  mask) UTP Unshielded Twisted Pair

STP Shielded Twisted Pair

10.0.0.0/8 (255.0.0.0)

24  bits

172.16.0.0  – 172.31.255.255

16,777,216 single  class  A

16  con=guous   class  Bs

172.16.0.0/12 (255.240.0.0)

20  bits

192.168.0.0  – 192.168.255.255

1,048,576

65,536

256  con=guous   class  Cs

192.168.0.0/16 (255.255.0.0)

16  bits

IP subnets

Binary Decimal CIDR

11111111.00000000.00000000.00000000 255.0.0.0 /8

11111111.11111111.00000000.00000000 255.255.0.0 /16

11111111.11111111.11111111.00000000 255.255.255.0 /24 IPv4 Addressing

Coaxial Cable

.

168

.

1

.

131

APIPA (Automatic Private IP Addressing)

.

10101000.

00000001

.

10000011

• Automatically assigned addresses

• 169.254.1.0 through 169.254.254.255

IPv6 Link Local Addresses

8)bits =

1)byte = 1)octet • Required on every IPv6-enabled interface

• fe80::/10 with only one subnet allocated (all zeros)

• The last 64 bits are created with a modified EUI-64

32)bits)=)4)bytes

IPv6 Addressing

fe80::5d18:652:cffd:8f52 fe80:0000:0000:0000:5d18:0652:cffd:8f52 fe80

:

0000

:

0000

:

0000

:

5d18

:

0652

:

cffd

:

8f52 1111111010000000

:

0000000000000000

:

0000000000000000

:

0000000000000000

:

0101110100011000

:

0000011001010010

:

1100111111111101

:

1000111101010010

16-bits =

2-bytes =

2-octets 128-bits-=-16-bytes Common Network Protocols

Protocol Port Name Descrip/on

DHCP udp/67-­‐68 Dynamic  Host  Configura:on  Protocol Automate  the  IP  address  configura:on  process

DNS udp/53 Domain  Name  System Convert  domain  names  to  IP  addresses LDAP tcp/389,  udp,  389 Lightweight  Directory  Access  Protocol Directory  service  protocol SNMP udp/161 Simple  Network  Management  Protocol Gather  metrics  and  manage  network  devices SMB tcp/445 Server  Message  Block Windows  file  transfers  and  printer  sharing SSH tcp/22 Secure  Shell Encrypted  remote  console  login AFP tcp/548 Apple  Filing  Protocol Mac  OS  file  transfers SLP tcp/427,  udp/427 Service  Loca:on  Protocol Find  Mac  OS  services  by  name

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 10

Common Network Ports

Protocol Port Name Descrip/on

FTP tcp/20,  tcp21 File  Transfer  Protocol Sends  and  receives  files  between  systems SSH tcp/22 Secure  Shell Encrypted  console  access Telnet tcp/23 TelecommunicaCon  Network Insecure  console  access SMTP tcp/25 Simple  Mail  Transfer  Protocol Transfer  email  between  mail  servers

DNS udp/53,  tcp/53 Domain  Name  System Convert  domain  names  to  IP  addresses HTTP tcp/80 Hypertext  Transfer  Protocol Web  server  communicaCon POP3 tcp/110 Post  Office  Protocol  version  3 Receive  email  into  a  email  client IMAP4 tcp/143 Internet  Message  Access  Protocol  v4 A  newer  email  client  protocol HTTPS tcp/443 Hypertext  Transfer  Protocol  Secure Web  server  communicaCon  with  encrypCon

RDP tcp/3389 Remote  Desktop  Protocol Graphical  display  of  remote  devices NetBIOS udp/137 NetBIOS  name  service Register,  remove,  and  find  Windows  services  by  name NetBIOS udp/138 NetBIOS  datagram  service Windows  connecConless  data  transfer NetBIOS tcp/139 NetBIOS  session  service Windows  connecCon-­‐oriented  data  transfer

SLP tcp/427,  udp/427 Service  LocaCon  Protocol Find  Mac  OS  services  by  name SMB tcp/445 Server  Message  Block Windows  file  transfers  and  printer  sharing   AFP tcp/548 Apple  Filing  Protocol Mac  OS  file  transfers Wireless Encryption Standards

WEP

• 64-bit or 128-bit key size

• Cryptographic vulnerabilties found in 2001

• WEP can no longer be used

WPA

• Short-term workaround after WEP

• Used RC4 cipher as a TKIP (Temporal Key Integrity Protocol)

• TKIP has its own vulnerabilities

WPA2

• Replaced TKIP with CCMP (Counter Mode with Cipher Block Chaining Message Authentication Code Protocol)

• Replaced RC4 with AES (Advanced Encryption Standard)

• WPA2 is the latest and most secure wireless encryption method

Configuring a SOHO Wireless Router

LAN

• Local Area Network

• A building or group of buildings

WAN

• Wide Area Network

• Spanning the globe (or the neighborhood)

PAN

• Personal Area Network

• Bluetooth, WiFi

• Automobile, mobile phone, health telemetry

MAN

• Metropolitan Area Network

• Contained in a regional area

Wireless channels and encryption

• Configure for the highest encryption possible

• Choose WPA2-AES over WPA or WEP NAT

• SOHO devices do this automatically

• Source NAT, also called PAT Port Forwarding

• 24x7 access to a service hosted internally

• External IP/port number maps to an internal IP/port

• Also called Destination NAT or Static NAT Port Triggering

• Similar to a port forward, but not static

• Internal client communicates externally on a particular port

• Reverse port forward is dynamically created Network Types

• Only one person can trigger at a time IP Addressing

• DHCP (automatic) IP addressing vs. manual IP addressing

• IP addresses are easy to see in an unencrypted wireless network Firewall and DMZ ports

• Every SOHO router is also a firewall

• DMZ ports can be configured to allow unrestricted access Managing QoS (Quality of Service)

• Change the priority of your traffic

• Prioritize applications, ports, or MAC addresses Firmware updates

• Can have dramatic change on wireless performance

• May improve compatibility with chipsets from other devices UPnP (Universal Plug and Play)

• Automatically configure and find other network devices

• Applications on the network can open inbound ports using UPnP

• Best practice would be to disable UPnP

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 11

PoE (Power over Ethernet)

• One wire for both network and electricity

• Built-in power - Endspans

• In-line power injector - Midspans Networking Tools

Internet Connection Types

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 12

Line-of-sight services

• Visual path between antennas - high frequencies

• Common in metropolitan areas

• WiMAX networking - Worldwide Interoperability for Microwave Access Network Devices

Cable Modem

• DOCSIS (Data Over Cable Service Interface Specification)

• 4 Mbits/s through 250 Mbits/s

• Data, Voice

Wireless access point (WAP)

• Bridges from wired network to wireless

• Makes forwarding decisions based on MAC address

Modem

• Modulator/Demodulator

• Converts analog sounds to digital signals

Dialup

• Network with voice telephone lines

• 56 kbit/s modems, compression up to 320 kbit/s

• Legacy systems, network utility

Fiber

• Fiber optics to the home

• Converged services - Voice, Video, Data

• Enhanced features

Hub

• Multi-port repeater

• Traffic going into one port is repeated to every other port

Switch

• Bridging done in hardware

• Forwards traffic based on data link address

Router

• Routes traffic between IP subnets

• Forwards traffic based on network address

• Routers inside of switches sometimes called “layer 3 switches”

Crimper

• “Pinches the connector onto the wire

• The final step of a cable installation

Cable stripper

• Quickly remove the insulation from the copper

• Speeds the cable installation process

Multimeter

• Measures voltage, current, resistance

• Check AC/DC voltage, cable continuity

Tone generator and probe

• Find a specific wire

• 2 pieces; tone generator and inductive probe

DSL

• ADSL (Asymmetric Digital Subscriber Line)

• 24 Mbit/s downstream / 3.3 Mbit/s upstream

• VDSL (Very-high-bit-rate DSL)

• 3 Mbit/s through 100 Mbit/s

Cable tester

• Measure the quality of a cable installation

• Crosstalk, signal loss, etc.

1

2

3

4

5

6

7

8

RJ-­‐45  Ethernet  and  Serial  Loopback  Plug

Loopback plug

• Used for diagnostics and troubleshooting

• Is a received signal is the same as the sent signal?

Satellite networking

• Non-terrestrial communication

• 15 Mbit/s down, 2 Mbit/s up

• High latency

• High frequencies - 2 GHz

ISDN - Integrated Services Digital Network

• BRI – Basic Rate Interface (2B+D)

• Two 64 kbit/s bearer (B) channels

• One 16 kbit/s signaling (D) channel

• PRI – Primary Rate Interface

• T1 – 23B + D, E1 – 30B + D + alarm channel

Cellular networks

• Mobile devices - “Cell” phones

• Tethering - Turn your phone into a wireless router

• Mobile hotspot - Standalone networking devices

Punchdown tool

• Forces wires into a wiring block

• Trims the wires and breaks the insulation

WiFi analyzer

• Purpose-built hardware or mobile device add-on

• Identify errors and interference

Patch panels

• Punch-down blocks and RJ-45 connectors

• Runs from desks are made once

• Patch panel to switch can be easily changed

Copper line drivers and extenders

• Extend the range of copper wire

• Used with serial links, copper Ethernet

• Powered device, regenerates the signal

PLC (Power Line Communication)

• May be marketed as Ethernet over Power (EOP)

• 500 megabits per second

Firewall

• Filters traffic by IP, port number, or application

Laptop Expansion Options

ExpressCard

• Two form factors - 34 mm and 54 mm wide

• 54 mm slot also accepts 34 mm cards

• Can be added and removed while the OS is running

SO-DIMM

• RAM upgrade

• Must power down to install

Flash memory

• USB connected

• Use as additional storage

Thunderbolt and Mini DisplayPort

• It’s the same connector

• Mini DisplayPort is video and audio

• Thunderbolt adds data

Interface Adapters

• USB to RJ-45 Ethernet

• USB to Wifi

• USB to Bluetooth

• USB optical drive

Laptop Displays LCD (Liquid Crystal Display)

• Lightweight, low power consumption

• Requires a separate backlight (florescent, LED) TN (Twisted Nematic) LCD + Fast response times (gaming!) + Low power draw - Poor viewing angles - color shifts IPS (In Plane Switching) LCD + Excellent color representation + No tailing when touched (mobile devices) - More expensive to produce than TN CCFL - Cold Cathode Fluorescent Lamp

• Higher voltage and power needed

• No longer a common backlight LED-backlit LCD display

• Backlight is LEDs instead of florescent

• The latest laptops are LED-backlit OLED Displays (Organic Light Emitting Diode)

• Thinner and lighter - no glass needed

• The organic compound provides the light

• Used for smaller mobile devices due to cost Laptop Features Special Keyboard Function Keys

• Can control external displays and audio

Display Options

• Use the Fn key as a toggle

• Toggle between LCD / external monitor

• May also have a physical / magnetic switch

Wireless Control

• Physical switch to enable/disable wireless

Volume Settings

• Fn key or standalone key

Screen Brightness

• Controls the backlight strength

• Helps to conserve battery life

Keyboard Backlight

• Set intensity, duration, or disable

Touch Pad

• Enable to help avoid inadvertent mouse clicks and movements

Screen Orientation

• Useful on rotating tablet / laptops

• Fn key, hotkey, or another method

Media Options

• Control audio / video from your keyboard

• Play, stop, rewind, fast forward

GPS

• Enable or disable the GPS radio

• May also be associated with airplane mode or other wireless settings

Docking Stations and Port Replicators

• Similar functionality

• Port replicator extends existing laptop interfaces

• Docking station adds additional features, options to include desktop adapter cards

Physical laptop locks

• Keep your laptop from walking away

• Connect to a solid object

Rotating / removable screens

• Combine a laptop keyboard with a tablet screen

• May include a stylus for input

Mobile Devices

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 13

Tablets

• Single-screen touch computers

• Designed for touch input,

• Application and media support

Smart phones

• Mobile communication

• Media viewer and mobile applications

Wearable technology

• Smart watches, fitness monitors

• Glasses and headsets

Phablets

• Phones and tablets

• ~5.5 inches to 7 inches diagonal size

e-Readers

• Specialized device

• Electronic paper

• WiFi and cellular network access

Smart camera

• Traditional digital cameras

• Powerful operating systems

• Smart features (touch screen, wireless networking, social media integration, etc.)

GPS

• In-car (and non-car) navigation

• Requires a view of the sky

• Periodic updates required

Troubleshooting Hardware Problems Unexpected shutdowns

• Check all fans and heat sinks

• Check Device Manager for failing hardware symptoms

• Eliminate what’s working Lockups

• Check for activity lights, update drivers and software patches

• Consider reverting to a restore point

• Hardware diagnostics may be helpful POST (Power On Self Test)

• Tests system components before starting the OS

• Failures are noted with beeps and/or codes

• Every manufacturer uses different codes Blank screen on boot

• Bad video, BIOS configuration issue, listen for the beeps Booting to an incorrect device

• Set boot order in BIOS configuration

• Confirm that the boot device has a valid OS Conginuous reboots

• How far does the boot process get?

• Try safe mode or OS recovery options Power and reboots

• No power - check the power supply

• Unexpected shutdown - run hardware diagnostics

• Only fans spin - check power supply output

Mobile Device Connections

Mobile Device Accessories

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 14

NFC (Near Field Communication)

• Send small amounts of data wirelessly over a limited area

• Short range with encryption support

Proprietary mobile interfaces

• Early mobile technology

• Every manufacturer was different

Micro-USB and mini-USB

• EU standardized on Micro-USB

• Older devices may use Mini-USB

Lightning

• Apple proprietary

• 8-pin digital signals

Speakers

• Mobile audio

• Battery powered

• Bluetooth wireless connection

Headsets

• Hands-free audio

• Wired - Connects to TRRS connector

• Wireless - Uses Bluetooth

Game pads

• EU standardized on Micro-USB

• Older devices may use Mini-USB Docking stations

• No wires to connect, charge and sync

• Places the phone upright

Overheating

• Heat generated from CPUs, video adapters, memory

• Check cooling fans and heat sinks

• Verify temperatures with monitoring software

Loud noises

• Loose components can rattle

• Hard drive scraping noises and clicking

• Check fans for obstructions

• Blown capacitor

Intermittent device failure

• Check and reseat, may be bad hardware

Status light indicators

• Seen often on network devices

• Power, link, speed, activity

Smoke and burning smell

• Electrical problems (Always disconnect power!)

• Locate and replace bad components

Crash screens

• Windows Stop Error (Blue Screen of Death)

• Contains important information

The spinning ball of death

• The Mac OS X Spinning Wait Cursor

• Application bug, bad hardware, slow paging to disk

Memory / microSD

• Increase storage capacity of smartphone or tablet

• MicroSD small form factor cards

Bluetooth

• High speed communication over short distances

• PAN (Personal Area Network)

• Connects our mobile devices

IR (Infrared)

• Included on many smartphones, tablets, and smartwatches

• Control your entertainment center

Hotspot / tethering

• Turn your phone into a WiFi hotspot

• Dependent on phone type and provider

• May require additional charges and data costs

Extra battery packs / battery chargers

• Swappable battery pack

• External USB chargers

Protective covers

• Screen protector

• Device protector

Credit card readers

• Phone or tablet becomes a point of sale terminal

• Uses the Internet link for approvals

Defragmentation tool

• Moves file fragments so they are contiguous

Troubleshooting the Boot Process

Boot loaders

• Bootstrap loader in the BIOS

• Second-stage boot loader - Winload, GRUB, LILO Fixing the Master Boot Record

• Boot from the installation media

• Start the Windows Command Prompt

• BOOTREC /fixmbr

• Fix the Master Boot Record on a physical drive

Fixing the Volume Boot Record

• Boot from the installation media

• Start the Windows Command Prompt

• BOOTREC /fixboot

• Writes a new boot sector DISKPART

• Replaces the Pre-Windows-XP FDISK command

• Add, remove, list volumes/partitions

POST card with USB power

• Detailed diagnostics during POST

• Connects to PCI/PCI Express/ Parallel

• Powered through USB Troubleshooting Hard Drives Troubleshooting disk failures

• Get a backup as soon as possible

• Check for loose or damaged cables

• Check for overheating - Especially if problems occur after startup

• Check power supply, especially if new devices were added

• Run hard drive diagnostics from the drive manufacturer Troubleshooting boot failures

• Check your cables

• Check boot sequence in BIOS

• Check for removable disks (especially USB)

• Check for disabled storage interfaces

• For new installation, check the hardware configuration

• Try the drive in a different computer

RAID

RAID failures

• Missing or faulty RAID controller

• Each RAID is different

• Don’t start pulling drives until you check the console! Crash screens

• Windows Stop Error, Apple spinning wait cursor

• May indicate a storage device issue

• Diagnostics needed for drive and motherboard S.M.A.R.T. Errors

• Self-Monitoring, Analysis, and Reporting Technology

• Avoid hardware failure, look for the warning signs

• Schedule periodic disk checks

Disks   Required

Failure  Condi4on

RAID  0 2  or  more A  single  drive  failure  breaks  the  array  with  data  loss

RAID  1 2  or  more Array  will  work  as  long  as  one  drive  is  opera>onal

RAID  5 3  or  more Need  all  drives  opera>onal  but  one

RAID  10 4  or  more Can  lose  all  but  one  from  each  set  of  mirrors Hard Drive Troubleshooting Tools

Hardware Troubleshooting Tools

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 15

CHKDSK

• CHKDSK /f - Fixes errors on the disk

• CHKDSK /r - Locate and recover from bad sectors

Screwdriver

• Flat, crosspoint, torx, non-magnetized

• Never open the drive!

External disk enclosure

• Move from internal to external on separate PC

• USB connected

• May not be bootable, but files are available

Multimeter

• Check AC, DC voltages, continuity, and more

• Can answer many electrical questions

Power supply tester

• Check the power supply power output

• LCD panel can show voltages

• Easier and faster than a multimeter

Loopback plugs

• Usually don’t need them until there’s a problem

• Need separate plugs for different media

FORMAT

• Adds a file system to a partition

• This also removes all file entries

File recovery software

• Recover deleted files, lost files

• Recover from disk, USB flash, memory card

Troubleshooting Video and Display Issues No video image

• Check physical connection

• Confirm input selection on monitor

• Check brightness

• Swap the monitor with a known-good

• Start Windows in VGA mode (F8) Image quality problems

• Flickering, colors missing, blur

• Check for bent interface connector pins

• Match Windows video refresh rate and resolution settings to the monitor specifications

• Disable hardware acceleration

• Set resolution higher to correct oversized images

Pixel problems

• Stuck pixels (constantly bright)

• Dead pixels (always black) Video artifacts

• Unusual graphics

• Check adapter

• Image persistance

• Turn off display

• Motion trails

• Disable advanced video features BSOD and overheating

• Update video drivers

• Monitor the PC's internal temperature Troubleshooting Networks No network connectivity

• Check for link light

• Ping loopback (127.0.0.1)

• Ping local IP address

• Ping default gateway

• Ping devices on router’s other side Automatic Private IP Addressing (APIPA)

• A link-local address

• IETF has allocated 169.254.1.0 through 169.254.254.255

• Automatically assigned Limited or no connectivity

• Windows alert in the system tray

• “Limited or No connectivity”, “No Internet Access”

• Check the local IP address

• If DHCP address is obtained, perform the ping tests Intermittent connectivity

• Check the system tray for broken LAN icons or messages

• May be a problem with the switch or wireless access point IP conflicts

• Windows will identify a duplicate

• Two identical IP addresses will conflict

• Reboot or reset the network adapter

Slow transfer speeds

• Router or infrastructure congestion

• Speed and duplex incompatibility

• Hardware issue with the adapter

• Malware infection

Low RF wireless signal

• Interference - Something else is using our frequency

• Signal strength - Check the signal and antenna locations

• Incorrect channel - Usually automatic, try tuning manually

• Bounce and latency - Multipath interference; flat surfaces

• Incorrect access point placement - Locate close to the users

Wireless interference

• Predictable

• Florescent lights, microwave ovens, etc.

• Unpredictable

• Multi-tenant building

• Measurements

• Signal strength, Performance Monitor

SSID not found

• Too far away

• Local networks are “louder”

• Wireless router has disabled SSID advertisements Network Troubleshooting Tools

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 16

Cable tester

• Relatively simple continuity test

• Can identify missing pins or crossed wires

• Not usually used for frequency testing

1

2

3

4

5

6

7

8

RJ-­‐45  Ethernet  and  Serial  Loopback  Plug

Loopback plug

• Useful for testing physical ports

• Serial / RS-232 (9 pin or 25 pin)

• Network connections - Ethernet, T1, fiber

Punch-down tool

• “Punch” a wire into a wiring block

• Trims the wires during the punch

Toner probe

• Follow the tone to trace a cable

• Tone generator puts an analog sound on the wire

• Inductive probe has a speaker

Cable crimper

• “Pinch” the connector onto a wire

• Connect the modular connector to the cable

• Metal prongs are pushed through the insulation

Wireless locator

• Software or hardware analyzers

• Identify all wireless networks

• View configuration, channel, frequency settings

Wire stripper

• Quickly remove insulation around the copper

• Automates the process

• Avoids inadvertent cuts and nics

Network Troubleshooting at the Command Line ping - Test reachability

• ping <ip address> - Test reachability to a TCP/IP address

• ping -t <ip address> - Ping until stopped with Ctrl-c

• ping -a <ip address> - Resolve address to a hostname

• ping -n <count> <ip address> - Send a # of echo requests

• ping -f <ip address> - Send with Don’t Fragment flag set

ipconfig – Windows TCP/IP configuration

• ipconfig /all - Show all TCP/IP details

• ipconfig /release - Release the DHCP lease

• ipconfig /renew - Renew the DHCP lease

• ipconfig /flushdns - Flush the DNS resolver cache

ifconfig - Linux/Unix TCP/IP configuration

• ifconfig <interface> - Show TCP/IP details

tracert (Windows) or traceroute (Linux/Unix)

• Determine the route a packet takes to a destination

• Takes advantage of ICMP Time to Live Exceeded error message

• Not all devices will reply with ICMP Time Exceeded messages

netstat - Network statistics

• netstat -a - Show active connections

• netstat -b - Show binaries

• netstat -n - Do not resolve names

nbtstat - Query NetBIOS over TCP/IP information

• nbtstat -n - List local NetBIOS names

• nbtstat -A <ip address> - List remote NetBIOS names

• nbtstat -a <device name> - List remote NetBIOS names

net - Windows network commands

• NET USE - Map a network share to a drive letter

• net use h: \\<servername>\<sharename>

• NET STOP - Stop a service

• net stop spooler

• NET START - Start a service

• net start spooler

• NET VIEW - View network resources

• net view \\<servername>

netdom - Manage Active Directory database

• Join a computer to the domain, add a domain account, etc.

• Reset domain password

• netdom resetpwd /s:server /ud:domain\User /pd:*

nslookup - Lookup information from DNS servers

• Canonical names, IP addresses, cache timers, etc.

• Find an ip address - nslookup www.professormesser.com

• Find a name - nslookup 8.8.8.8

Troubleshooting Laptops LCD issues

• Dim video may be caused by a bad backlight

• Video may be barely visible without the backlight

• May need to replace the LCD inverter or display Blurry video

• LCD displays have an optimal native resolution

• Check the manual to determine the correct resolution Flickering video

• Laptop LCDs are always opening and closing

• Connector issue, bad video cable or hardware Input issues

• Sticky keys require a cleaning - be careful, keycaps are delicate!

• Inadvertent mousepad touches cause cursor to jump around

• Updated drivers may help

• Laptops Num lock changes the keyboard from letters to numbers

Wireless antenna problems

• A laptop has multiple antennas; WiFi main, WiFi aux, Bluetooth

• Antenna wires wrap around the top of the LCD to get up high

• It’s easy to accidentally disconnect the wires during maintenance Battery not charging

• Battery may be too old to maintain a charge

• The charging hardware could be faulty

• Check the battery voltage with a multimeter No laptop power

• Check the external power adapter with a multimeter

• Try a master laptop reset, i.e., hold power button for 10 seconds External monitor issues

• Usually toggled with the secondary Fn keys

• Toggles between LCD / external monitor / both

• Use an external monitor to bypass the LCD Troubleshooting Mobile Devices Touchscreen completely black or not responding to input

• Apple iOS restart

• Press power button, slide to power off, press power button

• Hold down power button and Home button for 10 seconds

• Android device restart

• Remove battery, put back in, power on

• Hold down power and volume down until restart

• Some phones have different key combinations

• Some phone do not have a key-based reset Apps not loading or slow app performance

• Restart the phone - Hold power button, power off

• Stop the app and restart

• iPhone: Double-tap home button, slide app up

• Android: Settings/Apps, select app, Force stop

• Update the app - Get the latest version

Unable to decrypt email

• Each user has a private key

• Install individual private keys on every mobile device

• Use a Mobile Device Manager (MDM) Short battery life

• Bad reception

• Disable unnecessary features - 802.11 wireless, Bluetooth, GPS

• Check application battery usage

• Replace aging battery Overheating

• Check app usage - Some apps can use a lot of CPU

• Avoid direct sunlight Frozen system

• Soft and/or hard reset

• Ongoing problems may require a factory reset

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 17

Troubleshooting Mobile Devices (continued) No sound from speakers

• Check volume settings - both app and phone settings

• Bad software / delete and reload

• Try headphones

Sound starts but then stops

• Dueling apps

• Keep app in foreground

No speaker sound from any app

• Load latest software

• Factory reset

GPS not functioning

• Check settings to enable GPS

• iOS: Settings / Privacy / Location Services

• Android: Settings / Location

• Configure location mode

Swollen battery

• Buildup of gas - do NOT open the container

• Faulty battery - stop using immediately

• Dispose of properly

• Device can be damaged - better than having a fire

Device Disassembly Best Practices Device disassembly

• Many different pieces, intricately engineered

• Taking it apart is easy, getting it back together...

• Easy to break something - very delicate parts

Document and label

• Identify cable locations - many different cables

• Document screw locations - many screws in many locations

Organize parts

• Laptops disassemble in sections

• Take pictures, and use a big workspace

• Use containers to separate the sections

Manufacturer resources

• Often provide step-by-step repair guides

• Online written guides

• YouTube videos

Use appropriate hand tools

• Sometimes a single screwdriver

• Sometimes specialized tools

• Get a good tweezer

• Magnification will be needed

• Get a big anti-static cloth

Troubleshooting Printer Problems On-printer test page

• This checks the printer operation without any OS or application

• May be a power-on process or menu option

• If the printer's test page doesn't work, the printer is the problem

Windows printer test page

• Print a test page from the Windows printer properties

• A bad Windows printer test page may be related to a driver, cable, interface, or connection

Streaks and blurs

• Inkjet - Clean the print heads

• Laser - Check for scratches on the photosensitve drum

Faded prints

• Low toner and low ink

• Poor quality toner or ink

Ghost images

• The laser printer optical drum may not be cleaning properly

• Print shows ghost or "shadow" from previous drum rotation

Color prints in wrong color

• Low ink in one cartridge

• Other colors should print normally

Laser printer smudging

• The toner isn’t fusing to the paper

• The fuser unit may not be heating

• May require replacing the fuser unit

Paper jam

• Tray problem or bad pickup rollers may not feed paper properly

• Creased paper may cause problems in the paper path

• Check the paper weight with printer manufacturer specifications

Printer network issues

• No connectivity - Check for power and cabling

• Wireless printers may require additional configuration

• Check Windows printer security tab for rights and permissions

Garbled output

• Bad printer driver (PCL vs. PostScript)

• The application may be sending bad data

• Confirm the printer health with an on-printer test page

Operating System issues

• Printer driver installation issues may be related to OS permissions

• Make sure the correct driver is installing (32 bit vs. 64 bit)

• Queue may not be moving, print jobs “stuck” in the queue

• The print spooler may have crashed or not working

• Restart the spooler from the Windows Services

Error messages

• Visible on the printer display console

• Low memory problems are related to laser printer RAM

• Complex images and graphics consume more memory

No output

• Run a test print

• Check the connectivity

• Print from a different program

© 2015 Messer Studios, LLC Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 18

http://www.ProfessorMesser.com

Printer Troubleshooting Tools

Laser printer maintenance kit

• Replacement feed rollers, new fuser unit, etc.

• Based on the printer’s page counter

• Reset the page counter when kit is installed

Toner vacuum

• Specialized vacuum - low static, filtered for toner

• Use damp cloth on the outside

• Wipe dust away on the inside

• Don’t use a traditional vacuum cleaner

Common Printing Problems

Normal Windows Printer Test Page Windows Printer Test Page from a laser printer

with a scratched photosensitive drum

A “ghosting” Windows Printer Test Page from a laser printer with a faulty cleaning subsystem

Printer Test Page printed with the incorrect page description langugage or incorrect driver

© 2015 Messer Studios, LLC http://www.ProfessorMesser.com Professor Messer’s CompTIA 220-901 A+ Course Notes - Page 19

Compressed air

• “Canned air”

• Fluorocarbon gas compressed into a liquid

• Not a great choice for cleaning printers

• Consider using a chemical-free air compressor

Printer spooler

• Manages printing in the background

• Runs as a Windows service

• May need to restart after a crash