Flow Through A Venturi Meter

Fluid Mechanics Lab 1

Carleton University

Table of Contents

1. Summary	2

2. Nomenclature	3

3. Flow Analysis	4

4. Experimental Setup and Procedure	6

5. Results and Discussion	6

5.1 High Flow Rate	6

5.2 Low Flow Rate	6

5.3 Discussion	7

6. Conclusions	8

7. References	9

8. Appendix : Sample Calculations for High Flow Rate	10

9. Appendix: Data Tables	12

10. Appendix: Original Data Sheet	13

1.0 Summary

The experiment was done to analyze air flow through a venturi meter, to do this there were sixteen air taps at several different locations along the meter.  These air taps were connected to a manometer banks which showed several different heights of the fluid corresponding to its location on the meter.  The venturi meter is a long tube with belled ends and a collapsed throat in the middle.  Air was pulled through the meter at a high and low pressure and was analyzed using Bernoulli’s equation and the continuity equation.  This lab was performed to understand and analyze stagnation, static, and dynamic pressure.

2.0 Nomenclature

Qactual:actual volume flow rate m3s

Qideal  :ideal volume flow rate m3s

Cv         :venturi coefficient (no units)

P           :static pressure Pa

Patm     :atmospheric pressure=101.3 kPa

PT         :stagnation pressure, or total pressure Pa

Ps          :static pressure Pa

Pd          :dynamic pressure Pa

V           :velocity of fluid ms

ρair      :density of air= 1.23 kgm3

ρwater :density of water=1000kgm3

g           :acceleration due to gravity=9.81 ms2

h       :height of water in manometer taps m

A            :cross sectional area m2

3.0 Flow Analysis

The hydrostatic equation is used to find the pressure difference between two taps:

P1-P2= ρg(h2- h1)  (1)

Bernoulli’s equation:

P1 + 12ρairv12+ρwatergh1=P2 + 12ρairv22+ρwatergh2 (2)

The  ρwatergh terms on either side of the equation cancel and the equation reduces to:

P1 + 12ρairv12=P2 + 12ρairv22 (3)

V1 is equal to 0 because the air is not moving outside of the venturi meter, then rearranging to solve for V2:

V22=2(P1-P2)ρair      (4)

Inserting equation 1 into equation 4:

V22=2(Patm-(Patm-ρwatergh))ρair   (5)

Equation 5 is reduced further to solve for V2:

V2=2ρwaterghρair    (6)

Since:

Qactual=V2A2     (7)

Equations 6 and 7 can be married to form:

Qactual=2ρwaterghρair*A2   (8)

The idealized volume flow rate can be calculated as such.

Beginning with equation 3 we can isolate to:

12ρair(V22-V12)=P1-P2     (9)

Recalling the continuity equation:

A1V1= A2V2   (10)

Can be solved for V2:

V2=A1V1A2     (11)

Inserting equation 11 into equation 9, rearranging to solve for V1 yields:

V1=2(P1-P2)ρairA1A22-1      (12)

Recalling:

Qideal=V1A1   (13)

Marrying equations 12 and 13, we get:

Qideal=2(P1-P2)ρairA1A22-1*A1      (14)

Another important equation needed to find the Venturi Coefficient will be:

CV=QactualQideal 	(14)

4.0 Experimental Setup and Procedure

The apparatus and procedure went fully according with the MAAE 2300 Fluid Mechanic I Laboratory Exercises manual, images 1 and 2 in the Appendix of this report give a visual representation to the apparatus.

5.0 Results and Discussion

5.1