Chapter 4

- Point Defects are 0 Dimensional

- Line Defects are 1 Dimensional

- Planar Defects are 2 Dimensional

- Volume Defects are 3 Dimensional

- Point Defects

  - Intrinsic

    - Vacancy

    - Interstitial

  - Extrinsic

    - Substitution

    - Interstitial

- Vacancy

  - An atom is missing

- Self-interstitial

  - If the matrix atom occupies a space where it doesn’t belong

- Alloying

  - Done to improve strength, corrosion resistance, ductility and lower the melting temp

- Solid Solution

  - Forms when solute atoms are added to the host material and the crystal structure is maintained

  - Substitutional

    - Solute and Solvent are roughly the same size and solute will replace solvent

  - Interstitial

    - Solute is smaller than solvent and will occupy the interstitial positions in the lattice

- Hume Rothery Rules

  - Basic conditions for an element to dissolve in a metal (Substitutional)

    - Atomic radius must not differ by more/less than 15%

    - Similar electro-negativities

    - Same crystal structure

    - Same Valence

  - Interstitial

    - Solute is smaller than solvent

    - Similar Electroneg

    - Same Valence

- Line Defects

  - Edge and Screw Dislocations

    - Edge

      - Slicing partway through a crystal, spreading it apart and partially filling the space with an extra half plane

    - Screw

      - Cutting partway through a crystal and then skewing it by one atom spacing so it is twisted

- Surface

  - Grain Boundaries

    - Separates regions of different crystalline orientation

  - Tilt Boundary

    - Edge dislocations are aligned

  - Twin Boundary

    - One side of the  boundary are located in mirror image positions of the other

- Volume

  - Introduced during processing and fabrication steps

    - Internal cracks

    - Second phase particle

    - Inclusion

    - Porosity

- Electron Microscopes

  - Transmission type which looks through a thin slice

  - Scanning Type which looks at the surface of a solid

Chapter 5

- Diffusion

  - Material transport by the random motion of individual atoms from high to low concentration

  - Important because : Doping of semiconductors, Case Hardening, Diffusion Coatings & Homogenization of castings

- Diffusion Mechanisms

  - Requires an Empty adjacent site and sufficient energy to break the bond

  - Atomic vibration provides the energy

  - Higher the temp the more vibrations the more diffusion

- Vacancy type and interstitial type

- Diffusion is time dependent

- Temperature increases the speed of diffusion

- Interstitial is faster than vacancy type

- Constants are different for each solute and solvent

- Faster in polycrystalline than single crystal

Chapter 6

- Elastic and Plastic Deformations

  - Elastic is recoverable

  - Plastic is not

- Loading

  - Tensile

  - Compression

  - Shear

  - Torsion

  - Bending

- Tensile Test

  - Load Vs Displacement

  - Requires twice the load to produce the same elongation if the cross section is doubled

  - Stress is the ration of the applied load to the original cross section

  - Strain is the Extension over the original length

  - Young’s Modulus/Modulus of Elasticity

    - Depends on bond strength

    - Decreases with increasing temperature

  - Plastic Deformation

    - Stress and Strain are not proportional

    - Deformation is not reversable

    - Is caused by breaking and rearranging bonds

  - Yield Strength

    - The point where the material has permanent deformation

  - Tensile Strength

    - Max stress on the stress strain curve

    - Is the max stress that can be sustained in tension

    - Necking begins at this point

  - Ductility

    - How much strain a given stress produces

    - Highly ductile materials can experience a lot of strain where brittle cannot

  - Resilience

    - Ability to absorb energy during elastic deformation and give it back when unloaded

    - Measured with Ur

  - Toughness

    - Ability to absorb energy before fracture

    - Energy required to break a material

    - Around the area under the stress  strain curve

  - True Stress Strain

    - Takes into account the changing cross sectional area as the load is applied

  - Hardness

    - Material’s resistance to localized plastic deformation

      - Dents and Scratches

  - Hardness and tensile strength are proportional

  - Fatigue

    - Lowering of strength or failure of a material due to repetitive stress

  - Creep

    - Slow deformation over time under stress and temperature

    - Is a function of stress, time and temperature

Chapter 7

- When metals are plastically deformed, around 5% of energy is retained internally and the rest is dissipated as heat

- Slip System

  - Dislocations move more easily on specific planes and in specific directions

  - There is a preferred plane and direction

  - This is called the slip system

  - They are the plane and direction with the highest densities

  - FCC is on {111} planes and <110> directions

- Stress and Dislocation Motion

  - Edge and screw move ins response to shear stresses along the slip plane/direction

  - Shear components exist at all but thee parallel and perpendicular allignments

- Critical Resolved Shear Stress

  - When the shear stress becomes too large the crystal will yield

- Grain Boundary Strengthning

  - Grain Boundary acts as a barrier to dislocations

    - Because of their orientations

    - Atomic disorder within a boundary region will result in discontinuity of slip planes

  - Dislocation of same sign pile-up at the grain boundaries because their slip plane comes to a dead end

- Solid Solution Stregthening

  - Alloying with impurity atoms to strengthen

  - Increasing the concentration of the impurity results in an increase in tensile and yield strength

  - The impurity atoms impose lattice strains on the hosts

  - This causes dislocation movement to become restricted

  - Degree of strengthening depends on  -

    - A difference in size between atoms, the larger the atom the greater disruption making slip harder

    - The greater the amount of alloying element the greater the strengthening effect

- Strain Hardening

  - Before work hardening the lattice exhibits a defect-free pattern

  - As it is work hardened it becomes saturated with new dislocations

  - This creates a resistance to plastic deformation

  - Average distance between dislocations decreases and dislocations start to block each other’s motion

- Cold Work

  - Yield Strength Increases

  - TS increases

  - Ductility Decreases

  - Point defect and dislocation density increase

  - Done on ductile metals

- Annealing

  - Recovery

  - Recrystallization

  - Grain Growth

- Recovery

  - First Stage

  - Low temp

  - Physical properties of CW materials are restored

  - Some reduction in dislocation density

- Recrystallization

  - Thermally activated process that involves nucleation and growth of new strain free grains

  - High strain and High temp accelerate this stage

  - Recrystallization Temperature is ½ to 1/3 the melting temp

  - Also depends on amount of CW and purity

  - Increasing the percentage of CW enhances the rate

  - Driven by stored energy of CW

- Grain Growth

  - Driving force is a reduction in the energy which is stored in the material in the form of grain boundaries

  - Growth is driven by reduction in grain boundary energy

  - The driving pressure for growth is up to 2 orders of magnitude less than that of recrystallisation

  - Big grains grow at the expense of small ones

Chapter 8

- Applied tensile stress is amplified at the tip of a small incision or notch

- SEM microscopes are preferred

- Fracture

  - Separation of a body into two or more pieces in response to an imposed static stress and at low temp relative to melting temp

- Two kinds of fracture

  - Ductile

    - Plastic deformation occurs before final separation

    - Cup and cone features

    - High energy absorption

    - Increased stress due to necking

    - Final crack occurs by shear @ 45 degrees

    - Dimples form pointing toward the origin of the failure

  - Brittle

    - No deformation

    - Rapid crack propagation

    - Direction of crack is perpendicular

      - Trans granular fracture

        - Passes through the grains

        - Corresponds to the repeated breaking of bonds along specific planes

          - This process is called cleavage

        - Grainy or faceted texture under the microscope

      - Intergranular

        - Along grain boundaries

        - Normally results subsequent to the occurrence of processes that weaken or embrittle grain boundary regions

  - Ductile fracture is more desirable than a brittle one

  - A brittle fracture may occur in normally ductile materials

- Stress Concentration

  - Actual strength is much lower than calculated due to microscopic flaws and cracks

  - These flaws are a detriment to the fracture strength because an applied strength may be amplified at the tip

  - This is more significant in brittle materials

- Fracture toughness

  - Dependent on critical stress for crack propagation and crack length

- Plane Strain fracture toughness

  - Used when the specimen thickness is much greater than the crack dimensions

  - The constant then becomes independent of thickness

- Ductile to Brittle Transition

  - Related to the temperature dependence of the measured impact energy

- Fatigue

  - Can occur at a lower load if it is fluctuating or cyclic

  - Accounts for 90% of all failures

  - Is always brittle like no matter the material

  - Testing

    - Employs a rotating bending beam: alternating tension and compression stress of equal magnitude are imposed as it is simultaneously bent and rotated

  - S-N Curve

    - Stress vs number of cycles

    - Becomes horizontal at high N values for some steels

    - Failure will not occur below the fatigue limit which is a limiting stress level

  - Fatigue Crack

    - 3 Stages of Fatigue failure

      - Crack Initiation

      - Crack Propagation

      - Final Failure

  - Mean Stress

    - Effects fatigue life

    - Increasing the level leads to a decrease in life

  - Surface Effects

    - The max stress usually occurs at the surface, consequently most cracks originate at the surface

  - Thermal Fatigue

    - Elevated temperature by fluctuating thermal stresses

    - No need for mechanical stress

  - Corrosion Fatigue

    - Occurs by the simultaneous action of cyclic stress and chemical attack

    - Pits may form as a result of chemical reactions which serve as a point of stress concentration.

  - Creep

    - Elevated temperatures and static mechanical stresses

  - Testing for Creep

    - Subjecting a specimen to a constant load while maintain a constant temp for an elongated time

  - Creep Behavior

    - Instantaneous deformation

      - Mainly elastic

    - Primary/transient creep

      - Slope of strain vs time decreases with time : Work hardening

    - Secondary/Steady-State creep

      - Rate of straining constant: work hardening and recovery

    - Tertiary

      - Rapidly accelerating strain rate up to failure: formation of internal cracked, voids, grain boundary separation, necking etc.

  - Instantaneous strain at time of stress application increases

  - The steady state creep rate increases

  - Rupture lifetime decreases

  - Creep rate is a function of stress

Chapter 9

- Components and Phases

  - Component

    - Chemical species

    - Binary alloy has 2 components

    - Ternary alloy has 3 components

  - Phase

    - A portion with distinct, uniform physical or chemical characteristics

- Solubility Limit

  - Solvent

    - Host or major component

  - Solute

    - Minor component

  - Solubility Limit

    - Max amount that can be dissolved in a phase

- Microstructure

  - Properties depend on the proportions of the phases and how they are arranged

  - Phase diagrams help understand and predict microstructure

- Equilibrium

  - Constant temp, pressure and composition system is stable

  - Minimum in the free energy

- Metastable

  - Appears to be stable

- Phase Diagram

  - Combinations of temperature, pressure or composition for which specific phase exist at equilibrium

Chapter 10

- Bainite

  - Transformation rate controlled by microstructure growth (Diffusion) rather than nucleation

  - Competitive with pearlite

- Spheroidite

  - Annealing of pearlitic or bainitic at Temp just below Eutectoid

  - Relative amount of ferrite and cementite do not change, only shape of cementite

  - Driving force – reduction in total ferrite – cementite boundary area

- Martensite

  - Austentite quenched to room temp

  - Nearly instantaneous at required T

  - No diffusion

  - Metastable

  - Can coexist with other other phases

- Cementite is harder and more brittle than ferrite

- Increasing cementite fraction makes a harder less ductile material

- Strength and hardness are inversely related to the size of the microstructure

- Bainite, pearlite and spheroidite

  - Spheroidite is the softest

  - Fine pearlite is stronger and harder than coarse

  - Bainite is harder and stronger than pearlite

- Martensite is the hardest but most brittle

- Tempered Martensite

  - More ductile

  - Mechanical properties depend upon cementite particle size

  - Particle size increases with higher tempering temp/ longer time

Chapter 11

- Steels are iron-carbon alloys

- Mech properties are sensitive to carbon content

- Plain Carbon steels

  - Mainly contain carbon

- Alloy Steels

  - Alloying elements are intentionally added in specific concentrations

- Low Carbon

  - 0.25%wt carbon

  - Unresponsive to heat treatment

  - Accomplished by cold work

  - Microstructure has ferrite and pearlite

  - Soft and weak but high ductility and toughness

  - Machinable and weldable

- Medium Carbon

  - 0.25 and 0.60%wt

  - Can be heat treated

- High Carbon

  - 0.60 and 1.4wt%

  - Hardest, strongest and least ductile

  - Always used in a hardened and tempered condition

- Stainless Steels

  - Highly resistant to corrosion

  - Their predominant alloying element is chromium at least 11 wt%

  - Corrosion resistance can be enhanced by nickel and molybdenum

  - Divided into 3 classes based on their predominant phase

    - Martensitic

    - Ferritic

    - Austentitic

- Cast Irons

  - Above 2.14 wt%

  - Most are between 3.0 and 4.5 wt%

  - Carbon exists as graphite and both microstructure and mech behavior depend on comp and heat treatment

- Copper Alloys

  - Brass

  - Zinc is substitutional impurity

  - Predominant alloying element

  - Bronze is a alloy of copper and aluminium, silcon and nickel

- Magnesium Alloys

  - Very low density

  - Used where weight is factor (aircraft)

  - Difficult to deform at room temp

  - Most fabrication is by casting or hot working at 200 and 250

- Titanium

  - Relatively new and possess and extraordinary combination of properties

- Aluminum Alloy

  - Relatively low density

  - High electrical and thermal conductivities

  - Strength can be enhanced by cold work and alloying

- Precipitation hardening

  - Hardness may be enhanced by the formation of extremely small, uniformly dispersed particles of a second phase

  - Induced by heat treatments

  - Age hardening is also used to designate this procedure as strength develops with time

Chapter 12 - 13

- Bonding in ceramics

  - Can be ionic and/or Covalent

  - %ionic character increases with difference in electroneg of atoms

  - For those materials which the atomic bonding is predominantly ionic the crystal structures may be thought of as being composed of electrically charged ions

- Factors that determine crystal structure

  - Relative size of ions

  - Maintenance of charge neutrality

- Si and O are the most common elements on earth

  - The strong Si-O bonds lead to a high melting temo for silicate

- Glass

  - Noncrystalline

  - Fused silica to which no impurities have been added

- Carbon

  - Diamond

    - Hardest material

    - Very high thermal conductivity

    - Large single crystals

  - Graphite

    - Layered structure

    - Weak van der waal’s force between layers

    - Plane’s slide easily over each other

  - Fullerenes

    - Spherical cluster of 60 carbon atoms

  - Nanotubes

    - Sheet of graphite rolled into a tube

    - Ends capped with fullerene hemispheres

  - Graphene

    - Single layer of carbon atoms arranged in a hexagonal lattice

- Point defects

  - Vacancies

    - Exist for both cations and anions

  - Interstitals

    - Exist only for cations

    - Anions are too large for the site s

  - Frenkel Defect

    - A cation vacancy-cation intertitial pair

  - Shottky defect

    - A paired set of cation and anion vacancies

- Testing

  - 3 point bend test

Chapter 14-15

- Poly – mer

  - Many – repeat unit

- Characteristics

  - Cheap

  - Tough

  - Lightweight

  - Easy to shape

  - Flexible

  - Resistant to chemical attack

  - Thermally and electrically insulating

- BUT

  - Low stiffness and strength

  - Limited temperature range n

  - High coefficient of thermal expansion

  - Degrade when exposed to elements

  - Uncertainty in life expectancy

- Composition

  - Hydrocarbons

  - Double and triple bonds can be unstable

- Isomerism

  - Two compounds with the same chemical formula can have different structures

- DP

  - Degree of polymerization

  - Average number of repeat units per chain

- Copolymers

  - Two or more monomers polymerized together

    - Random

    - Alternating

    - Blocks

    - Graft

- Categories

  - Thermoplastics

    - Flexible linear chains

    - Polyethylene

  - Thermosetting

    - Rigid three dimensional network

    - Polyurethane

    - Epoxy

  - Elastomers

    - Thermoplastics or lightly cross-linked thermosets

    - Consist of spring like molecules

    - Rubber

- Thermoplastics

  - Most widely used

  - Classified according to their response to rising temperature

  - Soften when heated

  - Harden when cooled

- Thermosetting

  - Form a permanent shape

  - Harder and stronger than thermoplastics

  - Cannot be reformed

- Elastomer

  - Deformed to a large degree

  - Can spring back to original shape

  - Low Mod of elasticity

Chapter 16

- Composites

  - Combination of two or more individual materials

  - Used to obtain a more desirable combination of materials

  - Low Density High Strength

- Particle reinforced

  - Large particle

    - Particles are equiaxed and evenly distributed in the matrix

    - The rule of mixtures equations predict the elastic modulus

  - Cermets are examples of ceramic metal composites

    - Most common is cemented carbide

    - Contains tugsten carbide or titanium carbide embedded in a matrix of a metal such as cobalt or nickel

  - Fiber reinforced composites

    - Common fiber length is less than 15mm

    - Longer fibers provide more efficient stress transference from matrix

  - Fiber types

    - Whiskers

      - Graphite, Silicon, Nitride, Silicon Carbide

      - Large length to diameter ratio

      - Very expensive and very strong

    - Fibers

      - Polymers or ceramics

      - Alumina, Carbon, Aramid, E-Glass

    - Wires

      - Metal

      - Large Diameter

  - Matrix Phase

    - Binds the fibers together and acts as the medium by which a stress is transmitted

    - Protects the individual fibers from surface damage

    - Separates fibers and prevents the propagation of brittle cracks

  - Polymer Matrix Composites

    - Consist of polymer resin as the matrix with fibers as the reinforcement medium

    - Greatest diversity of composite applications

    - Good room temp properties

    - Classified as glass, carbon and aramid

  - Glass

    - Fiberglass

    - Produced in the largest quantities

    - Diameters of 3 and 20 microns

    - Low stiffness

    - Easily drawn into high strength fibers from the molten state

    - Cheap

    - Relatively strong

    - Good for corrosive environments

  - Carbon

    - Have the highest modulus and strength

    - Not effected by moisture at room temp

    - Diversity of physical and mechanical characteristics

    - Relatively inexpensive

  - Structural Composites

    - Laminates

      - Stacked and Bonded fiber reinforced sheets

      - Turned 90 degrees every layer

    - Sandwich panels

      - Honeycomb core between two sheets

      - Low density

      - Large bending stiffness

  - Production Methods

    - Pultrusion

      - Continuous fibers pulled through resin tank to impregnate fibers

      - Impregnated fibers pass through steel die that preforms desired shape

      - Preformed stock passes through a curing die to form the final shape

      - Cured in the die

    - Filament Winding

      - Continuous reinforcing fibers are positioned in a predetermined pattern to form a hollow shape

      - Fibers are fed through a resin bath to impregnate with thermosetting resin

      - Fibers are continuously wound

      - After the appropriate number of layers curing is carried out

    - Hand Lay up

Chapter 17

- Corrosion

  - Destruction or deterioration of material due to it’s environment

  - Mars is the largest corroded object

- Electrochemical process for metallic materials

- Rusting in Steel

  - Oxidation or rusting of iron in water which contains dissolved oxygen

- Eight Forms of Corrosion

  - Uniform Attack

    - Oxidation and reduction occur uniformly over surface

  - Selective Leaching

    - Preferred corrosion of one element

    - Solid Solution alloys

    - Microscopic galvanic corrosion

    - Loss of Mechanical Integrity

    - Can extract rare elements this way

  - Intergranular

    - Corrosion along grain boundaries

    - Material disintegrates across grain

    - S.S. is sensitized due to heat treatment

  - Stress Corrosion

    - Stress and Corrosion work together at crack tips

    - Combined action of  tensile stress and corrosion

    - Causes brittle failure

    - Stress level is well below TS

  - Galvanic

    - Dissimilar metals are physically joined, the more anodic one corrodes

      - Zn and Mg are very anodic

    - The more reactive metal will experience corrosion

    - Dependent on the anode to cathode areas that are exposed

    - For a given cathode area a smaller anode will corrode more rapidly

    - To reduce -

      - Choose metals close in the series

      - Use a large anode area

      - Electrically insulate dissimilar metals

      - Electrically connect a third anodic metal to the other two

  - Erosion Corrosion

    - Break down of passivating layer by erosion (pipe elbows)

    - Combined chemical attack and mechanical wear

    - Depends on –

      - Type of fluid

      - Velocity

      - Turbulence

    - To Prevent

      - Eliminate fluid turbulence effect

      - Erosion-Corrosion resistant materials

      - Remove particulate bubbles

  - Pitting

    - Downward propagation of small pits and holes

    - Very localized corrosion attack

    - Ordinarily penetrate from the top of a horizontal surface downwards

    - Often undetected

  - Crevice

    - Between two pieces of the same metal

- Corrosion Rate

  - Rate of material removal

  - Penetration rate

    - The thickness loss per unit time

    - Less than 20mpy is acceptable

- Environment

  - Atmospheric corrosion

    - Moisture containing dissolved oxygen

  - Water

    - Contains oxygen and minerals

    - Seawater is more corrosive than fresh

  - Soil

    - Moisture, oxygen and bacteria

- Prevention

  - Judicious selection of material

  - Changing the environment

  - Design Considerations

  - Application of coating

  - Cathodic Protection

    - Most effective means of protection

    - Feed electrons into it

    - Can be used for all types

    - Attach a more anodic material to the one being protected

    - Supply electrons to the metal to be protected from an external source to reverse the reaction