CIVE 2700

Civil Engineering Materials

Assignment 1

Instructor: Prof. Muslim Majeed

Ian Mokaya

#######

Date of submission: 8 November 2019

QUESTION 1:

The linear range is from -0.098 inches to 0.102

Calibration factor: Slope of best fit line of the linear range = 101.57 V/in

QUESTION 2:

Leadership in Energy and Environmental Design (LEED) requires that projects meet the prerequisites in some categories order to qualify for the lowest LEED certification level:

Sustainable sites – sustainable landscapes, storm water runoff management, heat island effect reduction, reduction in pollution

Water efficiency – reduction of potable water used by indoor plumbing fixtures

Energy and atmosphere – documentation of best management practices of energy efficiency, minimum energy efficiency performance, the building should not use any chlorofluorocarbon.

Materials and resources – solid waste management policies, implementation of an environmentally preferable purchasing policy.

Indoor Environmental Quality – Establishing a minimum standard for indoor air quality, implementation of controls for reducing, minimizing and preventing tobacco smoke exposure, implementation of a green cleaning procedures.

The LEED requirements for wood certification are as follows:

- Each wood product must be identified on a line-item basis

- Forest Stewardship Council (FSC) products must be identified on a line-item basis

- The dollar value of each line item must be shown

- The vendor’s chain of custody certificate number must be shown on any invoice that includes FSC products.

The LEED credit area that can be awarded the greatest number of points is the Energy and atmosphere category.

QUESTION 3:

Living Building Challenge is a rating system that is performance based and certifies projects such us landscape and infrastructure projects, partial renovations and complete building renewals, new building construction, neighborhood, campus and community design.

WELL Building Standard is a certification program for new and existing buildings, new and existing interiors, core and shell retail, education facilities, restaurant, commercial kitchen and multifamily residential

NZEB provides a certification option for a Net Zero Energy Building. These buildings have 100% of their energy is supplied by renewable energy on a net annual basis.

QUESTION 4:

[a] The elastic region is the region in which a material can deform linearly and proportionally to stress without going through permanent deformation. The plastic region is where a material exhibits permanent deformation when subjected to stress.

[b] A proton is a positively charged particle that is found in the nucleus of an atom.

An electron is a negatively charged particle that orbits the nucleus.

The atomic number is the number of protons in the nucleus of an atom.

Atomic weight refers to the sum of protons and neutrons in the nucleus of an atom

[c] Valence electrons are electrons located in the outermost shell of an atom that can be transferred to or shared with another atom. These electrons are important because they are used in bonding in order to create new compounds.

[d] Electrons tend to fill low energy shells and subshells first then fill in the order of s, p, d, f orbits. Individual electrons fill up a shell first and once all shells are occupied by at least an electron, the remaining electrons will occupy the shells already with an electron so that the shells have a pair of electrons.

[e] An ionic bond is the bond shared between an atom of a metallic element and an atom of a non-metallic element. The metallic element ‘donates’ electrons to the non-metallic element. A covalent bond is a type of bond that involves the sharing of atoms between atoms of different elements. A metallic bond is a type of bond formed between positively charged atoms in which the free electrons are shared among a lattice of cations.

QUESTION 5

[b]

| 0.2% Carbon |  | 0.5% Carbon |  | 0.8% Carbon |  |
| --- | --- | --- | --- | --- | --- |
| Stress (MPa) | Strain(mm/mm) | Stress (MPa) | Strain(mm/mm) | Stress (MPa) | Strain(mm/mm) |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 270.9453751 | 0.0014 | 270.9453751 | 0.0014 | 270.9453751 | 0.0014 |
| 279.0941082 | 0.002 | 389.1020049 | 0.002 | 389.1020049 | 0.002 |
| 289.2800246 | 0.003 | 399.2879212 | 0.003 | 580.5972324 | 0.003 |
| 299.4659409 | 0.01 | 409.4738376 | 0.01 | 660.04738 | 0.01 |
| 285.205658 | 0.02 | 405.3994711 | 0.02 | 780.2411931 | 0.02 |
| 315.7634071 | 0.05 | 480.7752521 | 0.05 | 910.6209224 | 0.05 |
| 399.2879212 | 0.1 | 600.9690651 | 0.1 | 853.5797908 | 0.1 |
| 460.4034194 | 0.15 | 684.4935793 | 0.15 |  |  |
| 490.9611685 | 0.2 | 694.6794956 | 0.2 |  |  |
| 444.1059532 | 0.25 | 619.3037146 | 0.25 |  |  |
| 399.2879212 | 0.275 |  |  |  |  |

[c]

[d] The proportional limits for each of the specimens are as follows:

Specimen 1 (0.2% carbon) – 270.95 MPa

Specimen 2 (0.5% carbon) – 389.20 MPa

Specimen 3 (0.8% carbon) – 580.60 MPa

[e]

The 0.2% offset yield strength for each specimen is as follows:

0.2% Carbon – 299.47 MPa

0.5% Carbon – 399.29 MPa

0.8% Carbon –  660.05 MPa

[f]

All the specimens have an equal linear proportion as it can be seen in the graph above. The Young’s Modulus for all three specimens is the slope f the linear proportion, which is 193705 MPa.

[g]

Yield Strength: increasing the carbon content in steel increases the yield strength, making the specimen stronger.

Modulus of elasticity: increasing the carbon content does not change the modulus of elasticity of seel

Ductility: an increase in the carbon content of steel reduces the ductility of the metal, thus making it more brittle.

Question 6:

[b]

Modulus of elasticity – 208911 MPa

[c] Proportional limit – 628.91 MPa

[d] Yield stress – 629.76 MPa

[e] Ultimate strength – 710.83 MPa

[f] Stress at 155 kN = 155000 N / (37.5*6.25) = 661.33 MPa

Strain = σ/E = 661.33 MPa/208911 MPa = 0.0031656 mm/mm

Permanent strain = 0.003161656-0.0030689 = 0.00009664 mm/mm

Permanent deformation = εLO = 0.00009664*203 = 0.0196 mm

[g] It is safe because if it is loaded then unloaded the structure will have only deformed by a small margin.

Question 7:

[b]

Young’s modulus – 72587 MPa

[c] Proportional limit – 320 MPa

[d]

Yield stress at 0.2% offset – 371.2 MPa

[e] Tangent modulus at 450 MPa:

dy/dx = -13907.2x + 2082.6

dy/dx at 0.04378 mm/mm = -13907.2(0.04378) +2082.6 = 1473.74 MPa

Tangent modulus at 450 MPa = 1473.74 MPa (ans.)

[f] Secant modulus: 450 MPa / 0.04378 = 10278.67 MPa

Question 8:

E = σε

75000 MPa = 275 MPaStrain

Strain = 27575000

Strain = 0.003667 mm/mm

Elongation = Strain * Original length = 0.003667 * 300 mm

Elongation = 1.1 mm

Question 9:

| Observation No. | P (lb) | Elongation (in) | Stress (psi) | Strain (in/in) | ui (psi) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | N/A |
| 1 | 1181 | 0.0015 | 6014.78361 | 0.00075 | 2.256688 |
| 2 | 2369 | 0.003 | 12065.2179 | 0.0015 | 6.783439 |
| 3 | 3550 | 0.0045 | 18080.0015 | 0.00225 | 11.31019 |
| 4 | 4738 | 0.0059 | 24130.4359 | 0.00295 | 14.78115 |
| 5 | 5932 | 0.0075 | 30211.4279 | 0.00375 | 27.74777 |
| 6 | 7008 | 0.0089 | 35691.4509 | 0.00445 | 23.07771 |
| 7 | 8336 | 0.011 | 42454.8994 | 0.0055 | 41.04764 |
| 8 | 9183 | 0.0146 | 46768.635 | 0.0073 | 80.34191 |
| 9 | 9698 | 0.018 | 49391.5084 | 0.009 | 81.77758 |
| 10 | 10196 | 0.0235 | 51927.8016 | 0.01175 | 139.3847 |
| 11 | 10661 | 0.0332 | 54296.0272 | 0.0166 | 257.7234 |
| 12 | 10960 | 0.0449 | 55818.8217 | 0.02245 | 322.2493 |
| 13 | 11159 | 0.0565 | 56832.3203 | 0.02825 | 326.854 |
| 14 | 11292 | 0.0679 | 57509.6838 | 0.03395 | 326.04 |
|  |  |  |  |  | ut = 1655.376 |

Question 10:

[a] Elastic modulus = 8053886.60 psi

[b] Proportional limit = 42454.89 psi

[c] Proportional limit with 0.2% offset = 47751.89 psi

[d] Tensile strength = 57509.68 psi

[e] Strain at 0.016 in = 0.016/2.0 = 0.008 in/in

Stress that produces 0.008 in/in strain = E*Strain = 8053886.60*0.008 = 64431.09 psi

Load that produces this stress = stress*area = 64431.09*(π*0.52)/4 = 12651.02 lb

12651.02 lb (ans.)

[f] Elastic recovery = σE = 64431.09 psi/8053886.60 psi = 0.008 in/in

Elastic recovery = 0.008 in/in *2.0 = 0.016 in

Permanent deformation = 0 in.

[g] Yes. It is possible for loads applied to a structure to cumulatively add up to 12,651.02 lb.